3GPP TSG-SA WG6 Meeting #63	S6-244621

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244330)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0342 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Verify UE location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the conclusuion of KI#4 of TR 23.700-21: “Solution #12 addressing the KI#4 can be considered in the normative work. The related function and procedure will be specified in 3GPP TS 23.434 [9] to support the solution.” There is a need to bring solution#12 to the normative work |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedure and information flow to support verify UE location |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature for verify UE location will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.x (new), 9.3.2.y (new), 9.3.2.x (new), 9.3.2.z (new), 9.3.2.u (new), 9.3.2.r (new), 9.3.x.1 (new), 9.3.x.1 (new), 9.3.x.2 (new), 9.3.x.3 (new), 9.4.1, 9.4.x.1 (new), 9.4.x.2 (new), 9.4.x.3 (new), 9.4.x.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-244330 |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 9.3.2.x	Confirm location service subscription request

Table 9.3.2.x-1 describes the information elements for the confirm location service subscription request from the VAL Server towards the location management server.

Table 9.3.2-1: Confirm location service subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user |
| Application identifier/VAL service id | M | Application identifier for which is the subscription request/ Identity of the VAL service for which the confirm location service is subscribed. |

#### 9.3.2.y	Confirm location service subscription response

Table 9.3.2.y-1 describes the information elements for the confirm location service subscription response from the location management server to the VAL Server.

Table 9.3.2.y-1: Confirm location service subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user |
| Subscription status | M | It indicates the subscription result |

#### 9.3.2.z	Confirm location request

Table 9.3.2.z-1 describes the information elements for the confirm location request from the location management client to the location management server.

Table 9.3.2.z-1: Confirm location request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of UE whose location information is to be confirmed |
| UE provided location | M | UE provided non-3GPP location information taken from functionality within UE (e.g. GNSS). It can include latitude and longitude, accuracy, timestamp. |
| Cell identifier | O | Cell identification information in the UE (NOTE) |
| Timestamp | M | Timestamp of the UE provided location |
| Application ID | M | Application identifier |
| NOTE: Cell identification information availability in LMC is depending on UE implementation. |  |  |

#### 9.3.2.u	Confirm location report

Table 9.3.2.u-1 describes the information elements for the confirm location request from the location management server to the location management client.

Table 9.3.2.u-1: Confirm location report

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of UE whose location information is to be confirmed |
| Confirm location status | M | unknown, verified, mismatch in the same country, mismatch in other country, roaming country mismatch, error unauthorized, error other (NOTE 1) |
| Cell identifier status (NOTE 2) | O | Connected to provided cell, not connected to provided cell |
| Timestamp | M | Timestamp of the confirm location check |
| Application ID | M | Application identifier |
| NOTE 1: Confirm location status granularity depends on the network capability. NOTE 2: Cell identifier status is available when there is cell information provided in the confirm location request. |  |  |

#### 9.3.2.r	Confirm location service usage notification

Table 9.3.2.r-1 describes the information elements for the confirm location service usage notification from the location management server to the VAL server.

Table 9.3.2.r-1: Confirm location service usage notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user |
| UE ID | M | The identifier of UE whose location information was confirmed |
| Timestamp | O | Timestamp of the confirm location service notification |

### 9.3.x	Verify UE location procedure

#### 9.3.x.1	General

Verify UE location could be used by multiple applications that need confirmed location to provide their service. The UE application logic would first confirm customer location before accepting customer requests for service in a specific location.

This procedure provides the VAL server application with subscription for verification service and supports verification of UE provided location by the LMS. Only UEs using application with such subscription can use the confirm location verification from LMS. Sharing of UE location to the network is only possible with user consent. The network provided location is not directly shared but only verification is provided to the requesting LM client. The verified location is for internal UE usage.

#### 9.3.x.2	Confirm location service subscription

The application is running on a VAL server. The application would like to use confirm location service and subscribes to LMS for it using the following procedure:

Figure 9.3.x.2-1: Procedure of confirm location service subscription

1.	The VAL server subscribes to LMS to receive notification every time UE using specific application is requesting confirmation of location.

2.	The LMS shall check if the VAL server is authorized to subscribe for the confirm location service notifications for a specific application identifier.

3.	If the VAL server and specific application are authorized to subscribe for confirm location service, the LMS creates subscription based on provided parameters in the request.

4.	The LMS sends to VAL server confirm location service subscription response.

#### 9.3.x.3	Confirm location verification

The location management client (LMC)/application in the UE gets from non-3GPP positioning (e.g. GNSS) in the UE its location. The application in the UE wants to check if this is really the correct location.

Pre-conditions:

1.	The LMC is authorized to discover and to use Confirm Location API provided by the LMS;

2.	LMS is authorized to use Nnef Event Exposure API, based on SLA with NEF/5GS of MNO;

3.	It is assumed there is user consent given in the UE for exposing location information from UE to LMS.

Figure 9.3.x.3-1: Procedure of confirm location verification

1.	The UE sends Confirm Location request from the LMC towards LMS. It contains the location measured from the UE (non-3GPP location information, e.g. GNSS). It contains latitude and longitude, accuracy, timestamp, carrier name, cell id (known to the UE from the network signalling), application identifier.

2.	 The LMS shall check if there is subscription (based on VAL subscription in clause 6.12.2.1) for confirm location service with the specific application identifier received in step1. If subscription is present, step 3 is performed. If subscription is not present the LMS replies back to LMC with confirm location status unknown (step 4).

3.	LMS requests UE location from 5GC/LMF via NEF (LMS send determine Location operation via NEF (N33 interface) and using Nlmf_Location API defined in 3GPP TS 29.572 [8] – parameters available from the location reporting are described in the TS). It also requests accuracy to be a parameter present in the response. LMS can also perform a location information request to GMLC directly or via NEF (as defined in 6.1. of 3GPP TS 23.273 [5]), acting as AF. LMS can also perform a location information request to 3rd party location servers via LM-3P interface. LMS can also use the T8 interface to obtain UE location. Or the LMS can have already the UE location based on the methods described in 3GPP TS 23.434 [9]. The received response contains the UE location from the network. LMS compares the location provided from the network with the location provided from the LMC. LMS can use event exposure using NEF according to table 4.15.3.1-1 of TS 23.502 to obtain user location information from AMF.

4.	LMS sends confirm location report to LMC.

5.	The LMS sends notification towards the VAL server with the UE identifier and application using the service.

* * * Next Change * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting Notify_Trigger_Location_Reporting | VAL server | Subscribe/Notify |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |
| SS_ConfirmLocation | Confirm_Location | LMC | Request/Response |
|  | ConfirmLocationServiceSubscription | VAL server | Request/Response |
|  | Notify_ConfirmLocationServiceUsage (NOTE) | VAL server | Subscribe/Notify |
| NOTE: The subscribe operation is part of ConfirmLocationServiceSubscription |  |  |  |

* * * Next Change * * *

### 9.4.x	SS_ConfirmLocation API

#### 9.4.x.1	General

API description: This API enables the VAL server to subscribe for Confirm Location Service and the LMC to use Confirm location service.

#### 9.4.x.2	Confirm Location

API operation name: Confirm_Location

Description: Confirm location service.

Known Consumers: LMC.

Inputs: See subclause 9.3.2.z

Outputs: See subclause 9.3.2.u.

See subclause 9.3.x.3 for the details of usage of this API operation.

#### 9.4.x.3	Confirm Location Service Subscription

API operation name: ConfirmLocationServiceSubscription

Description: Subscription for the Confirm Location Service.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.x

Outputs: See subclause 9.3.2.y

See subclause 9.3.x.2 for the details of usage of this API operation.

#### 9.4.x.4	Notify Confirm Location Service

API operation name: Notify_ConfirmLocationService

Description: Notification about usage of the Confirm Location Service.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.r

Outputs: None

See subclause 9.3.x.3 for the details of usage of this API operation.

* * * End of Changes * * *