3GPP TSG-SA WG6 Meeting #63	S6-244623

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244256)

Source:	Huawei, Hisilicon

Title:	Clarification on low latency signalling transmission connection establishment

Spec:	3GPP TS 23.700-35 v0.0.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yajie Hu (huyajie1@huawei.com)

1. Introduction

Currently, the API defined in SEALDD is not friendly to vertical application, thus this contribution is proposed to corresponding solution corresponding Key issue#X.

2. Reason for Change

Currently, the API defined in SEALDD is not friendly to vertical application:

- The API defined in SEALDD are not easily understood by 3rd party. The functionalities and values of the current APIs are not clear. For example, for the SEALDD transmission connection establishment API, it is not clear whether SEALDD can provide best-effort signaling transmission or low-latency signaling transmission for vertical industries. The vertical industries need to determine specific functions based on the open parameters of the API.

- Currently, one single API supports multiple functions because different parameter combinations in the current API support different functions. In this case, it is difficult for 3rd party to distinguish the necessary parameters required for a particular function, and therefore it is difficult for 3rd  party to invoke a particular function.

Thus the APIs defined in SEALDD need to be optimized.

3. Proposal

It is proposed to agree the following pCR to TR 23.700-35.

* * * First Change * * * *

8.X	Solution#X.2

9.2.2.A	SEALDD enabled low latency signalling transmission connection establishment procedure

Figure 9.2.2.1-1 illustrate the procedure for low latency signalling transmission connection establishment. When VAL server has the low latency requirement on signalling transmission, then the VAL server can invoke low latency signalling transmission connection establishment function of the SEALDD.

This procedure is similar to the SEALDD enabled best effort signalling transmission connection establishment procedure, the differences are as follows:

Figure 9.2.2.1-1: SEALDD enabled low latency signalling transmission connection establishment procedure

1.	When VAL server has low latency requirement on application signalling transmission, then the VAL server decides to use SEALDD service for low latency application signalling transfer and allocates address as SEALDD-S connection information for receiving the application signalling traffic from SEALDD server. The VAL server sends SEALDD enabled low latency signalling transmission connection establishment request to the SEALDD server. The service request includes the VAL server ID, VAL service ID to identify the VAL application signalling traffic, the SEALDD-S connection information of the VAL server, and the latency requirement of the signalling transmission.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server allocates a specific address or port used for SEALDD traffic transfer with the incoming SEALDD client(s) for the VAL server and responds with a SEALDD service response.

Step3-8 are same as procedure defined in clause 9.2.2.1, the only difference is that the SEALDD server and SEALDD client establish low latency signalling transmission connection to transmit application signalling based on the latency requirement of the signalling transmission.

9.2.3.A1	SEALDD enabled low latency signalling transmission connection establishment request

Table 9.2.3.A-1 describes the information flow from the VAL server to the SEALDD server for requesting the low latency signalling transmission connection establishment service.

Table 9.2.3.1-1: SEALDD enabled low latency signalling transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g., IP address and/or port, URL)/port and/or URL of the VAL server to receive the application packets signalling traffic from the SEALDD server |
| Signalling transmission requirement | M | Application signalling transmission requirement provided by VAL server |
| > Requested 5GS Delay |  | M |  | The Requested 5GS Delay provided by the VAL server |  |

9.2.3.A2	SEALDD enabled low latency signalling transmission connection establishment response

Table 9.2.3.A2-1 describes the information flow from the SEALDD server to the VAL server for responding to the low latency signalling transmission connection establishment transmission.

Table 9.2.3.A2-1: SEALDD enabled low latency signalling transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S connection information | O | Address information (e.g., IP address and/or port, URL)/port and/or URL of the SEALDD server to receive the packets signalling traffic from the VAL server for application traffic transfer |
| Cause | O See NOTE | Indicates the reason for the failure. |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * END of Change * * * *