3GPP TSG-SA WG6 Meeting #63	S6-244642

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244209)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on DC application and profile information retrieval

Spec:	3GPP TS 23.392 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the DC application and profile information retrieval procedure.

2. Reason for Change

With the conclusion of FS_eMMTelAPP for KI#7 and KI#2, the MMTel Enabler layer is responsible for the DC application management, the corresponding normative work need to be implementd.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.1.0.

* * * First Change * * * *

### 8.2.4	Getting DC application profile information

#### 8.2.4.1	General

Getting DC application profile information enables a Controlling Application Server to get the DC application profile information from the MMTel Enabler Server.

#### 8.2.4.2	Procedure

Figure 8.2.4.2-1 illustrates the procedure of getting DC application profile information.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

Figure 8.2.4.2-1: Getting DC application profile information

1.	The Controlling Application Server sends a DC Application and Profile information retrieval request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.4.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server gets the DC application Profile information based on the Application ID included in the Getting DC Application Profile information request.

3.	The MMTel Enabler Server sends a Getting DC Application Profile information response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.4.3.2.

#### 8.2.4.3	Information flows

##### 8.2.4.3.1	Getting DC Application profile information request

Table 8.2.4.3.1-1 describes information elements for the Getting DC Application profile information request from the Controlling Application Server to the MMTel Enabler server.

Table 8.2.4.3.1-1: Information elements in Getting DC Application Profile information request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| Application ID list | M | A list of Application ID for the request. |

##### 8.2.4.3.2	Getting DC Application profile information response

Table 8.2.4.3.2-1 describes information elements for the Getting DC Application profile information response from the MMTel Enabler Server.

Table 8.2.4.3.2-1: Information elements in Getting DC Application Profile information response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the Getting DC Application profile information request was successful. |
| > List of DC Application profile information | O | List of DC Application profile information as defined in Table 8.2.1.3.1-2. |
| Failure response | O | Indicates that the Getting DC Application profile information request failed. |
| > Cause | O | Indicates the cause of Getting DC Application profile information request failure. |

* * * End of Changes * * * *