3GPP TSG-SA WG6 Meeting #63	S6-244698

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244515, 4265)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0695 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service provisioning enhancement considering EES onboard |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solution #AE7 is concluded in 23.700-01, and this solution enable UE sequentially connect to corresponding EESs deployed on satellite without performing service provisioning again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhance the EES profile with satellite information - Enhance the service provisioning procedure considering EES deloyed on satellite |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The satellite deolyed case on service provisioning will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.x.A, 11.x.B, 11.x.y.z1, 11.x.y.z2, 11.x.y.z3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

11.x.A	EES determination based on satellite ID Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.x. The service provisioning procedure for satellite EDN is   same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.

When UE connects to the satellite, the service provisioning request message contains the UE serving Satellite ID (e.g. gNB corresponding Satellite ID) which the Satellite is connected by the UE. If the service provisioning request message contains the UE serving Satellite ID, then the ECS can determine EES based on the UE serving Satellite ID and EES Satellite ID.

Alternatively, When UE connects to the satellite, and the service provisioning request message doest not contain the UE serving Satellite ID, then service provisioning response message contains the list of EES along with the EES Satellite ID, then the EEC can determine EES based on the UE serving Satellite ID and EES Satellite ID

11.x.B	EES determination based on coverage information Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.x. The service provisioning procedure for satellite EDN is   same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.

When EEC sends the service request message to the ECS and the when registered EES is deployed on MEO or LEO satellites however the ECS is on the ground, then the ECS determines a list of EES based on the Predicted/Expected UE location, Prediction expiration time, and EES coverage information.

Alternatively, upon receiving the reqesut from the ECS, the ECS sends the service provisioning response which contains a list of EES information which EES is deployed on MEO or LEO satellites, the EEC can determine the EES based on the UE (predicted/expected) location, Prediction expiration time, EES Satellite ID and EES coverage information, then the UE may sequentially connect to corresponding EESs based on ephemeris information of the EESs. When an EES is about to leave the UE’s location, the UE may connect to a next EES in advance to obtain an edge computing service, and does not need to perform EES discovery again

Editor's Note: How the UE can connect to a next EES in advance to obtain an edge computing service without performing service provisioning again is FFS.

In addition, when a serving satellite is providing EES and EAS, and due to discontinuous coverage the same satellite comes back for serving, then the EEC connects to same EES and EAS (i.e. to which the EEC had previously connected to), without needing to discover EES and EAS endpoints again.

Editor's Note: Whether and how the ACR procedure need to be enahanced is FFS.

11.x.y.z1 EES profile

The EES profile is same as clause 8.2.6 with the following additions.

Table 11.x.y.z1-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.2.6-1 |  |  |
| Coverage information | O | The coverage information of the satellite corresponding to the EES along with the Valid Time Period |

11.x.y.z2 Service provisioning request

Table 11.x.y.z2 is same as clause 8.3.3.3.2 Table 11.3.3.3.2-1 with the following additions.

Table 11.x.y.z2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.3.3.3.2-1 |  |  |
| UE serving Satellite ID | O | The idenfier of the satellite which is serving UE in current location. |

* * * END of Change * * * *