3GPP TSG-SA WG6 Meeting #63	S6-244732

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244698, 4515, 4265)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0695 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service provisioning enhancement considering EES onboard |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solution #AE7 is concluded in 23.700-01, and this solution enable UE sequentially connect to corresponding EESs deployed on satellite without performing service provisioning again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhance the EES profile with satellite information - Enhance the service provisioning procedure considering EES deloyed on satellite |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The satellite deolyed case on service provisioning will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.x.A, 11.x.B, 11.x.y.z1, 11.x.y.z2, 11.x.y.z3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

11	Edge Enabler Using Satellite Access

11.1	General

This clause provides clarifications on the procedures and information flows necessary for enabling edge application using satellite access.

* * * NEXT Change * * * *

11.x.A	EES determination based on UE serving satellite ID Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.x. The service provisioning procedure for satellite EDN is   same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.

When UE connects to the satellite, the service provisioning request message contains the UE serving Satellite ID (i.e., serving gNB corresponding Satellite ID). If the service provisioning request message contains the UE serving Satellite ID, then the ECS determines the EES where the Satellite ID in the EES profile matches the UE serving Satellite ID.

When UE connects to the satellite, and the service provisioning request message doest not contain the UE serving Satellite ID, then service provisioning response message contains the list of EES along with the EES Satellite ID, and then the EEC determine EES where the Satellite ID in the EES profile matches the UE serving Satellite ID.

11.x.B	EES determination based on UE location and route Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.x. The service provisioning procedure for satellite EDN is   same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.

When EEC sends the service request message to the ECS and the request message contains the UE location, predicted/expected UE location, Prediction expiration time, then the ECS determines a list of EES where the Satellite coverage availability information in EES profile matches the UE location, predicted/expected UE location, Prediction expiration time, and EES Satellite coverage availability information. Alternatively, the ECS may queiry external server with the UE current location and UE predicted/expected location to obtain a list of Satellite IDs and Trajectory IDs corresponding the UE location and the UE trajectory, then the ECS determines list of EES based on the Satellite IDs and Trajecotry IDs in EES profiles by matching with the Satellite IDs and Trajectory IDs obtained from external server for the UE. The “Expected AC Geographical Service Area” in the AC profile, as specified in clause 8.2.2, is used by the ECS to request the external server (managed by the satellite service provider or operator) for such satellite IDs and trajectory IDs. When ECS sends the service provisioning response which contains a list of EES information which EES is deployed on MEO or LEO satellites, the EEC determines the EES determine EES where the Satellite ID in the EES profile matches the UE serving Satellite ID.

In addition, when a serving satellite is providing EES and EAS, and due to discontinuous coverage the same satellite comes back for serving, then the EEC connects to same EES and EAS (i.e. to which the EEC had previously connected to), without needing to discover EES and EAS endpoints again.

Editor's Note: Whether and how the ACR procedure need to be enahanced is FFS.

11.x.y.z1 EES profile

The EES profile is same as clause 8.2.6 with the following additions.

Table 11.x.y.z1-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.2.6-1 |  |  |
| Dynamic service area indication | O | Indicates if the service area is mobile or not. The service area is dynamic in case of EES on board a MEO or LEO satellite and not dynamic in case of a GEO satellite. |
| Satellite ID | M | The satellite ID which identifies a satellite EDN. |
| Trajectory ID (NOTE x) | O | For mobile EES, it is linked with the trajectory of the satellite and therefore identifies the trajectory of the EES on board a MEO and LEO satellites. |
| Satellite coverage availability information | O | The Satellite coverage availability information of the satellite where the EES is onboard as described in 3GPP TS 23.501[2] |
| NOTE x: The assignment of the trajectory ID for the EES and linking it to an actual trajectory of the satellite can be done by the operator and/or the satellite service provider. |  |  |

11.x.y.z2 Service provisioning request

Table 11.x.y.z2 is same as clause 8.3.3.3.2 Table 11.3.3.3.2-1 with the following additions.

Table 11.x.y.z2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.3.3.3.2-1 |  |  |
| UE serving Satellite ID | O | The idenfier of the satellite which is serving UE in current location. |
| UE location (NOTE x) | O | The location information of the UE. The UE location is described in clause 7.3.2, or Present UE location (e.g. latitude and longitude) for a reference grid point. |
| NOTE x: This IE is not an additional IE but update an existing IE in Table 8.3.3.3.2-1. |  |  |

Editor's Note: Whether and how the Satellite Connectivity Information is required is FFS.

Editor's Note: Whether and how the UE serving Satellite ID can be obtained is FFS.

* * * END of Change * * * *