3GPP TSG-SA WG6 Meeting #64	S6-245575

Orlando, United States, 18th – 22nd November 2024	revision of S6-245068

Source:	TNO

Title:	AI/ML model information update

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	nassima.toumi@tno.nl

1. Introduction

ML model information is defined in Table 8.11.4.1-2, and is used to store and discover AI/ML models. This paper updates and introduces new IEs to further facilitate model discovery.

2. Reason for Change

This paper updates and introduces new IEs:

- New training information IE: This IE provides information on the data that has been used to train the model, and the ID of the base model in case of Transfer Learning. This can be useful to determine whether the model needs to be re-trained, for example if the data is old, or if the base model that has been used for Transfer Learning is not accurate anymore.

- Update observed accuracy IE to observed performance information: Accuracy is one of many metrics that can be used to evaluate a model, as it only measures the true positive rate, other metrics such as recall and the F1 score include additional metrics such as true negatives, false positives and false negatives. Additionally, these metrics are used for models that perform detection or prediction, whereas other models might be used to provide decisions, and their performance is based on application-specific KPIs.

- Update ML model interoperability information IE to include input and output size and format to allow the proper use of the model.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

#### 8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| ML model | O (NOTE) | Provides the ML model. |
| ML model address | O (NOTE) | Represents the ML model address that can be used by Model repository to download the ML model. |
| ML model information | M | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE: At least one of these information elements shall be provided. |  |  |

Table 8.11.4.1-2: ML model information

| Information element | Status | Description |
| --- | --- | --- |
| ML model identifier | M | An identifier for the ML model |
| Analytics ID | O | Represents analytics ID for which the model can be used. |
| VAL service ID(s) | O | Identify the VAL service ID(s). |
| Domain | O | Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location prediction, etc.). |
| List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to retrieve the ML model and thereby also are interoperable to the model. |
| ML model interoperability information | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| ML Model phase | O (NOTE 1) | Represents the ML model phase, e.g., in training, trained, re-training, deployed. |
| > Observed performance | O (NOTE 2) | Provides information on the performance of the model e.g. accuracy, recall, F1score, or application-specific KPIs (if ML model is in trained or deployed phase). |
| > Training information | O (NOTE 2) | If the ML model is in trained or deployed phase: Information on the data that has been used to train the model (e.g. data sources, volume, freshness), and the base model ID in case of Transfer Learning. |
| ML model storage and discovery requirements | O (NOTE 1) | Represents the requirements for the ML repository for the ML model storage and discovery. |
| > Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| > Security and access requirements | O | Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs). |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available. |  |  |

* * * End of Changes * * * *