3GPP TSG-SA WG6 Meeting #65	S6-250033

Athens, Greece, 17th – 21st February 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0002 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor retrieve operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 8.5.2.5, there is Editor's Note: Adding get operation along with procedure is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add feature for spatial anchor retrieve operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Keep Editor's Note, miss feature for spatial anchor retrieve operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3.x	Spatial anchor retrieve

8.3.x.1	General

Spatial anchor retrieve enables a SAn client or VAL server to get spatial anchor(s) with the SAn server.

8.3.x.2	Procedure

Figure 8.3.x.2-1 illustrates the spatial anchor retrieve procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.x.2-1: Spatial anchor obtainment

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor retrieve request to the SAn server. The request includes a requestor identifier, security credentials, identifier of spatial anchors to get as described in clause 8.3.x.3.1.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized and the spatial anchor identifier provided in the request is found, the SAn server gets the spatial anchor information.

3.	The SAn server sends a spatial anchor retrieve response to the requestor with the the spatial anchors information.

8.3.x.3	Information flows

8.3.x.3.1	Spatial anchor retrieve request

Table 8.3.x.3.1-1 describes information elements for the spatial anchor retrieve request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.1-1: Spatial anchor retrieve request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| List of spatial anchors ID | M | List of identifiers of the spatial anchors to retrieve. |

8.3.x.3.2	Spatial anchor retrieve response

Table 8.3.x.3.2-1 describes information elements for the spatial anchor retrieve response from the SAn server to the SAn client or VAL server.

Table 8.3.x.3.2-1: Spatial anchor retrieve response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor retrieve request was successful. |
| List of spatial anchors | O | List of obtained spatial anchors. |
| > Spatial anchor ID | M | Identifier of the obtained spatial anchor. |
| > Position information | M | Position information of the spatial anchor |
| >> Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE) | O | Position information of the spatial anchor within the spatial map. It includes the identifier of the Spatial map, identifier of spatial map layer(s) and the corresponding localization information (The localization information is specific to spatial map format). |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor. |
| Failure response | O | Indicates that the spatial anchor retrieve request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: If Position information is included, at least one of the IEs shall be present. |  |  |

# * * * * Second change * * * *

8.5.2.5	Retrieve operation

API operation name: SS_SAnManagement_retrieve

Description: The consumer requests to retrieve spatial anchors.

Known Consumers: VAL server, SAn client.

Inputs: See Table 8.3.x.3.1-1.

Outputs: See Table 8.3.x.3.2-1.

See clause 8.3.x for details of usage of this operation.

# * * * * End of changes * * * *