3GPP TSG-SA WG6 Meeting #65	S6-250034

Athens, Greece, 17th – 21st February 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0003 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor usage information report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 8.4, there is Editor's Note: Whether and how the SAn client reports spatial anchor usage information and/or spatial anchor services accessed information to the SAn server is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add feature for spatial anchor usage information report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Keep Editor's Note, miss feature for spatial anchor usage information report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x, 8.4.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.4.x	Spatial anchor usage information on-demand report

8.4.x.1	General

This clause enables SAn server to request spatial anchor usage information on-demand report from the SAn client.

8.4.x.2	Procedure

Figure 8.4.x.2-1 depicts the procedure for spatial anchor usage information on-demand report.

Figure 8.4.x.2-1: Spatial anchor usage information on-demand report

1)	The SAn server sends a request message to the SAn client to get the spatial anchor usage information on-demand report as per Table 8.4.x.3.1-1.

2)	The SAn client provides the spatial anchor usage information on-demand report. The response message includes Identity of the VAL client for SAn client, list of spatial anchors, spatial anchors’ usage information.

8.4.x.3	Information flows

8.4.x.3.1	Spatial anchor usage information on-demand report request

Table 8.4.x.3.1-1 describes information elements for the spatial anchor usage information on-demand report request from the SAn server to the SAn client.

Table 8.4.x.3.1-1: Spatial anchor usage information on-demand report request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., SAn server). |
| Security credentials | M | The security credentials of the requestor. |
| On-demand report time | M | Indicates the on-demand report duration time for SAn client to report the spatial anchor usage information |

8.4.x.3.2	Spatial anchor usage information on-demand report response

Table 8.4.x.3.2-1 describes information elements for the spatial anchor usage information on-demand report response from the SAn client to the SAn server.

Table 8.4.x.3.2-1: Spatial anchor usage information on-demand report response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > VAL client ID | M | Identity of the VAL client for SAn client |
| > List of spatial anchors | M | List of spatial anchors to be reported. |
| >> Spatial anchor ID | M | Identifier of the spatial anchor. |
| >> Spatial anchor usage information | O | Indicates spatial anchor usage information. It includes number of times spatial anchor discovered and accessed, spatial anchor services accessed information. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

# * * * * Second change * * * *

8.4.y	Spatial anchor usage information periodic report

8.4.y.1	General

This clause enables SAn server to request spatial anchor usage information periodic report from the SAn client.

8.4.y.2	Procedure

Figure 8.4.y.2-1 depicts the procedure for spatial anchor usage information periodic report.

Figure 8.4.y.2-1: Spatial anchor usage information periodic report

1.	The SAn server sends a request message to the SAn client to get the spatial anchor usage information periodic report as per Table 8.4.y.3.1-1. It may include the periodicity for reporting notification and sampling interval.

2.	The SAn client sends a spatial anchor usage information periodic report response to the SAn server.

3.	The SAn client samples the information related to spatial anchor usage as per the sampling interval. It provides notification to the SAn server as per the reporting interval. The notification message includes Identity of the VAL client for SAn client, list of spatial anchors, spatial anchors’ usage information.

8.4.y.3	Information flows

8.4.y.3.1	Spatial anchor usage information periodic report request

Table 8.4.y.3.1-1 describes information elements for the spatial anchor usage information periodic report request from the SAn server to the SAn client.

Table 8.4.y.3.1-1: Spatial anchor usage information periodic report request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., SAn server). |
| Security credentials | M | The security credentials of the requestor. |
| Sampling interval | O | Indicates the interval to sample the spatial anchor usage information |
| Reporting interval | O | Indicates the periodic reporting interval of spatial anchor usage information |
| notification endpoint | O | Indicates the endpoint(e.g., IP address, FQDN, URI) to send the spatial anchor usage information notification |

8.4.y.3.2	Spatial anchor usage information periodic report response

Table 8.4.y.3.2-1 describes information elements for the spatial anchor usage information periodic report response from the SAn client to the SAn server.

Table 8.4.y.3.2-1: Spatial anchor usage information periodic report response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

8.4.y.3.3	Spatial anchor usage information periodic report notification

Table 8.4.y.3.3-1 describes information elements for the spatial anchor usage information periodic report notification from the SAn client to the SAn server.

Table 8.4.y.3.3-1: Spatial anchor usage information periodic report notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL client ID | M | Identity of the VAL client for SAn client |
| List of spatial anchors | M | List of spatial anchors to be reported. |
| > Spatial anchor ID | M | Identifier of the spatial anchor. |
| > Spatial anchor usage information | O | Indicates spatial anchor usage information. It includes number of times spatial anchor discovered and accessed, spatial anchor services accessed information. |

# * * * * Third change * * * *

8.4.2	Spatial anchor usage information (Sub-Notify)

8.4.2.1	General

This clause enables VAL server to request spatial anchor usage information from the SEAL SAn server.

8.4.2.2	Procedure

Figure 8.4.2.2-1 depicts the procedure for subscribing to receive information related to spatial anchor usage for metaverse applications. The service is provided by SAn server.

Figure 8.4.2.2-1: Spatial anchor usage information subscription

1.	A consumer (e.g. VAL server) sends a spatial anchor usage information subscription request to the SA server). The request can include the VAL server identifier, security credentials, a notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor filters like spatial anchor identifiers, spatial anchor location information, position of the user, proximity distance, and application service identifier. It may also include the periodicity for reporting notification and sampling interval.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and sends a spatial anchor usage subscription response to the consumer.

3.	The SAn server samples the information related to spatial anchor usage related information based on the obtained data and the stored spatial anchor data as per the sampling interval. It provides response to the consumer as per the reporting interval. The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information.

# * * * * End of changes * * * *