3GPP TSG-SA WG6 Meeting #65	S6-250035

Athens, Greece, 17th – 21st February 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0004 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor usage information broadcast report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 8.4, there is Editor's Note: Whether and how the SAn client reports spatial anchor usage information and/or spatial anchor services accessed information to the SAn server is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add feature for spatial anchor usage information broadcast report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Keep Editor's Note, miss feature for spatial anchor usage information broadcast report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.4.x	Spatial anchor usage information broadcast report

8.4.x.1	General

This clause enables SAn server to request spatial anchor usage information broadcast report from the SAn clients.

8.4.x.2	Procedure

Figure 8.4.x.2-1 depicts the procedure for spatial anchor usage information broadcast report.

Pre-conditions:

1.	The SAn clients have already set with SAn server broadcast channel information (pre-configuration or after registration, e.g. broadcast notification URL, callback URL, etc.); and

2.	The SAn clients have received security credentials authorizing them to communicate with the SAn server.

Figure 8.4.x.2-1: Spatial anchor usage information broadcast report

1)	The SAn server sends a broadcast message to the SAn clients by broadcast channel to get the spatial anchor usage information report as per Table 8.4.x.3.1-1. The report type can be on-demand or periodic.

2)	The SAn clients provides the spatial anchor usage information report. The response message includes Identity of the VAL client for SAn client, report type, list of spatial anchors, spatial anchors’ usage information.

8.4.x.3	Information flows

8.4.x.3.1	Spatial anchor usage information report broadcast

Table 8.4.x.3.1-1 describes information elements for the spatial anchor usage information report broadcast from the SAn server to the SAn clients.

Table 8.4.x.3.1-1: Spatial anchor usage information report broadcast

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., SAn server). |
| Security credentials | M | The security credentials of the requestor. |
| Report type | M | Report type: on-demand or periodic. |
| On-demand report | O | Indicates the on-demand report for SAn client to report the spatial anchor usage information |
| > Duration time | O | Indicates the on-demand report duration time for SAn client to report the spatial anchor usage information |
| Periodic report | O | Indicates the periodic report for SAn client to report the spatial anchor usage information |
| > Sampling interval | O | Indicates the interval to sample the spatial anchor usage information |
| > Reporting interval | O | Indicates the periodic reporting interval of spatial anchor usage information |

8.4.x.3.2	Spatial anchor usage information report callback

Table 8.4.x.3.2-1 describes information elements for the spatial anchor usage information report callback from the SAn clients to the SAn server.

Table 8.4.x.3.2-1: Spatial anchor usage information report callback

| Information element | Status | Description |
| --- | --- | --- |
| VAL client ID | M | Identity of the VAL client for SAn client |
| Report type | M | Report type: on-demand or periodic. |
| List of spatial anchors | M | List of spatial anchors to be reported. |
| > Spatial anchor ID | M | Identifier of the spatial anchor. |
| > Spatial anchor usage information | O | Indicates spatial anchor usage information. It includes number of times spatial anchor discovered and accessed, spatial anchor services accessed information. |

# * * * * End of changes * * * *