3GPP TSG-SA WG6 Meeting #65	S6-250036

Athens, Greece, 17th – 21st February 2025	(revision of S6-xxxxxx)

Source:	ZTE Corporation

Title:	Fix for clause 8.3.3

Spec:	3GPP TS 23.438 v1.0.0

Agenda item:	9.12

Document for:	Approval

Contact:	Shao Weixiang, shao.weixiang@zte.com.cn

1. Introduction

This contribution provides fix for clause 8.3.3.

2. Reason for Change

In clause 8.3.3, there are “Asset identifier”, “Asset name”, “Asset type”, “Owner user”, “Discovered asset profile identifier”,  “Asset status”,  “Asset descriptions”. It is not inconsistent with terms and clause 8.2 Digital asset profile management.

3. Proposal

It is proposed to approve these changes for 3GPP TS 23.438 v1.0.0.

* * * First Change * * * *

8.3.3	Information flows

8.3.3.1	Digital asset discovery request

Table 8.3.3.1-1 describes the information elements for a digital asset discovery request sent from a requestor (e.g. DA client or VAL server) to a DA server.

Table 8.3.3.1-1: Digital asset discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Requesting asset identifier | M | The identifier of the asset of the requestor. |
| Asset discovery filters | M | Set of characteristics to determine matching asset (e.g., Digital asset identifier, Digital asset name, Digital asset description, location, VAL client type, Digital asset type, Digital asset owner, Allowed user, Digital asset profile). |

8.3.3.2	Digital asset discovery response

Table 8.3.3.2-1 describes the information elements for the digital asset discovery response sent from a DA server to a requestor (e.g. DA client or VAL server).

Table 8.3.3.2-1: Digital asset discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |
| Discovery results | M | List of discovery results matching the discovery criteria. |
| > Digital asset identifier | O (NOTE) | Identifier of the digital asset matching the discovery criteria. |
| > Digital asset name | O (NOTE) | Name of the digital asset matching the discovery criteria. |
| > Discovered digital asset profile identifier | O (NOTE) | Profile identifier of the digital asset matching the discovery criteria. |
| > URL | O | A URL of the service provider the digital asset is associated with. |
| > Digital asset status | M | Status of the digital asset. |
| > Digital asset description | O | Description of the digital asset. |
| NOTE: At least one of the information elements must be present. |  |  |

Editor's Note: Media aspects associated with digital assets are FFS.

* * * End of Changes * * * *