3GPP TSG-SA WG6 Meeting #65	S6-250041

Athens, Greece 17th – 21st February 2025

Source:	Airbus

Title:	Initial analysis of TR23784 for FS_MCLOG_Ph2

Agenda Item:	10

Contact:	Jukka Vialen jukka.vialen@airbus.com

Abstract: This discussion paper provides an initial analysis of what can be utilized from TR 23.784 (Rel-16) in the FS_MCLOG_Ph2 Rel-20 study.

# Scenarios (Clause 4)

#### General

- TR 23.784 does not contain recording (or logging) specific scenarios.

- It contains only one subclause in the scenarios section that is related to recording/logging:

  - Clause 4.8 / Scenario 7: Logging scenarios – where it says: “The call related scenarios can be considered to be the same as those described in scenarios 1 to 6 related to discreet listening, except that the recipient of the information is a logging client or interface instead of the authorized user”.

- The “scenarios 1 to 6” are analysed in more detail below.

#### MCPTT and MCVideo private calls and group calls

- TR23.784:

  - Clause 4.2.1 / Scenario 1.1: Discreet listening of MCPTT and MCVideo private calls in the same MC service server

  - Clause 4.3 / Scenario 2: Discreet listening of MCPTT and MCVideo group calls

- Analysis:

  - Recording solutions for these scenarios already exist in Rel-19 SA6 TSs i.e. there is no need to add them into FS_MCLOG_Ph2.

- Action:

  - no action needed

#### MCPTT and MCVideo private calls and group calls with target receiving service from different MC service server

- TR23.784:

  - Clause 4.2.2 / Scenario 1.2: Discreet listening of MCPTT and MCVideo private calls with target receiving service from different MC service server

  - Clause 4.3 / Scenario 2: Discreet listening of MCPTT and MCVideo group calls / NOTE 3

- Analysis:

  - These scenarios in TR 23.784 look like they have been written with a specific solution in mind:

    - “…the request for discreet listening is sent by MC service client A to MC service server A…”

    - “…MC service server A requests the discreet listening function from MC service server B…”

  - These ‘assumptions’ are not valid for the recording architecture/solution in Rel-19 TSs

  - The issue/topic is valid for FS_MCLOG_Ph2, but the scenario must be rewritten

  - A better title would be “recording of migrated users”

- Action:

  - Rewrite the whole scenario. Rename it to “Recording of migrated users” and include both private calls and group calls. Use the recording architecture in TS 23.280 Rel-19 as a starting point. If necessary, split into ‘sub’-scenarios.

#### One to one MCVideo pull and push

- TR23.784:

  - Clause 4.4.1 / Scenario 3.1 Discreet listening of one to one MCVideo pull

  - Clause 4.5.1 / Scenario 4.1 Discreet listening of one to one MCVideo push

- Analysis:

  - As described in these clauses (in TR 23.784), one to one Video pull/push is a MCVideo private call with specific parameter values.

  - A key issue for Rel-20 is to study if the Rel-19 solution for MCX server – Recording server interface (REC-4 reference point / SIPREC) can be used also for MCVideo pull/push or if any additions are required.

- Action:

  - The scenarios can be copied into FS_MCLOG_Ph2, some changes/rewording is necessary to adapt them with (SA6) Rel-19 recording architecture and terminology.

  - A KI (see ‘analysis’ above) can be added. A possible solution could be that nothing needs to be added as the recording of these scenarios is already covered by SIPREC, but this must be confirmed. If no changes/additions are needed then the ‘solution’ only needs to add text to relevant places describing that also these services can be recorded – e.g. the NOTE in TS23280/7.5.2.34.

#### One from server MCVideo pull and one to server MCVideo push

- TR23.784:

  - Clause 4.4.2 / Scenario 3.2 Discreet listening of one from server MCVideo pull

  - Clause 4.5.2 / Scenario 4.2 Discreet listening of one to server MCVideo push

- Analysis:

  - These are valid scenarios for FS_MCLOG_Ph2.

  - These scenarios in TR 23.784 look like they have been written with a specific solution in mind.

  - They need to be rewritten; the starting point must be recording architecture/solution in Rel-19 TSs.

- Action:

  - Rewrite these scenarios, use Rel-19 recording architecture as a starting point.

#### Remotely initiated MCVideo push

- TR23.784:

  - Clause 4.5.3 / Scenario 4.3: Discreet listening of remotely initiated MCVideo push

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

  - Like all other scenarios in TR 23.784, this has been written with a specific solution in mind

  - This needs to be rewritten; the starting point must be recording architecture/solution in Rel-19 TSs.

- Action:

  - Rewrite the scenario, use Rel-19 recording architecture as a starting point.

#### MCData SDS

- TR23.784:

  - Clause 4.6 / Scenario 5: Discreet listening of MCData SDS

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

  - Like all other scenarios in TR 23.784, this has been written with a specific solution in mind

  - This needs to be rewritten; the starting point must be recording architecture/solution in Rel-19 TSs.

- Action:

  - Rewrite the scenario, use Rel-19 recording architecture as a starting point.

#### MCData file upload and file download

- TR23.784:

  - Clause 4.7.1 / Scenario 6.1: Discreet listening of file upload and file download

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

  - TR 23.784 clause 4.7.1 says: “This scenario is similar to those for MCVideo push and MCVideo pull from server described in subclauses 4.4.2 and 4.5.2 of the present document”.

  - The above sentence needs to be verified.

- Action:

  - Verify the claim in TR 23.784/clause 4.7 (see above).

  - If true, follow the logic in TR 23.784 i.e. for “MCData file upload/download” a title + dedicated scenario number is enough, with reference to related MCVideo scenarios.

  - If not true, rewrite the scenario, use Rel-19 recording architecture as a starting point.

#### MCData one to one file distribution

- TR23.784:

  - Clause 4.7.2 / Scenario 6.2: Discreet listening of one to one file distribution

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

  - Like all other scenarios in TR 23.784, this has been written with a specific solution in mind

  - This needs to be rewritten; the starting point must be recording architecture/solution in Rel-19 TSs.

- Action:

  - Rewrite the scenario, use Rel-19 recording architecture as a starting point.

#### Scenarios for non-communication related activities

- TR23.784:

  - Nothing

- Analysis:

  - Required for FS_MCLOG_Ph2.

- Action:

  - Add new scenario(s)

#### Scenarios for remotely controlled MCVideo service

- TR23.784:

  - Nothing

- Analysis:

  - TBD, waiting for decisions on S6-250028.

- Action:

  - TBD.

# Key Issues (Clause 5)

#### Selection of MC service

- TR23.784:

  - Clause 5.1 / Key Issue 1: Selection of MC service for discreet listening

- Analysis:

  - In TR 23.784 this KI is for discreet listening only, but it is a valid question also for recording

  - Granularity with the Rel-19 solution is already at a service level i.e. MCPTT / MCData / MCVideo recording can be set independently for each user.

  - If granularity within each MC service (PTT/Video/Data) is needed, that should be a requirement from SA1.

- Action:

  - To be decided: either no action or LS to SA1?

#### End to end encrypted calls

- TR23.784:

  - Clause 5.5 / Key Issue 5: Logging and replay of end to end encrypted calls

- Analysis:

  - During Rel-19 work for recording, this topic was discussed with SA3 delegates.

  - In informal discussions it was clarified that:

    - The e2e key (PCK) for private calls, created in the call initiating MCX UE, can be fetched and decrypted (by MCX server or Recording server) with key material available from KMS. This assumption is valid at least when both MCX UEs (clients) use the same KMS.

    - For group calls, GMK is sent from GMS to the MC UEs in the group. I.e. the same GMK can be provided also to MCX server or Recording server.

- Action:

  - Re-check the Rel-19 TS(s).

  - If corrections are needed -> CR to Rel-19.

  - If the issue is not a FASMO -> new KI+solution to Rel-20 TR

#### Migrated users

- TR23.784:

  - Clause 5.7 / Key Issue 7: Discreet listening and logging for interconnected MC system

  - Clause 5.8 / Key Issue 8: Discreet listening and logging for migrated MC service user

- Analysis:

  - The target for recording is always a user (not MC system) i.e. the KI should be about migrated users (KI8), which includes all the variations described under 5.7 and 5.8 of TR 23.784.

  - The text in 5.7 and 5.8 can be partially utilized, together with the Rel-19 recording architecture to rewrite these KIs.

  - One KI may be enough, but the issues can also be split into multiple KIs, this is TBD.

- Action:

  - New KIs needed, after the relevant scenario(s) is(are) defined.

#### Recording of a user receiving MC service(s) on multiple devices

- TR23.784:

  - Clause 5.9 / Key Issue 9: Discreet listening and logging for user receiving MC service on multiple devices

- Analysis:

  - Since recording is a server function and based on MC service IDs, this topic is not be an issue

  - TR 23.784 does not have any dedicated solution for this KI. In the solutions part the whole topic (multiple devices) is not even mentioned.

  - Implicitly solved in Rel-19 recording architecture / configurations, no KI needed in Rel-20.

- Action:

  - No action needed

#### Recording of MC service users and MC service groups

- TR23.784:

  - Clause 5.10 / Key Issue 10: Logging of MC service users and MC service groups

- Analysis:

  - Already solved in Rel-19.

- Action:

  - No action needed.

#### Recording of MCData content server and message store services

- TR23.784:

  - Clause 5.11 / Key Issue 11: Logging of MCData content server and message store services

- Analysis:

  - Relevant new topic in FS_MCLOG_Ph2.

- Action:

  - KI or KIs needed, after the relevant scenario(s) is(are) defined.

# Solutions (Clause 6)

#### Functional model, configuration and procedures

- TR23.784:

  - Clause 6.3 / Solution 3: Functionality for on-network logging and replay

  - Clause 6.10 / Solution 10: Functionality for on-network logging and replay

- Analysis:

  - Application plane functional models:

    - The system level on-network func.model TS 23.280: solved in R19, no action needed

    - The system level off-network func.model TS 23.280: update needed

    - The service level on-network func.models (TS 23.379/281/282): update needed

    - The service level off-network func.models (TS 23.379/281/282): update needed

  - Configurations:

    - solved in R19 for on-network recording

    - configs for off-network recording missing

  - Procedures:

    - on-network recording - done in R19

    - off-network recording – missing

  - The existing text in Solution 3 and Solution 10 is not usable as it is based on ‘wrong’ recording architecture.

- Action:

  - Several actions needed (see above). Relevant KI (or KIs) needed before solutions.

#### Interconnection and migration

- TR23.784:

  - Clause 6.4 / Solution 3: On-network logging for interconnection and migration

- Analysis:

  - Relevant topic for FS_MCLOG_Ph2.

  - This solution can be used to get ideas what is needed in a solution description, but it cannot be copied to the new TR since the recording architecture is different.

- Action:

  - Solution for “migrated users” or “interconnection and migration” must be added after relevant KI (or KIs) exist.

#### MCData message store and MCData content server

- TR23.784:

  - Clause 6.7 / Solution 7: Logging of MCData message store and MCData content server

- Analysis:

  - Relevant topic for FS_MCLOG_Ph2.

  - KI / KIs are needed first.

  - This solution can be used as a starting point but it must be adapted to the R19 recording architecture/configurations/procedures. Some new/enhancements may be needed between MCData server and Recording server.

- Action:

  - Solution(s) needed, but KI/KIs must be done first.

# General observations of TR 23.784

General:

- it is obvious that TR 23.784 was produced in the order: 1) discreet listening 2) logging

  - Scenarios: 7 for DL, 1 for Logging

  - Key Issues: 11 for DL, 3 for Logging

  - Solutions: 6 for DL, 3+1 for Logging (“3+1” = two overlapping solutions for one issue)

  - NOTE: some of the clauses for Logging refer to multiple clauses for DL i.e. ‘similar’ scenario/KI/solution is applicable to Logging than for DL. This saves some pages but makes reading of this TR difficult.

- Mapping of KIs to Solutions exists in TR 23.784, but mapping of SA1 user requirements to KIs and/or Solutions is missing. This needs to be added to ensure that we will cover all user requirements in this Rel-20 study.

Scenarios:

- the scenarios in this TR are written with a specific architecture on mind

- they have too much text and partially contain already KIs and even solutions

- preference for the FS_MCLOG_Ph2 (Rel-20) TR is that scenarios are as short as possible, just explaining the required functionality per each SA1 user requirement, not speculating on possible procedures, configurations or other details that belong to KIs or solutions

- all on-network scenarios shall be based on the Rel-19 recording architecture in TS 23.280

- each scenario should mention the related SA1 user requirement (this mapping is missing from TR 23.784)

Key Issues:

- the scenarios in this TR are written with a specific architecture on mind

Solutions:

- Solutions may contain ‘extra’ text (explanations, different options, …) in the first phase of the study, but before TR approval, all text and tables in (selected) solutions should be edited into a format that is as close to a normative CR as possible:

  - in each (selected) solution, the impacted TS and the clause in that TS shall be mentioned. It is much faster and easier to finalize all details of the solutions in a TR than in the normative CR phase.

  - The final TR version shall not contain ‘options’ inside (selected) solutions. This is one issue with TR 23.784.

- should provide clear (unambiguous) starting point for normative CRs.