3GPP TSG-SA WG6 Meeting #65	S6-250049

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0361 | rev | - | Current version: | 19.4.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of response elements in the 5GSAT procedure in TS23.434 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_APP |  |  |  |  |  | Date: |  |  | 2025-2-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution adds details about the response elements in the 5GSAT procedure. While some other sections (such as clause 14.3.2) include Cause in the response, NTN-related sections do not. So, this information was added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When there is a failure in the response, the cause is being appended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The response elements are inadequate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.2.3.2, 21.3.3.2, 21.3.3.5, 21.3.3.7, 21.3.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

21.2.3.2	Satellite coverage availability information configuration response

Table 21.2.3.2-1 describes the information flow from the configuration management client to the configuration management server for the response of the satellite coverage availability information configuration request.

Table 21.2.3.2-1: Satellite coverage availability information configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| Result | M | Indicates the request result. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

21.3.3.1	Satellite S&F events subscription request

Table 21.3.3.1-1 describes the information flow from VAL server to NRM server or from the NRM server to NRM client for satellite S&F events subscription request.

Table 21.3.3.1-1: Satellite S&F status events subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Satellite S&F events | O | Lists of the satellite S&F events that to be monitored for one or more UEs (e.g. UE status in S&F mode, the DL S&F estimated delivery time). |
| Triggering conditions | M | Identifies the conditions that triggered NRM client sending the notification report. |

21.3.3.2	Satellite S&F events subscription response

Table 21.3.3.2-1 describes the information flow from NRM server to the VAL server or from NRM client to NRM server for response of satellite S&F events subscription request.

Table 21.3.3.2-1: Satellite S&F events subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Subscription status | M | It indicates the subscription result |
| Cause | O | The cause for the request failure. |

21.3.3.3	Satellite S&F events notification

Table 21.3.3.3-1 describes the information flow from NRM server to VAL server or from the NRM client to NRM server to notify the subscribed satellite S&F events.

Table 21.3.3.3-1: Satellite S&F status events notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Satellite S&F events | O | List the notification for satellite S&F events that to be monitored for one or more UE(s). |
| > UE status in S&F mode | O (NOTE) | Indicates the UE is registered in S&F mode or moving from S&F mode to not registered in S&F mode. |
| > The DL S&F estimated delivery time | O (NOTE) | Indicates the estimated/expected time required to deliver the data to the UE from the time data has been received in the network. |
| Triggering events | M | Identifies the events that triggered NRM client sending the report. |
| Timestamp | O | Timestamp of the notification report. |
| NOTE: The definition refers to the clause 5.6.3 of 3GPP TS 23.682 [13]. |  |  |

21.3.3.4	Satellite S&F events unsubscribe request

Table 21.3.3.4-1 describes the information flow from VAL server to NRM server or from the NRM server to NRM client for satellite S&F events unsubscribe request.

Table 21.3.3.4-1: Satellite S&F status events unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requesting ID for VAL server or NRM server. |

21.3.3.5	Satellite S&F events unsubscribe response

Table 21.3.3.5-1 describes the information flow from NRM server to the VAL server or from NRM client to NRM server for response of satellite S&F events unsubscribe request.

Table 21.3.3.5-1: Satellite S&F events unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | It indicates the unsubscribe request result. |
| Cause | O | The cause for the request failure. |

21.3.3.6	Report UE satellite information request

Table 21.3.3.6-1 describes the information flow from the NRM client to the NRM server for reporting the UE satellite related capabilities.

Table 21.3.3.6-1: Report UE satellite information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| UE type | O | Indicate the type of VAL UEs (e.g. IoT). |
| Satellite information | O | The satellite information of VAL UE when the VAL UE supports the satellite access. |
| > Satellite ID | O | Indicates the serving satellite ID for the VAL UE. |
| > Satellite type | O | Indicates the accessing satellite RAT types (LEO, MEO and GEO) for the VAL UE. |
| > S&F mode indicator | O | Indicates the UE support the S&F mode or not. |
| > Maximum data storage quota | O | Indicates the maximum data storage quota for the VAL UE for all of services on the application layer. |
| > Maximum data retention | O | Indicates the maximum data retention for the VAL UE per service on the application layer. |

21.3.3.7	Report UE satellite information response

Table 21.3.3.7-1 describes the information flow the NRM server to the NRM client for the response of reporting the UE satellite related information request.

Table 21.3.3.7-1: Report UE satellite information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | It indicated the request result. |
| Cause | O | The cause for the request failure. |

21.3.3.8	Satellite S&F configuration request

Table 21.3.8-1 describes the information flow from the NRM server to the NRM client for satellite S&F configuration request.

Table 21.3.3.8-1: Satellite S&F configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Satellite S&F configuration information | O | Indicates the S&F configuration per UE per service when it is using the satellite S&F operation. |
| > Maximum S&F data storage quota | O | Indicates the maximum data storage quota per UE per service on the application layer when the UE is using the satellite S&F operation. |
| > Maximum S&F data retention | O | Indicates the maximum data retention per UE per service on the application layer when the UE is using the satellite S&F operation. |

21.3.3.9	Satellite S&F configuration response

Table 21.3.3.9-1 describes the information flow from the NRM client to the NRM server for the response of satellite S&F configuration request.

Table 21.3.3.9-1: Satellite S&F events subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL users or VAL UEs or NRM server. |
| Result | M | It indicates the request result |
| Cause | O | The cause for the request failure. |

* * * End of Changes * * * *