3GPP TSG-SA WG6 Meeting #65	S6-250050

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0241 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of response elements in the CAPIF procedure in TS23.222 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-2-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution adds details about the response elements in the CAPIF procedure. While the cause of failure is described clause 8.25, there is no mention of the cause prior to that section. So, this corrections were made. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When there is a failure in the response, the cause is being appended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The response elements are inadequate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2, 8.6.2.2, 8.8.2.1, 8.8.2.2, 8.8.2.6, 8.8.2.8, 8.15.2.2, 8.17.2.2, 8.19.2.2, 8.23.2.2, 8.25.2.2, 8.25.2.4, 8.25.2.6, 8.25.2.8, 8.25.2.10, 8.25.2.12, 8.25.2.15, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.2.2	Service API get response

Table 8.5.2.2-1 describes the information flow service API get response from the CAPIF core function to the API publishing function.

Table 8.5.2.2-1: Service API get response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of retrieving the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O (see NOTE) | The service API information as specified in Table 8.3.2.1-1. |
| NOTE: Shall be present if the Result information element indicates that the service API get request is successful. Otherwise service API information shall not be present. |  |  |

* * * Next Change * * * *

8.6.2.2	Service API update response

Table 8.6.2.2-1 describes the information flow service API update response from the CAPIF core function to the API publishing function.

Table 8.6.2.2-1: Service API update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. |

* * * Next Change * * * *

8.8.2.2	Event subscription response

Table 8.8.2.2-1 describes the information flow for event subscription response from the CAPIF core function to the subscribing entity.

Table 8.8.2.2-1: Event subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event subscription operation |
| Cause | O | The cause for the request failure. |
| Subscription identifier | O (see NOTE) | The unique identifier for the event subscription. |
| NOTE: Shall be present if the Result information element indicates that the event subscription operation is successful. Otherwise subscription identifier shall not be present. |  |  |

8.8.2.3	Event notification

Table 8.8.2.3-1 describes the information flow for event notification from the CAPIF core function to the subscribing entity. A notification about an event is sent to a subscribing entity if the event criteria in the related subscription match the corresponding attributes of the event content.

Table 8.8.2.3-1: Event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the event subscription |
| Event identifier | M | The unique identifier for the event. For the list of events, refer subclause 8.8.6 |
| Event related information | M | The event related information (e.g. time at which the event originated, location of event) |
| Event content | M | The content of the event information. |

8.8.2.4	Event notification acknowledgement

Table 8.8.2.4-1 describes the information flow event notification acknowledgement from the subscribing entity to the CAPIF core function.

Table 8.8.2.4-1: Event notification acknowledgement

| Information element | Status | Description |
| --- | --- | --- |
| Acknowledgement | M | Acknowledgement for the event notification received. |

8.8.2.5	Event unsubscription request

Table 8.8.2.5-1 describes the information flow for event unsubscription request from the subscribing entity to the CAPIF core function.

Table 8.8.2.5-1: Event unsubscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | The information to determine the identity of the subscribing entity |
| Subscription identifier | M | The unique identifier for the event subscription that was provided to the subscribing entity during the CAPIF event subscription operation. |

8.8.2.6	Event unsubscription response

Table 8.8.2.6-1 describes the information flow for event unsubscription response from the CAPIF core function to the subscribing entity.

Table 8.8.2.6-1: Event unsubscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event unsubscription operation |
| Cause | O | The cause for the request failure. |

8.8.2.7	Event subscription update request

Table 8.8.2.7-1 describes the information flow for event subscription update request from the subscribing entity to the CAPIF core function.

Table 8.8.2.7-1: Event subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | The information to determine the identity of the subscribing entity |
| Subscription identifier | M | The unique identifier for the event subscription that was provided to the subscribing entity during the CAPIF event subscription operation. |
| Event criteria changes | O (NOTE) | Updates to the event criteria which are defined in clause 8.8.2.1 |
| Notification reception information changes | O (NOTE) | Updates to the information of the subscribing entity for receiving the notifications for the event |
| NOTE: At least one of these information elements shall be present. |  |  |

8.8.2.8	Event subscription update response

Table 8.8.2.8-1 describes the information flow for event subscription update response from the CAPIF core function to the subscribing entity.

Table 8.8.2.2-1: Event subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event subscription update operation |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.15.2.2	Service API invocation response

The information flow service API invocation response from the AEF to the API invoker is service API specific and the complete detail of the service API invocation response is out of scope of the present document. Table 8.15.2.2-1 describes only the CAPIF related information elements which are included in the service API invocation response.

Table 8.15.2.2-1: Service API invocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of service API invocation. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.17.2.2	Service API invocation response

The information flow service API invocation response from the AEF to the API invoker is service API specific and the complete detail of the service API invocation response is out of scope of the present document. Table 8.17.2.2-1 describes only the CAPIF related information elements which are included in the service API invocation response.

Table 8.17.2.2-1: Service API invocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of service API invocation. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.19.2.2	API invocation log response

Table 8.19.2.2-1 describes the information flow API invocation log response from the CAPIF core function to the API exposing function.

Table 8.19.2.2-1: API invocation log response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of API(s) invocation log request |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.23.2.2	Revoke API invoker authorization response

Table 8.23.2.2-1 describes the information flow revoke API invoker authorization response from the CAPIF core function to the API exposing function or from the API exposing function to the CAPIF core function.

Table 8.23.2.2-1: Revoke API invoker authorization response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of revoke API invoker authorization. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.25.2.2	Interconnection API publish response

Table 8.25.2.2-1 describes the information flow interconnection API publish response from CAPIF core function to CAPIF core function.

Table 8.25.2.2-1: Interconnection API publish response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of publishing the service API information |
| Cause | O | The cause for the request failure. |
| Service API published information reference | O (see NOTE) | The information which can be used for referencing the information (set) about the published service API by the CCF which publishes service APIs |
| NOTE: This information element is included when the Result indicates success. |  |  |

8.25.2.3	Interconnection service API discover request

Table 8.25.2.3-1 describes the information flow interconnection service API discover request from one CAPIF core function to another CAPIF core function.

Table 8.25.2.3-1: Interconnection service API discover request

| Information element | Status | Description |
| --- | --- | --- |
| CAPIF core function identity information | M | Identity information of the CAPIF core function discovering service APIs |
| Query information | M | Criteria for discovering matching service APIs or CAPIF core function (e.g. service API category, Serving Area Information (optional), UE IP address (optional), preferred AEF location (optional), required API provider name (optional), interfaces, protocols, Service KPIs (optional), and Network Slice Info (optional)) (see NOTE) |
| NOTE: It should be possible to discover all the service APIs. |  |  |

8.25.2.4	Interconnection service API discover response

Table 8.25.2.4-1 describes the information flow interconnection service API discover response from one CAPIF core function to another CAPIF core function.

Table 8.25.2.4-1: Interconnection service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O (see NOTE) | List of service APIs corresponding to the request, including service API information as specified in Table 8.25.2.1-1. |
| CAPIF core function identity information | O (see NOTE) | Indicates the CAPIF core function matching the query criteria |
| NOTE: The service API information or the CAPIF core function identity information or both shall be present, if the Result information element indicates that the interconnection service API discover operation is successful. Otherwise, both shall not be present. |  |  |

8.25.2.5	Interconnection API unpublish request

Table 8.25.2.5-1 describes the information flow interconnection API unpublish request from a CAPIF core function to another CAPIF core function.

Table 8.25.2.5-1: Interconnection API unpublish request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function which unpublishes service APIs, may include identity, authentication and authorization information |
| Service API published information reference (see NOTE) | M | The information which can be used for referencing the information (set) about the published service API by the CCF which publishes service APIs |
| NOTE: Obtained during the interconnection API publish procedure in clause 8.25.3.1. |  |  |

8.25.2.6	Interconnection API unpublish response

Table 8.25.2.6-1 describes the information flow interconnection API unpublish response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.6-1: Interconnection API unpublish response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of unpublishing the service API information |
| Cause | O | The cause for the request failure. |

8.25.2.7	Interconnection get service API request

Table 8.25.2.7-1 describes the information flow interconnection get service API request from a CAPIF core function to another CAPIF core function.

Table 8.25.2.7-1: Interconnection get service API request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function that wants to retrieve service APIs information. It may include identity, authentication and authorization information |
| Service API published information reference (see NOTE) | M | The information which can be used for referencing the information (set) about the published service API by the CCF which publishes service APIs |
| NOTE: Obtained during the interconnection API publish procedure in clause 8.25.3.1. |  |  |

8.25.2.8	Interconnection get service API response

Table 8.25.2.8-1 describes the information flow interconnection get service API response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.8-1: Interconnection get service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of getting the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O (see NOTE) | The service API information includes the service API name, service API category, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional) , and Network Slice Info (optional). |
| NOTE: Shall be present if the Result information element indicates that the interconnection get service API request is successful. Otherwise, service API information shall not be present. |  |  |

8.25.2.9	Interconnection update service API request

Table 8.25.2.9-1 describes the information flow interconnection update service API request from a CAPIF core function to another CAPIF core function.

Table 8.25.2.9-1: Interconnection update service API request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CCF requesting the update operation. It may include identity, authentication and authorization information |
| Service API information | O (see NOTE 1) | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional and applicable only on CAPIF-6 interface), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), and Network Slice Info (optional). |
| Service API category | O (see NOTE 1) | The category of the service APIs to be published, (e.g. V2X, IoT) |
| Shareable information | O (see NOTE 2) | Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained. |
| NOTE 1: Either the Service API information or Service API category shall be present. NOTE 2: If the shareable information is not present, the service API information is not allowed to be shared. There is one and only one CAPIF provider domain information sharable via the CAPIF-6e interface. |  |  |

8.25.2.10	Interconnection update service API response

Table 8.25.2.10-1 describes the information flow interconnection update service API response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.10-1: Interconnection update service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. |

8.25.2.11	Interconnection revoke API invoker authorization request

Table 8.25.2.11-1 describes the information flow interconnection revoke API invoker authorization request from one CCF to another CCF.

Table 8.25.2.11-1: Interconnection revoke API invoker authorization request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the requesting CAPIF core function, may include identity, authentication and authorization information. |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

Editor's note:	It is FFS whether additional IEs need to be included based on SA3 progress on CCF interconnection.

8.25.2.12	Interconnection revoke API invoker authorization response

Table 8.25.2.12-1 describes the information flow interconnection revoke API invoker authorization response from one CCF to another CCF.

Table 8.25.2.12-1: Interconnection revoke API invoker authorization response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of interconnection revoke API invoker authorization. |
| Cause | O | The cause for the request failure. |

8.25.2.13	Interconnection revoke API invoker authorization notify

Table 8.25.2.13-1 describes the information flow interconnection revoke API invoker authorization notify from one CCF to another CCF.

Table 8.25.2.13-1: Interconnection revoke API invoker authorization notify

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the notifying CAPIF core function, may include identity, authentication and authorization information. |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

Editor's note:	It is FFS whether additional IEs need to be included based on SA3 progress on CCF interconnection.

8.25.2.14	Interconnection obtain access control policy request

Table 8.25.2.14-1 describes the information flow interconnection obtain access control policy request from one CCF to another CCF.

Table 8.25.2.14-1: Interconnection obtain access control policy request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | Identity information of the entity requesting the access control policy |
| Service API identification | M | The identification information of the service API for which the access control policy is being requested. |

8.25.2.15	Interconnection obtain access control policy response

Table 8.25.2.15-1 describes the information flow interconnection obtain access control policy response from one CCF to another CCF.

Table 8.25.2.15-1: Interconnection obtain access control policy response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the obtain access control policy operation |
| Cause | O | The cause for the request failure. |
| Access control policy information | O (see NOTE) | The access control policy information corresponding to the requested service API. (See Table E-1) |
| NOTE: Shall be present if the Result information element indicates that the obtain access control policy operation is successful. Otherwise access control policy information shall not be present. |  |  |

* * * End of Changes * * * *