3GPP TSG-SA6 Meeting #65	S6-250054

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0004 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Aligning AIMLE client discovery with the AIMLE client registration IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the AIMLE client discovery procedure in clause 8.8.2.1, step 2, the discovery is mentioned to be done based on the AIMLE client profile. However, this is not the complete story as the list of supported services is also registered by the AIMLE clients and used for discovery. In fact, the AIMLE client profiles are matched for specific services. Therefore, this creates both unclarity and incompleteness. The options are either to move the supported services to AIMLE client profiles or to clarify the AIMLE client discovery. We took the latter approach because different AIMLE services may have different profiles. In addition, there is unnecessary redundancy of “AIMLE client discovery criteria” in step 2 of clause 8.8.2.1, which also needs to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposal adds clarifications that AIMLE client supported services are used to discover clients during AIMLE client discovery in step 2 of clause 8.8.2.1. In addition, this proposal also removes the redundant “AIMLE client discovery criteria” from step 2 of clause 8.8.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be unclear. Furthermore, the AIMLE client discovery procedure will skip a condition during discovery, leading to an incomplete procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.8	AIMLE client discovery

### 8.8.1	General

Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, VAL servers need to discover suitable AIMLE clients to fulfil the requirements for the AI/ML application. The VAL server can then use the discovered AIMLE clients to select a set of AIMLE clients to perform AI/ML operations.

The following clauses specify procedures, information flows, and APIs for AIMLE client discovery.

### 8.8.2	Procedure

#### 8.8.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with AIMLE clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The request may also include AIML client task capability requirements to discover clients who can perform the AIML tasks like AIML model training/offload/split with compute requirements and task performance preference like green task performance. The AIMLE client discovery request includes information as described in Table 8.8.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

The AIMLE server also obtains candidate UEs from the ML repository.

Editor’s Note: Whether Member UE filtering criteria is included in the request is FFS.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e., AIMLE clients) that fulfils the discovery criteria based on supported services and AIMLE client profiles.

The AIMLE server determines whether the discovered UEs fulfil the location information in the discovery criteria by using their identifiers to determine their location and includes in the discovery response only those UEs that meet the requirements. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server may discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

Editor’s Note: Whether AI/ML Policy IDs is included in the request is FFS.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.8.3.2-1. If the required number of AIMLE clients are not included in the response message from the AIMLE server (i.e., the AIMLE clients in the response message are less than the required minimum number of AIMLE clients), then the AIML service consumer may discover the remaining required AIMLE clients from other AIML enablement server indicating the required number of AIMLE clients. The response also includes the information related to supported tasks with guaranteed task KPIs, available AIML tasks service duration, AIML task expected KPI like latency.