3GPP TSG-SA6 Meeting #65	S6-250055

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0005 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLE client selection subscription update and subscription cancel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are no provisions for AIML client selection subscription updates and unsubscriptions. If there is a subscription and no possibility to unsubscribe, the subscription will remain indefinitely, even if it is no longer needed. Additionally, when the criteria from the VAL server change, if the subscription is not updated accordingly, it will result in a stale state that no longer reflects the requirements of the VAL server. Therefore, subscription and update procedures are necessary for AIML client selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provisions for AIML client selection subscription updates and unsubscriptions have been introduced.Corresponding procedures, information elements, and API have also been added. Additionally, new general sections have been introduced, resulting in the renumbering of existing sections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.13.2, 8.13.2.1 (new), 8.13.2.2 (updated), 8.13.2.x (new), 8.13.2.y (new), 8.13.3.1 (new), 8.13.3.2, 8.13.3.3, 8.13.3.4, 8.13.3.w (new), 8.13.3.x (new), 8.13.3.y (new), 8.13.3.z (new), 9.2.8.1, 9.2.8.2, 9.2.9.3, 9.2.8.x (new), 9.2.8.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.13	AIMLE client selection subscription and notification

### 8.13.1	General

AIMLE client selection subscription request and notification enable VAL Servers to subscribe for monitoring and selection of AIMLE clients and receive notification when there is an update on the selected and re-selected AIML client’s status when re-selection is performed according to AIML member selection criteria.

The AIMLE server interacts with the NEF and/or SEAL services to monitor AIML members who meet the selection criteria and obtain their identifiers and configuring the AIML traffic session(s) between the VAL Server and the selected AIMLE Client(s) who meet the criteria. When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed.

### 8.13.2	Procedures

#### 8.13.2.1	General

The following are supported for AIMLE client selection subscription and notification :

-	AIMLE client selection subscription and notification;

- 	AIMLE client selection subscription update; and

-	AIMLE client selection unsubscribe

#### 8.13.2.2 AIMLE client selection subscription and notification

Figure 8.13.2.2-1: AIMLE client selection subscription and notification

1.	A VAL server sends an AIMLE client selection subscription request to the AIMLE Server. The AIMLE client selection subscription request includes information as described in Table 8.13.3-1 which includes selection criteria.

2.	The AIMLE server validates the AIMLE client selection subscription request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to subscribe to the selected AIMLE client selection subscription request.

3.	The AIMLE server sends the AIMLE client selection subscription response to the VAL server.

4.	The AIME server monitors AIMLE clients whether they fulfil the selection criteria as provided in step 1. The AIMLE server interacts with the NEF and/or SEAL services (including SEALDD) to establish monitoring. The AIMLE server utilizes SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.11, or 3GPP TS 23.434 [5] clause 9.3.12) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to establish monitoring of UEs entering or present in the target location provided in the location information in the selection criteria.

5.	The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients which do not fulfil the selection criteria. The AIMLE server uses the location monitoring for selecting UEs that fulfil the location criteria and removing UEs which cease to fulfil the location criteria as provided in the location information in the AIMLE client selection criteria. The AIMLE Server may determine the application QoS parameters (e.g. bandwidth, latency, jitter) for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed.

The AIMLE server may determine the application QoS parameters based on the VAL Service ID.

If a desired service in the selection criteria is ceased to be provided by the client or its profile change so that it no longer meets the selection criteria, the AIML server removes the AIMLE clients which ceases to fulfil the criteria and selects other clients that fulfil selection criteria.

NOTE 1:	To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1.

NOTE 2:	In case of AIMLE server determining the application QoS parameters based on the VAL service ID, how it does this is up to implementation.

6.	The AIMLE Server notifies the VAL server about the selected and re-selected AIMLE clients e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B.

Editor’s Note: Service operation definition in the API clause is FFS.

* * * Next Change * * * *

#### 8.13.2.x AIMLE client selection subscription update

Figure 8.13.2.x-1 illustrates the procedure for an VAL server to update a subscription with the AIMLE server.

Pre-conditions:

1.	The AIMLE client or VAL server has subscribed for AIMLE client selection with the AIMLE Server.

Figure 8.13.2.x-1: AIMLE client selection subscription update

1.	A VAL server sends a AIMLE client selection subscription update request to the AIMLE server. The AIMLE client selection subscription update request includes the subscription identifier and may include information as described in Table 8.13.3.w-1 for updated subscription.

2.	Upon receiving the request from the VAL server, the AIMLE server validates if the VAL server is authorized for the request. If the VAL server is authorized, the AIMLE server updates the subscription.

3.	The AIMLE server sends a AIMLE client selection subscription update response to the VAL server. If the AIMLE server has updated the subscription, the response includes an indication of success. If the AIMLE server has not updated the subscription, the response includes an indication of failure and may include a reason for failure.
If the subscription update request in step 1 include updated information as in Table 8.13.3.w-1, the AIMLE server also adjusts step 4 and 5 as in clause 8.13.2.2 based on the updated information in the subscription update.

* * * Next Change * * * *

#### 8.13.2.y AIMLE client selection unsubscribe

Figure 8.13.2.y-1 illustrates the procedure for a VAL server to unsubscribe with the AIMLE server.

Pre-conditions:

1.	The VAL server has subscribed for AIMLE client selection with the AIMLE Server.

Figure 8.13.y-1: AIMLE client selection unsubscribe

1.	A VAL server sends a AIMLE client selection unsubscribe request to the AIMLE server. The request includes the subscription identifier.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the VAL server is authorized for the request. If the VAL is authorized, the AIMLE server unsubscribes the subscription.

3.	The AIMLE server sends a AIMLE client selection unsubscribe response to the VAL server. If the AIMLE server has unsubscribed the subscription, the response includes an indication of success. If the AIMLE server has not unsubscribed the subscription, the response includes an indication of failure and may include a reason for failure.
If the AIMLE server has successfully unsubscribed the subscription, it also cancels its corresponding subscriptions with the NEF and/or SEAL services (including SEALDD) associated with the subscription. Additionally, the AIMLE server reverses the QoS adjustments performed as described in step 5 of clause 8.13.2.2.

* * * Next Change * * * *

### 8.13.3	Information flows

#### 8.13.3.1	General

The following information flows are specified for AIMLE client registration:

-	AIMLE client selection subscription and notification request and response;

-	AIMLE client selection subscription update request and response; and

-	AIMLE client selection unsubscribe request and response

#### 8.13.3.2	AIMLE client selection subscription request

Table 8.13.3.2-1 shows the request sent by a VAL server to an AIMLE server for the Selected AIMLE Client selection subscription.

Table 8.13.3.2-1: AILME client selection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client selection criteria | M | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | M | Represents the endpoint at the VAL server for receiving the notifications on the selected AIMLE Client’s status update. |

#### 8.13.3.3	AIMLE client selection subscription response

Table 8.13.3.3-1 shows the response sent by the AIMLE server to the VAL server for the selected AIMLE client selection subscription response.

Table 8.13.3.3-1: AIMLE client selection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription was successful. |
| > Subscription ID | M | The identifier of the subscription. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription status, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated subscription never expires. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

#### 8.13.3.4	AIMLE client selection update notification

Table 8.13.3.4-1 shows the notification sent by the AIMLE Server to the VAL server for the AIMLE client selection update notification.

Table 8.13.3.4-1: AIMLE client selection update notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor. |
| List of the selected AIMLE Client status update events | M | Represents the list of selected AIMLE Client status update events, e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B. |

#### 8.13.3.w	AIMLE client subscription update request

Table 8.13.3.w-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client selection subscription update request.

Table 8.13.3.w-1: AIMLE client selection subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the existing subscription for which the update request applies. |
| AIMLE client selection criteria | O | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | O | Represents the endpoint at the VAL server for receiving the notifications on the selected AIMLE Client’s status update. |

#### 8.13.3.x	AIMLE client selection subscription update response

Table 8.13.3.x-1 shows the AIMLE client selection subscription update response sent by the AIMLE server to the VAL server.

Table 8.13.3.x-1: AIMLE client selection subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription status, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated subscription never expires. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure. |

#### 8.13.3.y	AIMLE client selection unsubscribe request

Table 8.13.3.y-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client selection unsubscribe request.

Table 8.13.3.y-1: AIMLE client selection unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the existing subscription for which the unsubscribe request applies. |

#### 8.13.3.z	AIMLE client selection unsubscribe response

Table 8.13.3.z-1 shows the AIMLE client selection unsubscribe response sent by the AIMLE server to the VAL server.

Table 8.13.3.z-1: AIMLE client selection unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the unsubscribe request was successful. |
| Failure response | O | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure. |

* * * Next Change * * * *

### 9.2.8	AIMLE client Selection Subscribe API

#### 9.2.8.1	General

Table 9.8.1-1 illustrates the API for AIMLE client selection subscription for enabling a VAL server to monitor and select AIMLE clients for participation in AI/ML operations.

Table 9.8.1-1: Aimles_ClientSelection API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimles_ClientSelection | Subscribe | Subscribe/Notify | VAL Server |
|  | Notify |  |  |
|  | Update |  |  |
|  | Unsubscribe |  |  |

#### 9.2.8.2	Aimles_ClientSelection_subscribe operation

API operation name: Aimles_ClientSelection_subscribe

Description: The consumer subscribes for monitoring and selection of AIMLE clients for AI/ML operations.

Inputs: See clause 8.13.3.2.

Outputs: See clause 8.13.3.3.

See clause 8.13.2.2 for details of usage of this operation.

#### 9.2.8.3	Aimles_ClientSelection_notify operation

API operation name: Aimles_ClientSelection_notify

Description: The consumer is notified of selected AIMLE client status update events.

Inputs: See clause 8.13.3.4.

Outputs: None.

See clause 8.13.2.3 for details of usage of this operation.

#### 9.2.8.x	Aimles_ClientSelection_update operation

API operation name: Aimles_ClientSelection_update

Description: The consumer updates selection of AIMLE clients for AI/ML operations.

Inputs: See clause 8.13.3.w.

Outputs: See clause 8.13.3.x.

See clause 8.13.2.x for details of usage of this operation.

#### 9.2.8.y	Aimles_ClientSelection_unsubscribe operation

API operation name: Aimles_ClientSelection_ unsubscribe

Description: The consumer unsubscribes of selected AIMLE clients for AI/ML operations.

Inputs: See clause 8.13.3.y.

Outputs: See clause 8.13.3.z.

See clause 8.13.2.y for details of usage of this operation.

* * * End of Changes * * * *