3GPP TSG-SA WG6 Meeting #65	S6-250062

Athens, Greece 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0010 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional AIMLE identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 7 on AIML identifiers is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add definition for ML model profile ID and AIMLE client set ID. Add example identifiers to the FL member ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will lack a clear definition of the key AIML service identifiers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6, 7.x & 7.y. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.6	FL member ID

The FL member ID uniquely identifies the participant entity in a Federated Learning process which is supported by AIMLE service, e.g., AIMLE server ID, VAL server ID or EAS ID.

* * * Second Change * * * *

## 7.x	ML model profile ID

The ML model profile ID uniquely identifies a ML model profile that is created by the ML repository as a result of a successful ML model storage request by a AIMLE server.

* * * Third Change * * * *

## 7.y	AIMLE client set ID

The AIMLE client set ID is a unique identifier assigned by an AIMLE server that is associated with the set of AIMLE clients that have been selected and have agreed to participate in performing AI/ML related operations, e.g., ML model training.

* * * End of Changes * * * *