3GPP TSG-SA WG6 Meeting #65	S6-250064

Athens, Greece 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0243 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for Revoking Resource Owner Authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The solution #11 of 3GPP TR 23.700-22 was concluded to be considered as candidate for normative work for revoking resource owner authentication. This CR proposes implementing this solution in 3GPP TS 23.222. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding a procedure to describe the communication between resource owner function, CAPIF core function, API invoker and AEF for revoking resource owner authentication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be no standardized mechanism for revoking resource owner authentication, affecting the integrity of the authorization process. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new), 8.x.1 (new), 8.x.2 (new), 8.x.2.1 (new), 8.x.2.2 (new), 8.x.2.3 (new), 8.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

8.X	Revoking resource owner authorization

8.X.1	General

The procedure in this subclause corresponds to the architectural requirements for revoking resource owner authorization.

8.X.2	Information flows

Editor’s Note:	The security aspects of this procedure are to be detailed by SA3.

8.X.2.1	Revoke resource authorization request

Table 8.X.2.1-1 describes the information flow revoke resource authorization request from the resource owner function to the CAPIF core function.

Table 8.X.2.1-1: Revoke resource authorization request

| Information element | Status | Description |
| --- | --- | --- |
| Resource owner identity information | M | The information that determines the identity of the resource owner (e.g., GPSI). |
| API invoker identity information | M | The information that determines the identity of the API invoker. |
| Service API identification | M | The identification information of the service API for which the resource authorization is revoked. |
| Cause | M | The cause for revoking the resource authorization |

8.X.2.2	Revoke resource authorization response

Table 8.23.2.2-1 describes the information flow revoke resource authorization response from the CAPIF core function to the resource owner function.

Table 8.X.2.2-1: Revoke resource authorization response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of resource authorization. |

8.X.2.3	Revoke resource authorization notify

Table 8.23.2.3-1 describes the information flow revoke resource authorization notify from the CAPIF core function to the API invoker or from the CAPIF core function to the API exposing function.

Table 8.X.2.3-1: Revoke resource authorization notify

| Information element | Status | Description |
| --- | --- | --- |
| Resource owner identity information | M | The information that determines the identity of the resource owner (e.g., GPSI). |
| API invoker identity information | M | The information that determines the identity of the API invoker. |
| Service API identification | M | The identification information of the service API for which the resource authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

8.X.3	Procedure

Figure 8.X.3-1 illustrates the procedure for revoking resource owner authorization to access service API initiated by the resource owner function.

Pre-conditions:

- The API invoker is authenticated and authorized to use the service API.

- Authorization details of the resource owner are available with the CAPIF core function.

Figure 8.X.3-1: Procedure for revoking resource owner authorization

1.	The resource owner function triggers the revocation of the resource authorization.

2.	The AEF sends revoke resource authorization request to the CAPIF core function with the details of the resouce owner, the API invoker and the service API.

3.	Upon receiving the information to revoke the resource authorization for service API invocation, the CAPIF core function invalidates the resource authorization corresponding to the service API.

4.	The CAPIF core function sends a revoke resource authorization response to the resource owner function.

5.	Upon successful revocation of resource authorization corresponding to the service API at the CAPIF core function, the resource owner function invalidates the resource authorization corresponding to the service API.

6.	The CAPIF core function sends a revoke resource authorization notify to the API invoker whose resource authorization to access the service API has been revoked.

7.	The CAPIF core function sends a revoke resource authorization notify to the API exposing function whose resource authorization to access the service API has been revoked.

* * * End of Changes * * * *