3GPP TSG-SA WG6 Meeting #65	S6-250066

Athens, Greece 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0245 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource Owner authorization Management for Nested API Invocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The solution #2 of 3GPP TR 23.700-22 was concluded to be considered as candidate for normative work for resource owner authorization management for nested API invocation. This CR proposes enhancing current procedure for reducing authorization information inquiry in a nested API invocation in 3GPP TS 23.222 based on these solutions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhancing current procedure (8.32 Reducing authorization information inquiry in a nested API invocation) to obtain resource owner authorization in step 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the scenario of nested API invocation, AEF1 lacks the resource owner authorization information when obtaining the authorization to access AEF2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.32.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

8.32.3		Procedure

Figure 8.32.3-1 illustrates the procedure to obtain authorization information in a nested API invocation, in which an API exposing function receiving the service API invocation request interacts with another API exposing function to provide the service.

Pre-conditions:

1.	The resource owner function can communicate with the API invoker.

2.	The API exposing functions 1 and 2 are in the same trust domain.

Figure 8.32.3-1: Procedure for obtaining authorization information in a nested API invocation

1.	The API invoker requests authorization information to invoke the service API exposed by API exposing function 1.

NOTE 1:	This step may use either the existing procedure to obtain authorization to access service API specified in clause 8.11 or the procedure that involves the resource owner function to get authorization information. For the latter case, the mechanisms to support interactions with the Resource owner are specified in 3GPP TS 33.122 [12], with further possible CAPIF support for these mechanisms being out of scope of the current release.

2.	The API invoker sends a service API invocation request to API exposing function 1 with the authorization information received in step 1.

3.	Based on the service API invocation request, API exposing function 1 decides to invoke another service API exposed by API exposing function 2.

4.	API exposing function 1, acting as an API invoker, obtains the authorization information to access the service API exposed by API exposing function 2.

NOTE 2:	To obtain the authorization information further interaction with the API invoker can be avoided if API exposing function 1 exchanges the authorization information provided in the API invoker's request (in step 2) with authorization information for accessing to access the service API exposed by API exposing function 2. The security mechanism involved in this exchange are out of scope of the present document.

5.	API exposing function 1, acting as an API invoker sends a service API invocation request to API exposing function 2 with the authorization information received in step 4.

6.	The API exposing function 1 receives the service API invocation response resulting from the service API invocation once API exposing function 2 has checked whether the API invoker is authorized to invoke that service API based on the authorization information.

7.	The API invoker receives the service API invocation response resulting from the service API invocation.

* * * End of Changes * * * *