3GPP TSG-SA WG6 Meeting #65	S6-250070

Athens, Greece, 17th – 21st Feb 2025	(revision of S6-245xxx)

Title:	LS on Dynamic Generation of AF specific UEID

Release:	Rel-20

Work Item:	EDGEAPP, SNAAPP

Source:	3GPP TSG SA WG6

To:	SA2

Cc:	CT3

Contact person:	Shahram Mohajeri, sm7084@att.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

# 1	Overall description

As part of the EDGEAPP work item, SA6 in 23.558 clause 8.6.5 defined the “UE Identifier API” which resulted in having SA2 specify NEF’s “AF specific UE ID retrieval” procedure in 23.502 clause 4.15.10 (and associated Nnef_UEId_Get operation). AF specific UE ID is a GPSI in External ID format in order to protect user’s privacy when used by external AFs over NEF northbound APIs to target a UE.

Step 10 of the “AF specific UE ID retrieval” procedure as specified in 23.502 clause 4.15.10, doesn’t specify the case where UDM may not be able to find the AF specific UE ID in the subscriber’s record (see step 10 of clause 4.15.10 as copy/pasted below):

23.502- Figure 4.15.10-1: AF specific UE ID retrieval

“10. The UDM responds to the NEF with an AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the Application Port ID, MTC Provider Information or AF Identifier.”

On the other hand, stage 3 in 29.522 clause 4.4.32.2 “Retrieve AF specific UE ID service operation” states that UDM would reply to NEF with a "404 Not Found" error status code if AF specific UE ID is not available (see related paragraph from clause 4.4.32.2 copy/pasted below):

“If the UDM indicates that the requested UE Identifier is not available in the subscription data, the NEF shall respond to the AF with a "404 Not Found" error status code.”

Given that 23.502 doesn’t specify how “AF Specific UEIDs” are created and stored in the subscriber’s record in UDM/UDR, may lead one to assume that, “AF Specific UEIDs” may be preloaded in subscriber records which is not a workable solution due to the hundred millions number of subscriber records and dynamic nature of AFs being onboarded on to the MNOs CAPIF/NEF platform and also dynamic nature of “AF Specific UEIDs” which maybe removed per subscriber’s request.

Additionally, leaving it unspecified (as to how AF Specific UEIDs are generated), doesn’t help the industry which needs a consistent and reliable method of retrieving such an opaque UEID which on one hand avoids user privacy issues (vs using MSISDN over NEF APIs) and on the other can reliably be used by the AF to identify the UE across PDU sessions (vs UE IP address which changes).

App developers need a consistent behavior from NEF APIs across MNOs. Hence, leaving it unspecified leads to inconsistencies in NEF API implementations and in turn diminishes the AF Specific UEID’s usefulness for AFs using SA6 defined APIs (e.g. “UE Identifier API” defined in EDGEAPP and general-purpose northbound APIs used as part of SNAAPP).

Given the above, there is a need for a mechanism enabling the dynamic generation and storage of the AF Specific UEIDs when it doesn’t exist in UDM/UDR upon Nnef_UEId_Get invoction.

Such an enhancement to “AF specific UE ID retrieval” procedure (as defined in 23.502 clause 4.15.10) will increase the success rate of NEF northbound API usage by AFs which rely on “AF Specific UEID” availability.

# 2	Actions

To SA2

ACTION: 	SA6 asks SA2 to kindly consider the enhancement needed in “AF specific UE ID retrieval” procedure (as defined in 23.502 clause 4.15.10) which would enable the dynamic generation and storage of AF specific UE ID if one doesn’t already exist in UDM/UDR.

# 3	Dates of next TSG SA WG 6 meetings

SA6#66                 7th – 11th April 2025 		Gothenburg, Sweden

SA6#67                 19th – 23rd May 2025 		Fukuoka, Japan