3GPP TSG-SA WG6 Meeting #65	S6-250073

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on corrections on the DC application profiles downloading procedure

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to do some corrections as specified in reason for change.

2. Reason for Change

Correct some editorial and text issues, e.g. the description of step1 in the figure was missing.

Move the definition of the root application to the beginning.

Uniform the terminology of root application, some are lowercase, some are capitalized.

The operation after downloading the DC Application Profile list is unclear, this PCR propose to add some clarification.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v1.0.0.

* * * First Change * * * *

#### 8.3.2.2	Procedure

Figure 8.3.2.2-1 illustrates the DC application profiles downloading procedure.

Pre-conditions:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.2.2-1: MMTel Enabler client in the UE requests the Data Channel Application profile list

The root application is e.g. layout, and/or home page etc. of the Data Channel Application that the subscriber could select and use, as specified in GSMA NG.134 [5].

When the MMTel service provider provides a root application, step 1, step 2 and step 3 apply: Otherwise the step 1, step 2 and step 3 are skipped. Further, if root application is locally stored in the MMTel Enabler client and if root application validity is not expired then step 1, step 2 and step 3 are skipped.

1.	The MMTel Enabler client in the UE sends a Get root application request to MMTel Enabler server to get the root application. The request message includes information elements as specified in Table 8.3.2.3.1-1. The application type is used by MMTel Enabler server to suggest DC applications for download.

NOTE 1:	in a implementation, the DCSF may support the DC application profile functionality of MMTel Enabler server function, the DCSF may also interact with the MMTel Enabler server to support this procedure, i.e. The MMTel Enabler client sends a request to the DCSF, which then sends the request to the MMTel Enabler Server. The response in the opposite direction is also sent to the MMTel Enabler client through the DCSF.

Editor’s note:	if the DCSF interacts with the MMTel Enabler server to support this procedure, whether the  interactions between DCSF and MMTel Enabler server are needed to be specified or implementation is FFS.

2.	The MMTel Enabler server checks whether the root application version included in the Get root application request is the newest version of the root application. If the root application version IE is not included, the root application version is not considered as newest.

3.	The MMTel Enabler server sends a Get root application response to the MMTel Enabler client in the UE. The response message includes information elements as specified in Table 8.3.2.3.2-1.

If root application is locally stored in the MMTel Enabler cslient, validity of the application will be updated to the value received in root application validity IE.

NOTE 2:	When service provider wants the UE to delete the stored root application, the response message includes negative value in the root application validity IE.

4.	The MMTel Enabler client in the UE send a Get Application Profile List request to MMTel Enabler server to get the Data Channel Application profile list available for this user. The request message includes information elements as specified in Table 8.3.2.3.3-1.

5.	The MMTel Enabler server sends a  Get Application Profile List response to the MMTel Enabler client in the UE. The response message includes information elements as specified in Table 8.3.2.3.4-1. The response message includes list of Data Channel Application profile.

The MMTel Enabler client locally stores the downloaded DC application profiles and displays the related application information from the DC application profile on the root application. Users can select and download corresponding DC applications based on the profile information on the root application. The MMTel Enabler client may stores the downloaded DC Application which may be obsoleted in the condition of, e.g. the validity is expired, update available for application, etc.

* * * End of Changes * * * *