3GPP TSG-SA WG6 Meeting #65	S6-250076

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on overview

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the overview descriptions.

2. Reason for Change

The overview clause was missed in the current specification.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v1.0.0.

* * * First Change * * * *

4	Overview

For the application enablement aspects of MMTel service, The MMTel Enabler layer can provide MMTel clients with DC Application Profiles for controlling DC application download behavior and also can expose services to vertical service providers.

- Application Provider uploads the DC Application and Profile(s) to the Controlling Application Server. The Controlling Application Server may send received DC Application and Profile(s) to the MMTel Enabler Server. The procedures are specified in clause 8.2.

The MMTel Enabler server generates customized DC Application Profile(s). The DCMTSI Client downloads DC Application based on the customized DC Application Profile get from the MMTel Enabler server. the procedures are specified in clause 8.3.

- The MMTel enabler layer exposes services towards the Vertical service providers. The exposed capabilities include the services of the MMTel Enabler layer, which orchestrate OMA capabilities and IMS Core Network capabilities. MMTel service enablement procedures are specified in clause 8.4.

* * * End of Changes * * * *