3GPP TSG-SA WG6 Meeting #65	S6-250077

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on remove the element of application type in the Get Root application request

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to remove the element of application type in the Get Root application request.

2. Reason for Change

The element of application type is included in the Get Application Profile List request, it was not necessary in the Get Root application request. The root application is not differentiated by application type, e.g. emergency, conference, XR, etc.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v1.0.0.

* * * First Change * * * *

##### 8.3.2.3.1	Get Root application request

Table 8.3.2.3.1-1 describes information elements for the Get Root application request from the MMTel Enabler client to the MMTel Enabler server.

Table 8.3.2.3.1-1: Information elements in Get Root application request

| Information element | Status | Description |
| --- | --- | --- |
| UE Information (see NOTE) | M | The UE related information required to identify the Root application (e.g. device type, device vendor, etc) |
| Root application version | O | The newest version of the Root application stored in the MMTel Enabler client in the UE. This IE presents if Root application is locally stored in the MMTel Enabler client. |
| NOTE: The MMTel service provider can provide the MMTel Enabler client with different Root application based on this IE. |  |  |

* * * End of Changes * * * *