3GPP TSG-SA WG6 Meeting #65	S6-250079

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on uniform the terminology of MMTel Enabler Server and MMTel Enabler Client

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to uniform the terminology of MMTel Enabler Server and MMTel Enabler Client.

2. Reason for Change

The terminology of MMTel Enabler Server and MMTel Enabler Client in this specification is not uniform, some are lowercase, some are capitalized, this pCR proposes to uniform the terminology.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v1.0.0.

* * * First Change * * * *

## 6.2	MMTel Enabler architecture

Figure 6.2-1 shows the MMTel Enabler architecture.

The MMTel Enabler Server is hosted within the PLMN operator network which supports the MMTel services capabilities exposure.

Figure 6.2.1-1: MMTel Enabler architecture

The MMTel functional entities on the UE and the server sides are MMTel Enabler Client and MMTel Enabler Server respectively.

The MMTel Enabler Client communicates with the MMTel Enabler Server over the MMTel-1 reference point to support the application layer interactions needed in MMTel service.

NOTE	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

The MMTel Enabler Client also communicates with the DCMTSI client over the MMTel-4 reference point to downloading behavior on DCMTSI Client based on the application policy provided by the MMTel Enabler Server. The DCMTSI client is specified in 3GPP TS 26.114 [4].

The Application Server communicates with the MMTel Enabler Server over the MMTel-2 reference point to invoke the MMTel service related capabilities provided by MMTel Enabler Server,

The Controlling Application Server communicates with the MMTel Enabler Server over the MMTel-3 reference point to manage the information needed in MMTel service.

The Application Server communicates with the Application Client on the UE over the App-1 reference point to provide the Data Channel Application specific logic.

The MMTel Enabler Client(s) interacts with SEAL Clients over the SEAL-C reference point specified for each SEAL service. The MMTel Enabler Server(s) interacts with SEAL Servers over the SEAL-S reference point specified for each SEAL service. The interaction between a SEAL Client and the corresponding SEAL Server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [6].

* * * Next Change * * * *

## 6.3	Functional entities

### 6.3.1	General

The functional entities for MMTel Enabler are described in the following subclauses.

Editor's note:	 whether more functionalities of the MMTel Enabler Client and the MMTel Enabler Server are needed is FFS.

### 6.3.2	MMTel Enabler Server

The MMTel Enabler Server provides the server side functionalities corresponding to the MMTel service.

The MMTel Enabler Server supports:

-	the Application Server and Controlling Application Server to discover, to authenticate and to get authorization the MMTel/MMTel service APIs provided by the MMTel Enabler Server;

-	the interaction with Controlling Application Server to configure the data channel application profile;

-	the integration of the multiple network layer capabilities provided by the underlying network, including the 3GPP core network and IMS into one service enabler layer API. The MMTel Enabler Server exposes these service enabler layer APIs to the Application Server and Controlling Application Server instead of exposes the network layer capabilities directly. The capabilities provided by the underlying network include:

a)	the APIs provided by DCSF via DC3/N33 reference point;

b)	the APIs provided by DCSF via DC4 reference point; and

c)	the APIs provided by MF/MRF via MDC3/MDC4 reference points;

-	the communication with the corresponding functions in the underlying network on behalf of the Application Server and Controlling Application Server when the capabilities provided by the underlying network are needed to be invoked;

-	the interaction with MMTel Enabler Client for exchanging service configuration related information, e.g. the data channel application profile, needed in MMTel service,

-	providing MMTel/MMTel specific value added services which can be used by the Application Server in the mmtel communication between UE and Application Server;

-	the interaction with SEAL functionalities to use the corresponding SEAL services in MMTel service, e.g. to use  SEAL notification management service to update of data channel application profiles to UE;

-	the storage of the application layer information associated with the MMTel services;

-	the media function to support the media handling/translation, e.g. acts as a media gateway of WebRTC or if supported media type of application is different with the UE; and

-	the HTTP Proxy to support the establishes a Bootstrap data channel and application data channel, e.g. if the address type of application is different with the UE, e.g. IPv4 and IPv6, etc.

Editor's note:	Whether other functionalities, e.g. function to handle MMTel service related Signalling, are needed in the MMTel Enabler Server is FFS.

### 6.3.3	MMTel Enabler Client

The MMTel Enabler Client provide the client side functionality needed in MMTel service.

The MMTel Enabler Client interact with DCMTSI Client in the same UE to control the downloading behavior on DCMTSI Client based on the data channel application profiles provided by the MMTel Enabler Server.

NOTE 1:	the DCMTSI client is responsible for creating bootstrap channel and actually downloading the data channel applications.

NOTE 2	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

### 6.3.4	Application Server

The Application Server communicates with the MMTel Enabler Server over the MMTel-2 reference point to invoke the MMTel service related capabilities to communicate with the UE, e.g. establishes a MMTel session with or without IMS data channel to the UE, or establishes an application data channel to the UE. It may also communicate with Application Client to provide Application specific logic.

NOTE:	The Application Server needs to support MMTel-2 reference point, but the detailed functionalities of the Application Server is out of scope of the present document.

### 6.3.5	Controlling Application Server

The Controlling Application Server is a specific Application Server that only used by the MMTel service provider, i.,e. MNO, to manage the service configuration related information, e.g. data channel application profiles, needed in MMTel service. It communicates with the MMTel Enabler Server over the MMTel-3 reference point to invoke the MMTel service related capabilities and does not interacts with Application Client on the UE directly.

NOTE:	The functionalities of the Controlling Application Server is out of scope of the present document.

Editor's note:	 Whether the controlling AS functionalities can be provided by OAM or not is FFS and may require coordination with SA5.

### 6.3.6	Application Client

The Application Client is an entity in the application layer to implement and perform the application service logic for its own service.

NOTE:	The Application Client can be the data channel application client. The functionalities of the Application Client is out of scope of the present document.

### 6.3.1	SEAL Client

The following SEAL Clients for MMTel enablement service are supported:

-	Notification management client as specified in clause 17 of TS 23.434 [6];

### 6.3.2	SEAL server

The following SEAL servers for MMTel enablement service are supported:

-	Notification management server as specified in clause 17 of TS 23.434 [6];

* * * Next Change * * * *

## 6.4	Reference points

### 6.4.1	General

The reference points for MMTel Enabler are described in the following subclauses.

Editor's note:	 The functions of the reference points are FFS and to be updated per solution.

### 6.4.2	MMTel-1 (between the MMTel Enabler Client and the MMTel Enabler Server)

The MMTel-1 reference point supports the interactions between a MMTel Enabler Client and the corresponding MMTel Enabler Server. This reference point supports the exchange of service configuration related information needed in MMTel service.

### 6.4.3	MMTel-2 (between the MMTel Enabler Server and the Application Server)

The interactions between the Application Server and the underlying network are provided by the MMTel Enabler Server via MMTel-2 reference point to ensure the security access of the underlying network. This reference point supports the Application Server to discover MMTel service APIs, to authenticate, to get authorization and to communicate with the MMTel service.

### 6.4.4	MMTel-3 (between the MMTel Enabler Server and the Controlling Application Server)

The interactions between the MMTel Enabler Server and the Controlling Application Server are referred to MMTel-3 reference point. This reference point supports the Controlling Application Server to manage the service configuration related  information needed in MMTel service.

### 6.4.5	MMTel-4 (between the MMTel Enabler Client and the DCMTSI client)

The interaction between MMTel Enabler Client and DCMTSI Client.are referred to MMTel-4 reference point. This reference point is internal interaction in the dialler application on the UE and implementation specific.

NOTE:	The functionalities of the MMTel-4 reference point is out of scope of the present document.

### 6.4.6	App-1 (between the Application Client and the Application Server)

The App-1reference point supports the communication between  Application Client and Application Server to provide the Data Channel Application specific logic.

NOTE:	The functionalities of the App-1 reference point is out of scope of the present document.

### 6.4.1	SEAL-C

The following SEAL-C reference points for MMTel enablement service are supported:

-	NM-C reference point for notification management as specified in 3GPP TS 23.434 [6];

### 6.4.2	SEAL-S

The following SEAL-S reference points for MMTel enablement service are supported:

-	NM-S reference point for notification management as specified in 3GPP TS 23.434 [6];

### 6.4.3	SEAL-UU

The following SEAL-UU reference points for MMTel enablement service are supported:

-	NM-UU reference point for notification management as specified in 3GPP TS 23.434 [6];

* * * Next Change * * * *

##### 8.2.1.3.1	DC Application and profile configuration request

Table 8.2.1.3.1-1 describes information elements for the DC Application and profile configuration request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.1.3.1-1: Information elements in DC Application and Profile configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application are listed in Table 8.2.1.3.1-2. |

Table 8.2.1.3.1-2: Information elements in DC application list

| Information element | Status | Description |
| --- | --- | --- |
| Application Identifier | M | Identifier of the Data Channel Application assigned by the Application Provider |
| Application Name (see NOTE 1) | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon Url | O | Icon url of the Data Channel Application |
| Application Version (see NOTE 1) | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. Precall+Incall: the Data Channel Application is allowed to be used during the entire Precall and incall. |
| Autoload (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC (see NOTE 2) | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario (see NOTE 2) | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition (see NOTE 2) | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint (see NOTE 2) | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection (see NOTE 4) | O | Indicates whether this Data Channel Application will collect the user personal data (along with related consent opt-in/opt-out procedures). |
| PersonalDataCollectionInfoURL (see NOTE 4) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected, their processing their protection etc. |
| Application Package (see NOTE 1) | O | The Code package of the Data Channel Application. |
| NOTE 1: This IE shall be present when when configuring the DC application. NOTE 2: This IE shall be present when when configuring the DC application profile. NOTE 3: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be applied by the user on the UE. The relationship with PersonalDataCollection should be considered, which is out of 3GPP scope. NOTE 4: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

* * * Next Change * * * *

##### 8.2.2.3.1	DC Application and profile update request

Table 8.2.2.3.1-1 describes information elements for the DC Application and profile update request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.2.3.1-1: Information elements in DC Application and Profile update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application and profile are listed in Table 8.2.1.3.1-2. |

Table 8.2.2.3.1-2: Information elements in DC application list

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon Url | O | Icon url of the Data Channel Application |
| Application Version | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. Precall+Incall: the Data Channel Application is allowed to be used during the entire Precall and incall. |
| Autoload | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection | O | Indicates whether this Data Channel Application will collect the user personal data (along with related consent opt-in/opt-out procedures). |
| PersonalDataCollectionInfoURL | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected, their processing their protection etc. |
| Application Package (see NOTE 2) | O | The Code package of the Data Channel Application. |
| NOTE 1: At least one of these optional IEs is present in the update request. A special case. When the application is updated, the request must carry the application package and application version. NOTE 2: The Application Package also support incremental updates. |  |  |

* * * Next Change * * * *

8.2.3.3.1	DC Application delete request

Table 8.2.3.3.1-1 describes information elements for the DC Application delete request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.3.3.1-1: Information elements in DC Application delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| List of Application ID | M | List of identifiers of the DC application to be deleted. |

* * * Next Change * * * *

8.2.4.3.1	Getting DC Application profile information request

Table 8.2.4.3.1-1 describes information elements for the Getting DC Application profile information request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.4.3.1-1: Information elements in Getting DC Application Profile information request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| Application ID list | M | A list of Application ID for the request. |

* * * Next Change * * * *

### 8.3.2	DC application profiles downloading on UE

#### 8.3.2.1	General

Application profiles downloading enables a MMTel Enabler Client to download the DC application profile from MMTel Enabler Server and use it to control the downloading behavior on DCMTSI Client in the same UE. This procedure is a detailed HTTP interaction procedure in the bootstrap data channel which established as per Annex AC.7.1

#### 8.3.2.2	Procedure

Figure 8.3.2.2-1 illustrates the DC application profiles downloading procedure.

Pre-conditions:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.2.2-1: MMTel Enabler Client in the UE requests the Data Channel Application profile list

When the MMTel service provider provides a Root application which specified in GSMA NG.134 [5], containing layout, and/or home page etc. of the Data Channel Application profile list, step 1, step 2 and step 3 apply: Otherwise the step 1, step 2 and step 3 are skipped. Further, if root application is locally stored in the MMTel Enabler Client and if root application validity is not expired then step 1, step 2 and step 3 are skipped.

1.	The MMTel Enabler Client in the UE sends a Get Root application request to MMTel Enabler Server to get the Root application (e.g. layout, and/or home page etc. of the Data Channel Application profile list). The request message includes information elements as specified in Table 8.3.2.3.1-1. The application type is used by MMTel Enabler Server to suggest DC applications for download.

NOTE 1:	in a implementation, the DCSF may support the DC application profile functionality of MMTel Enabler Server function, the DCSF may also interact with the MMTel Enabler Server to support this procedure, i.e. The MMTel Enabler Client sends a request to the DCSF, which then sends the request to the MMTel Enabler Server. The response in the opposite direction is also sent to the MMTel Enabler Client through the DCSF.

Editor’s note:	if the DCSF interacts with the MMTel Enabler Server to support this procedure, whether the  interactions between DCSF and MMTel Enabler Server are needed to be specified or implementation is FFS.

2.	The MMTel Enabler Server checks whether the Root application version included in the Get Root application request is the newest version of the Root application. If the Root application version IE is not included, the Root application version is not considered as newest.

3.	The MMTel Enabler Server sends a Get Root application response to the MMTel Enabler Client in the UE. The response message includes information elements as specified in Table 8.3.2.3.2-1.

If root application is locally stored in the MMTel Enabler cslient, validity of the application will be updated to the value received in Root application validity IE.

NOTE 2:	When service provider wants the UE to delete the stored root application, the response message includes negative value in the Root application validity IE.

4.	The MMTel Enabler Client in the UE send a Get Application Profile List request to MMTel Enabler Server to get the Data Channel Application profile list available for this user. The request message includes information elements as specified in Table 8.3.2.3.3-1.

5.	The MMTel Enabler Server sends a  Get Application Profile List response to the MMTel Enabler Client in the UE. The response message includes information elements as specified in Table 8.3.2.3.4-1. The response message includes list of Data Channel Application.

The MMTel Enabler Client locally stores the downloaded applications until the validity is expired or update available for application or notification received to update the root application.

#### 8.3.2.3	Information flows

##### 8.3.2.3.1	Get Root application request

Table 8.3.2.3.1-1 describes information elements for the Get Root application request from the MMTel Enabler Client to the MMTel Enabler Server.

Table 8.3.2.3.1-1: Information elements in Get Root application request

| Information element | Status | Description |
| --- | --- | --- |
| UE Information (see NOTE) | M | The UE related information required to identify the Root application (e.g. device type, device vendor, etc) |
| Root application version | O | The newest version of the Root application stored in the MMTel Enabler Client in the UE. This IE presents if Root application is locally stored in the MMTel Enabler Client. |
| application type | O | Type of application (e.g. emergency, conference, XR, etc.) |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different Root application based on this IE. |  |  |

##### 8.3.2.3.2	Get Root application response

Table 8.3.2.3.2-1 describes information elements for the Get Root application response from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.2-1: Information elements for Get Root application response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of Get Root application request. |
| Update needed | M | Indicates whether the Root application is needed to be updated by the MMTel Enabler Client in the UE |
| Root application version | O | The newest version of the Root application. This IE presents only if the Root application is needed to be downloaded or updated by the MMTel Enabler Client in the UE. |
| Root application validity | O | The validity of the Root application. This IE presents only if the Root application is needed to be downloaded or updated by the MMTel Enabler Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the MMTel service provider, e.g. layout, and/or home page etc of the Data Channel Application list. This IE presents if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the newest version on the MMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |

##### 8.3.2.3.3	Get Application Profile List request

Table 8.3.2.3.3-1 describes information elements for the Get Application Profile List request from the MMTel Enabler Client to the MMTel Enabler Server.

Table 8.3.2.1-3: Information elements in Get Application Profile List request

| Information element | Status | Description |
| --- | --- | --- |
| MMTel Enabler Client version (see NOTE) | M | The version of MMTel Enabler Client |
| application type | O | type of application (e.g. emergency, conference, XR, etc.) |
| Begin index | O | The starting index of the first Data Channel Application required in the Data Channel Application list. This IE presents if the requested Data Channel Application is not the first one in the Data Channel Application list, e.g. when the Data Channel Application list is needed to be shown in multiple pages, this IE is needed to be included when request the pages other than the first page. The default value of this IE is 0 if this IE is absent. |
| Number of application profile required | O | The total number of Data Channel Application profiles required in this request |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different Data Channel Application profile list based on this IE. |  |  |

##### 8.3.2.3.4	Get Application Profile List response

Table 8.3.2.3.4-1 describes information elements for the Get Application Profile List response from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.4-1: Information elements in Get Application Profile List response

| Information element | Status | Description |
| --- | --- | --- |
| Data Channel Application profile list | M | A list of Data Channel Application profiles available for this user. Each element in this list contains a DC application profile of this Data Channel Application. The detailed information elements of DC application profile are listed in Table 8.3.2.1-5. |

Table 8.3.2.3.4-2: Information elements in DC application profile

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon | O | Icon of the Data Channel Application |
| Application Version | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. |
| Autoload (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection (see NOTE 2) | O | Indicates whether this Data Channel Application will collect the user personal data. |
| PersonalDataCollectionInfoURL (see NOTE 2) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected (along with related consent opt-in/opt-out procedures), their processing their protection etc. |
| NOTE 1: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be applied by the user on the UE. NOTE 2: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

* * * Next Change * * * *

### 8.3.3	DC application profiles updating on UE

#### 8.3.3.1	General

There are two modes in this procedure:

the MMTel Enabler Server notifies the MMTel Enabler Client in the UE to send application request messages to the MMTel Enabler Server to request the profiles of data channel applications;

the MMTel Enabler Server sends the profiles to the UE directly after the UE subscribes to the profile notification.

The two detailed procedures of DC application profiles updating on UE and the related information flow are specified in clause 8.3.3.2 and 8.3.3.3.

NOTE :	The updating procedures go over the top of DCSF. The detailed procedures to update data channel applications to the UE are out of scope of 3GPP TS 23.228 [3].

#### 8.3.3.2	Procedure

##### 8.3.3.2.1	The MMTel Enabler Client request DC application profiles based on notification

The MMTel Enabler Server can notify the MMTel Enabler Client in the UE to send application request messages to the MMTel Enabler Server to request the DC application profiles, the detailed procedure is illustrated in Figure 8.3.3.2.1-1.

NOTE:	The MMTel Enabler Server can instruct the MMTel Enabler Client to request root application and data channel application profiles as required. e.g. root application version update, offline of a data channel application, etc.

Pre-condition:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.3.2.1-1: MMTel Enabler Client request DC application profiles based on notification

1.	The notification channel has been established between notification management client and notification management Server as specified clause 17.3.3 of TS 23.434 [6].

2.	The MMTel Enabler Client subscribes to the MMTel Enabler Server for receiving notifications.

3.	The MMTel Enabler Server sends a Root application and profile update notification to notify the MMTel Enabler Client in the UE to get the Root application or data channel profile. The Root application and profile update notification includes information elements as specified in Table 8.3.3.3.1-1.

The step 4 to step 8 are same  as steps 1 through 5 in clause 8.3.2.2.

##### 8.3.3.2.2	The MMTel Enabler Server notifies the DC application profile to the MMTel Enabler Client

The MMTel Enabler Server notifies the DC application profiles to the MMTel Enabler Client is illustrated in Figure 8.3.3.2.2-1.

Pre-condition:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.3.2.2-1: MMTel Enabler Server notifies the DC application profile list to UE

1.	The notification channel has been established between notification management client and notification management server as specified clause 17.3.3 of TS 23.434 [6].

2.	The MMTel Enabler Client subscribes to the MMTel Enabler Server for receiving notifications.

3.	The MMTel Enabler Server checks the provision of Root application and DC application profile list is needed, e.g. the UE has IMS DC capability, an application version or application profile update is available, etc.

NOTE:	The detail additional checklists in this step are out of scope of the present document.

4.	The MMTel Enabler Server sends a Root application notification message to the MMTel Enabler Client in the UE. The notification message includes information elements as specified in Table 8.3.3.3.2-1.

The MMTel Enabler Server sends a data channel DC application profile List notification to the MMTel Enabler Client in the UE. The notification message includes information elements are as same as the elements in Table 8.3.2.3.4-1.

#### 8.3.3.3	Information flows

##### 8.3.3.3.1	Root application and profile update notification

Table 8.3.3.3.1-1 describes information elements for the Root application and profile update notification from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.1-1: Information elements in Root application and profile update notification

| Information element | Status | Description |
| --- | --- | --- |
| Urgent | O | Indicates whether the operation is urgent to UE, i.e. whether the Root application and profile need to be updated immediately. E.g. the update of Root application and profile is urgent if the available DC Application for the user is changed |

##### 8.3.3.3.2	Root application notification

Table 8.3.3.3.2-1 describes information elements for the Root application notification from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.3.3.2-1: Information elements for Root application notification

| Information element | Status | Description |
| --- | --- | --- |
| Update needed | M | Indicates whether the Root application is needed to be updated by the MMTel Enabler Client in the UE |
| Root application version | O | The newest version of the Root application. This IE presents only if the Root application is needed to be updated by the MMTel Enabler Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the MMTel service provider, e.g. layout, and/or home page etc. of the Data Channel Application list. This IE presents if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the newest version on the MMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |

* * * End of Changes * * * *