3GPP TSG-SA WG6 Meeting #65	S6-250090

Athens, Greece 17th – 21st February 2025	(revision of 250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0725 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the specification for Satellite edge computing considering EAS onboard |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The descriptions in clause 11.1.x of TS23.558 need to be added based on the study of FS_5GSAT_Ph3_App. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds descriptions of Satellite edge computing considering EAS onboard in FS_5GSAT_Ph3_App. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot reflect changes based on study of FS_5GSAT_Ph3_App for Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.x, 11.1.x.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

11.1.x	Satellite edge computing considering EAS onboard

This procedure is applicable for Satellite edge computing considering EAS onboard.

Figure 11.1.x-1: EAS onboard satellite deployment

In this deployment option, ECS and EES are deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The UPF to access EAS is deployed on satellite. RAN (e.g. gB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF. The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground, which is not depicted in the figure for simplicity. UPF can be deployed on ground to access the ECS and EES, the EEC (in UE) can reach the ECS by EDGE-4 (on ground or via space), and EEC (in UE) can reach the EES by EDGE-1 (on ground).

Considering that EAS(s) deployed in multiple MEO or LEO satellites, so that the EAS(s) are mobile as the satellites are moving relative to the Earth's surface. In this case, if the ephemeris information of the EAS is not considered for the discovery, frequent interruption may be experienced when there is a mismatch between the movement of the UE and the movement of the EAS on board satellite.

As for the EAS on MEO and LEO satellites however the EES is on the ground, in the EAS discovery procedure, the EES can determine a list of EAS based on the UE (predicted/expected) location, Prediction expiration time, Satellite ID and EAS ephemeris information and the EES provide list of EAS information in the EAS discovery response message. Upon receiving a list of EAS information, the UE can determine the EAS based on the UE (predicted/expected) location, Prediction expiration time, EAS Satellite ID and EAS ephemeris information, and UE may sequentially connect to corresponding EAS based on ephemeris information of the EASs. When an EAS is about to leave the UE’s location, the UE may connect to a next EAS in advance to obtain an edge computing service, and does not need to perform EAS discovery again. In doing so, the service interruption caused by periodic EAS discovery can be avoided.

Besides, the EEC can send Satellite ID in service provisioning request message to the ECS for EES information.

In addition, when a serving satellite is providing EAS and due to discontinuous coverage the same satellite comes back for serving, then the EEC connects to same EAS (i.e. to which the EEC had previously connected to), without needing to discover EAS endpoints again.

Editor's note: The procedures and the use of the satellite ID and ephemeris information in the procedures is FFS

* * * 2nd Change * * * *

11.1.x.1	EES profile

The EES profile is same as clause 8.2.6 with the following additions.

Table 11.1.x.1-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.2.6-1 |  |  |
| Satellite ID | O | The Satellite ID corresponding to the EAS. |
| ephemeris information | O | The ephemeris information of the satellite corresponding to the EAS |