3GPP TSG-SA WG6 Meeting #65	S6-250092

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0011 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | remove the EN about the split operation node registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-2-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.482 has been approved. Split operation node registration remains a EN “Whether an AIMLE client can register as a processing node is FFS”. besides VAL server, UE also has capability of computing. AIMLE client can invoke these resource to archive split operation. So it is proposed to remove the EN in clause 8.14.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN about the AIMLE client register as a processing node in clause 8.14.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN unsolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2, 8.14.3.6-8.14.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.14.2.4	Split operation node registration

8.14.2.4.1	General

Clause 8.14.2.4.2 illustrates the split operation node registration procedure.

Clause 8.14.2.4.3 illustrates the split operation node registration update procedure.

Clause 8.14.2.4.4 illustrates the split operation node de-registration procedure.

8.14.2.4.2	Split operation node registration

Figure 8.14.2.4.2-1 illustrates the procedure for VAL server/AIMLE client to register with the AIMLE server to indicate split operation capabilities.

Pre-conditions:

1.	The VAL server/AIMLE client has received information (e.g. URI, IP address) related to the AIMLE Server;

2.	The VAL server/AIMLE client has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.14.2.4.2-1: Split operation node registration

1.	The VAL server/AIMLE client sends a split operation node register request to the AIMLE server to indicate the split operation capabilities of the VAL server/AIMLE client. The request includes information defined in Table 8.14.3.6-1.

2.	Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request, the AIMLE server stores the registration information.

3.	The AIMLE server sends a split operation node register response to the requestor. If the AIMLE server has registered the requestor, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation node registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure.

8.14.2.4.3	Split operation node registration update

Figure 8.14.2.5-1 illustrates the procedure for VAL server/AIMLE client to update a split operation node registration with the AIMLE server.

Pre-conditions:

1.	The VAL server/AIMLE client has registered with the AIMLE Server.

Figure 8.14.2.4.3-1: Split operation node registration update

1.	The VAL server/AIMLE client sends a split operation node registration update request to the AIMLE server to update an existing split operation registration. The request includes information defined in Table 8.14.3.8-1.

2.	Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the registration information.

3.	The AIMLE server sends a split operation node registration update response to the requestor. If the AIMLE server has successfully updated the registration, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure.

8.14.2.4.4	Split operation node de-registration

Figure 8.14.2.4.4-1 illustrates the procedure for VAL server/AIMLE client to de-register for split operation with the AIMLE server.

Pre-conditions:

1.	The VAL server/AIMLE client has registered with the AIMLE Server.

Figure 8.14.2.4.4-1: Split operation node de-registration

1.	The VAL server/AIMLE client sends a split operation node de-register request to the AIMLE server. The request includes information defined in Table 8.14.3.10-1.

2.	Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server terminates the registration for split operation.

3.	The AIMLE server sends a split operation node de-register response to the requestor. If the AIMLE server has successfully terminated the registration for split operation node, the response includes an indication of success. If the registration for split operation was not terminated, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

8.14.3.6	Split operation node register request

Table 8.14.3.6-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation node register request.

Table 8.14.3.6-1: Split operation node register request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID, AIMLE client ID). |
| Security credentials | M | The security credentials of the requestor. |
| Node information | M | Information about the VAL server/AIMLE client node (e.g., identifier, endpoints, etc.). |
| Split operation capabilities | M | Split operation capabilities of the VAL server/AIMLE client. |
| > Model information | M | Information about ML model capabilities of the VAL server/AIMLE client for split operation (e.g., identifiers, versions, etc.). |
| > Usage information | O | Information about usage capabilities of the VAL server/AIMLE client for split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Expiration time | O | The proposed expiration time of the registration. |

8.14.3.7	Split operation node register response

Table 8.14.3.7-1 shows the response sent by an AIMLE server after processing of split operation node register request.

Table 8.14.3.7-1: Split operation node register response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > Registration identifier | M | The identifier of the registration. |
| > Expiration time | O | The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |

8.14.3.8	Split operation node registration update request

Table 8.14.3.8-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation node registration update request.

Table 8.14.3.8-1: Split operation node registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the registration. |
| Security credentials | M | The security credentials of the requestor. |
| Node information | O | Information about the VAL server/AIMLE client node (e.g., identifier, endpoints, etc.). |
| Split operation capabilities | O | Split operation capabilities of the VAL server. |
| > Model information | O | Information about ML model capabilities of the VAL server/AIMLE client for split operation (e.g., identifiers, versions, etc.). |
| > Usage information | O | Information about usage capabilities of the VAL server/AIMLE client for split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Expiration time | O | The proposed expiration time of the registration. |

8.14.3.9	Split operation node registration update response

Table 8.14.3.9-1 shows the response sent by an AIMLE server after processing of split operation node registration update request.

Table 8.14.3.9-1: Split operation node registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > Expiration time | O | The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |

8.14.3.10	Split operation node de-register request

Table 8.14.3.10-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation de-register request.

Table 8.14.3.10-1: Split operation node de-register request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the registration. |
| Security credentials | M | The security credentials of the requestor. |

8.14.3.11	Split operation node de-register response

Table 8.14.3.11-1 shows the response sent by an AIMLE server after processing of split operation de-register request.

Table 8.14.3.11-1: Split operation de-register response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |

* * * END of Change * * * *