3GPP TSG-SA WG6 Meeting #64	S6-250100

Orlando, USA, 18th – 122nd November 2024	(revision of S6-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0128 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add update and delete service operation of SEALDD client policy configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-2-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SEALDD client policy configuration is been approved, but the SEALDD client policy configuration update and delete need to be added. Secondly, the Expiration time is added in the SEALDD client policy configuration request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add update and delete service operation of SEALDD client policy configuration. Add Expiration time in the SEALDD client policy configuration request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEALDD client policy configuration can not be updated and deleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.10.4.1 New: 9.10.2.b， 9.10.2.c， 9.10.3.m， 9.10.3.n， 9.10.3.o， 9.10.3.p， 9.10.4.x， 9.10.4.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.10.2.b	SEALDD client policy configuration update request

Figure 9.10.2.b-1 illustrates the procedure for SEALDD client policy configuration update request from the SEALDD server to the SEALDD client.

Pre-condition:

- The SEALDD connectivity has been established between the SEALDD client and SEALDD server.

Figure 9.10.2.b-1: SEALDD client policy configuration update request

- Based on policy received from the VAL server, or SEALDD server determines that the policy needs to be updated to the SEALDD client, the SEALDD server sends the SEALDD client policy configuration update request to the SEALDD client.

2.	The SEALDD client performs authorization check to verify whether the SEALDD server can be authorized to update the SEALDD client policy.

3.	Upon successful authorization, the SEALDD client updates the SEALDD client policy and replies to the SEALDD server with the SEALDD client policy configuration update response.

9.10.2.c	SEALDD client policy configuration delete request

Figure 9.10.2.c-1 illustrates the procedure for SEALDD client policy configuration delete request from the SEALDD server to the SEALDD client.

Pre-condition:

- The SEALDD connectivity has been established between the SEALDD client and SEALDD server.

Figure 9.10.2.c-1: SEALDD client policy configuration delete request

- Based on request received from the VAL server, or SEALDD server determines that the policy needs to be deleted, the SEALDD server sends the SEALDD client policy configuration delete request to the SEALDD client.

2.	The SEALDD client performs authorization check to verify whether the SEALDD server can be authorized to delete the SEALDD client policy.

3.	Upon successful authorization, the SEALDD client deletes the SEALDD client policy and replies to the SEALDD server with the SEALDD client policy configuration delete response.

* * * Next Change * * * *

9.10.3.m	SEALDD client policy configuration update request

Table 9.10.3.m-1 describes the information flow from the SEALDD server to the SEALDD client for requesting SEALDD client policy configuration update.

Table 9.10.3.m-1: SEALDD client policy configuration update request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identifier of the SEALDD client policy. |
| Updated SEALDD client policy | O | The updated SEALDD client policy. |

9.10.3.n	SEALDD client policy configuration update response

Table 9.10.3.n-1 describes the information flow from the SEALDD server to the VAL server for response SEALDD client configuration policy update.

Table 9.10.3.n-1: SEALDD client policy configuration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure of the request. |

9.10.3.o	SEALDD client policy configuration delete request

Table 9.10.3.o-1 describes the information flow from the SEALDD server to the SEALDD client for requesting SEALDD client policy configuration delete.

Table 9.10.3.o-1: SEALDD client policy configuration delete request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identity of the SEALDD client policy. |

9.10.3.1p	SEALDD client policy configuration delete response

Table 9.10.3.p-1 describes the information flow from the SEALDD server to the VAL server for response SEALDD client policy Configuration delete.

Table 9.12.3.p-1: SEALDD client policy configuration delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure of the request. |

* * * Next Change * * * *

9.10.4	APIs

9.10.4.1	General

Table 9.10.4.1-1 illustrates the APIs exposed by SEALDD server for SEALDD policy configuration.

Table 9.10.4.1-1: List of SEALDD server APIs for policy configuration

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_PolicyConfiguration | Request | Request/Response | VAL server |
|  | Update |  |  |
|  | Delete |  |  |
| Sdd_ClientPolicyConfiguration | Request | Request/Response | SEALDD server |
|  | Update |  |  |
|  | Delete |  |  |

9.10.4.2	Sdd_PolicyConfiguration operation

API operation name: Sdd_PolicyConfiguration_Request

Description: The consumer requests for one time for SEALDD policy configuration.

Inputs: See clause 9.10.3.1.

Outputs: See clause 9.10.3.2.

See clause 9.10.2.1 for details of usage of this operation.

9.10.4.3	Sdd_PolicyConfiguration update operation

API operation name: Sdd_PolicyConfiguration_Update

Description: The consumer requests for one time for SEALDD policy configuration update.

Inputs: See clause 9.10.3.3.

Outputs: See clause 9.10.3.4.

See clause 9.10.2.2 for details of usage of this operation.

9.10.4.4	Sdd_PolicyConfiguration delete operation

API operation name: Sdd_PolicyConfiguration_Delete

Description: The consumer requests for one time for SEALDD policy configuration deletion.

Inputs: See clause 9.10.3.5.

Outputs: See clause 9.10.3.6.

See clause 9.10.2.3 for details of the usage of this operation.

9.10.4.5	Sdd_ClientPolicyConfiguration operation

API operation name: Sdd_ClientPolicyConfiguration_Delete

Description: The consumer requests for one time for SEALDD Client policy configuration.

Inputs: See clause 9.10.3.7.

Outputs: See clause 9.10.3.8.

See clause 9.10.2.4 for details of the usage of this operation.

9.10.4.x	Sdd_ClientPolicyConfiguration_Update operation

API operation name: Sdd_ClientPolicyConfiguration_Update

Description: The consumer requests for SEALDD Client policy configuration update.

Inputs: See clause 9.10.3.m.

Outputs: See clause 9.10.3.n.

See clause 9.10.2.b for details of the usage of this operation.

9.10.4.y	Sdd_ClientPolicyConfiguration_Delete operation

API operation name: Sdd_ClientPolicyConfiguration_Delete

Description: The consumer requests for SEALDD Client policy configuration delete.

Inputs: See clause 9.10.3.o.

Outputs: See clause 9.10.3.p.

See clause 9.10.2.c for details of the usage of this operation.