3GPP TSG-SA WG6 Meeting #65	S6-250104

Athens, Greece, 17th – 21st February 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0247 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dynamic AEF instantiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Base on the conclusion, the Solution#6 and solution#7 is concluded as candicate solutions of KI#7. However, the motivation of KI#7 is to improve the resource usage efficiency. (1)The AEF which providing service APIs may be dynamically instantiated in different cases, e.g., triggered by service API discovery from a particular API invoker, or due the statistics of service API discovery failure with no service API is found, or due to the service API is overload. When a AEF instance is instantiated, all its services are created accrodingly during the instantiation. It is supported by SA5 in TS 28.538 as described in LS S6-2450008/ S5-245379. Also, as indicated in TS 28.538, the EAS, EES, ECS can act as the AEF which can be dynamically instantiated by the ECSP management system defined by SA5. After that, the service API status can be changed but it will not impact the resource allocaiton/deallocation. (2)As described TS 23.222 (i.e., cl.6.3.6 and cl.6.4.8) and TS 28.538, the API management function can act as the consumer of the MnS provided by management system (e.g., ECSP management system). In order to support the above use case, the CCF indicates the API management function to explicitly request the ECSP to instantiate the AEF to make the service API being active. Alternatively, the ECSP may be triggered by some event/information reported by CCF or AEF. This CR is proposed to introduce the dynamic AEF instantiation as a standalone procedure to the TS 23.222, and update the other related procedures accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the API publish procedure and the API discover procedure. - Introduce a new dynamic AEF instantiation procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dynamic service API activation is not supported by CAPIF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 8.3.2.1, 8.7.2.2, 8.7.3, new {8.aa, 8.aa.1, 8.aa.2, 8.aa.2.1, 8.aa.2.2, 8.aa.3} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

4.2.2	Requirements

[AR-4.2.2-a] The CAPIF shall provide a mechanism to publish the service API information to be used by the API invokers to discover and subsequently invoke the service API.

[AR-4.2.2-b] The CAPIF shall provide a mechanism for the API invokers to discover the published service API information as specified in [AR-4.2.2-a] according to the API invokers' interest.

[AR-4.2.2-c] The CAPIF shall provide a mechanism to restrict the discovery of the published service API information by the API invokers, based on configured policies.

[AR-4.2.2-d] The CAPIF shall provide a mechanism to configure policies to restrict the discovery of the published service API information.

[AR-4.2.2-e] The CAPIF shall provide mechanism to support Serving Area Information related to service APIs.

[AR-4.2.2-g] The CAPIF shall provide mechanism to dynamically instantiate an AEF.

# /**************************** Next changes **********************/

8.3.2.1	Service API publish request

Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function.

Table 8.3.2.1-1: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M (see NOTE 1) | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), service API operation(s) and resource(s) (optional), and Network Slice Info (optional), security methods. |
| Shareable information | O (see NOTE 2) | Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained. |
| NOTE 1: The interface details shall be not present, if the AEF is not available. NOTE: If the shareable information is not present, the service API information is not allowed to be shared. |  |  |

The Service KPIs is defined as below:

Table 8.3.2.1-2: Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the API Invoker supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the API Invoker's service requests. |
| Availability | O | Advertised percentage of time the server is available for the API Invoker's use. |
| Available Compute | O | The maximum compute resource available for the API Invoker. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the API Invoker. |
| Available Memory | O | The maximum memory resource available for the API Invoker. |
| Available Storage | O | The maximum storage resource available for the API Invoker. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the API Invoker's use. |

# /**************************** Next changes **********************/

8.7.2.2	Service API discover response

Table 8.7.2.2-1 describes the information flow service API discover response from the CAPIF core function to the API invoker.

Table 8.7.2.2-1: Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1, NOTE 3) | List of service APIs corresponding to the request, including service API information such as service API name, API provider name (optional), service API category, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI) (NOTE 4) (optional), protocols, version, data format, Service KPIs (optional), service API operation(s) and resource(s) (optional), and Network Slice Info (optional), security methods. |
| Interface information | O (NOTE 3) | The interface details (e.g. IP address, port number, URI) of the requested service API(s) |
| CAPIF core function identity information | O (see NOTE 1, NOTE 3) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. NOTE 3: "Interface information" IE shall not be present when "Service API information" or "CAPIF core function identity information" is present. NOTE 4: The interface details (e.g. IP address, port number, URI) and service API status shall not present, if the AEF status is not available. |  |  |

8.7.3	Procedure

Figure 8.7.3-1 illustrates the procedure for discover service APIs.

The service API discovery mechanism is supported by the CAPIF core function.

Pre-conditions:

1.	The API invoker is onboarded and has received an API invoker identity.

2.	The CAPIF core function is configured with a discovery policy information (e.g. to restrict discovery to category of APIs) for API invoker(s).

3.	The CAPIF core function obtains the service API information with the corresponding API status through service API publish procedure.

4.	The CAPIF core function has obtained the AEF status from the management function (e.g. API management function), the AEF status may include instantiated, and instantiable but not yet instantiated.

Figure 8.7.3-1: Discover service APIs

1.	The API invoker sends a service API discover request to the CAPIF core function. It includes the API invoker identity, either query information or the service API identification.

For service API discovery for interested service APIs, the query information is included.

2.	Upon receiving the service API discover request, the CAPIF core function verifies the identity of the API invoker (via authentication).

If the query information is included, the CAPIF core function retrieves the stored service API(s) information from the CAPIF core function (API registry) as per the query information in the service API discover request. Further, the CAPIF core function applies the discovery policy and performs filtering of service APIs information retrieved from the CAPIF core function.

If the service API identification is included, the CAPIF core function retrieves the interface information of the requested service API(s).

3.	If the AEF is not available (i.e., the AEF status is instantiable but not be instantiated), the procedure as described in clause 8.aa is performed.

4.	The CAPIF core function sends a service API discover response to the API invoker.

If the query information is included, the list of service API information is returned.

If the service API identification is included, the interface information is returned.

# /**************************** Next changes **********************/

8.aa	Instantiate AEF on demand

8.aa.1	General

The AEF which provides the service APIs may be dynamically instantiated in different cases, e.g., triggered by service API discovery from a particular API invoker, or due the statistics of service API discovery failure with no service API is found, or due to the existing service API is overload. When a AEF is instantiated, all the service API provided by this AEF is created together.The servie API status may be also dynamically changed via interaction the management system.

8.aa.2	Information flows

8.aa.2.1	AEF instantiate request

Table 8.aa.2.1-1 describes the information flow AEF instantiate request from the CAPIF core function to the API management function.

Table 8.aa.2.1-1: AEF instantiate request

| Information element | Status | Description |
| --- | --- | --- |
| AEF information | M | The AEF to be instantiated. |
| NOTE 1: at least one of those IEs shall be present. |  |  |

8.aa.2.2	AEF instantiate response

Table 8.aa.2.2-1 describes the information flow AEF instantiate response from the API management function to the CAPIF core function.

Table 8.aa.2.2-1: AEF instantiate response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the activation/instantiation |

8.aa.3	Procedure

Figure 8.aa.3-1 illustrates the procedure for dynamic AEF instantiation.

Pre-conditions:

-	The API management function has subscribed the CAPIF event from the CAPIF core function as described in clause 8.8.

Figure 8.aa.3-1: Procedure for dynamic AEF instantiation – triggered by CAPIF

1.	The triggers at the CAPIF core function or the API management is met, e.g., the CAPIF core function receives service API discovery request but failing to retrieve service API, the API management function detects the AEF providing the service API is overload.

The API management function may receive the AEF instantiate request/Event notification from CAPIF core function (1a). The API management function interacts with the management system (e.g., ECSP management system) to instantiate the AEF with the service API and make the service API in active status (1b) based on local policy or information received in step 1a.

2.	The service API management system performs authorization check. If successful, it performs the AEF instantiation and enables the requested service API in active status.

3.	The API management function sends the AEF instantiate response or event notification acknowledge to the CAPIF core function.

NOTE: Step 3 can be sent before the AEF instantiation completion.

4.	After the AEF instantiation is complete, the API management function registers the AEF to the CAPIF core function.

5.	Also, the API publish function updates the service API information towards the CAPIF core function.

NOTE:	The detailed AEF instantiation is defined by SA5.