3GPP TSG-SA WG6 Meeting #65	S6-250105

Athens, Greece, 17th – 21st February 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.946 | CR | 0003 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to figure 6.3-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are something wrong in figure 6.3-1: (1) Currently, the OP in figure 6.3 acts as the API invoker who aggregates services from multiple PLMN operators. However, the OP now is within one of the PLMN domain which is conflict with its role as the aggregator. It is needs to be corrected. (2) there is also another deployment that the OP is hold by the operator and integrated with the CAPIF core function and API provider domain functions. In this case, the OP can directly interact with the 3GPP NFs, i.e., the SBI NR should be mapped to the 5G core NF interfaces without via the NEF. Moreover, GSMA role represents their requirements, it is not functionalities. When the OP is integrated with CAPIF, each role is supported by multiple CAPIF functionalities based on the role descriptions in GSMA PRD document. For example, the capability exposure role, it covers all kind of services/capabilities exposure. So both of the CAPIF core function and the AEF functions support this requirements and should have this role. Also, the Service resource manage role should be mapped to the OAM system provided by SA5. For the federation, both the network capabilities and resource are interacting via the EWBI, so both the CAPIF and the OAM support such role. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the figure 6.3 to move the OP out of the PLMN trust domain. Add a new figure to capture the case where the OP is owned by MNO operator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong description of GSMA Operator Platform deployment via CAPIF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

6.3	GSMA Operator Platform deployment via CAPIF

The GSMA Operator Platform defines a common platform exposing operator services / capabilities to customers / application providers in the 5G-era. The requirement that an Operator Platform’s SBI-NR can work with CAPIF when available is specified in clause 5.1.4.2.2 of GSMA PRD OPG.02 [11] and application session continuity support for handovers between 4G and 5G based on combined SCEF+NEF via CAPIF is described in clause 5.2.2.6.7 of GSMA PRD OPG.02 [11].

Figure 6.3-1 illustrates integrated fundamental GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF. In this deployment model, the GSMA Operator Platform functions are owned by the MNO and mapped to CAPIF core function and the API provider domain functions.

Figure 6.3-1: GSMA Operator Platform deployment of the SCEF and the NEF with CAPIF  – OP acting as CAPIF provider

Figure 6.3-2 illustrates integrated fundamental GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF. In this deployment model, the GSMA Operator Platform functions act as the API invoker.

NOTE:	An Operator Platform's SBI-NR can work without CAPIF. If CAPIF is not supported, the SBI-NR API will provide an alternate means of providing these functions.

Figure 6.3-2: GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF– OP acting as API invoker