3GPP TSG-SA WG6 Meeting #65	S6-250109

Athens, Greece, 17th – 21st February 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0132 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing figures to clause 9.12.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1.The S6-245713 was agreed at SA#64 with the clause 9.12.2.3 being introduced. Howeve, the figure of Figure 9.12.2.3.2-1, 9.12.2.3.3-1, 9.12.2.3.4-1 are missing. 2.The “3GPP TS 23.542 [x]” needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing figures from S6-245713 to clause 9.12.2.3. Correc the “3GPP TS 23.542 [x]” Correct some grammer, typo issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.2.3.2, 9.12.2.3.3, 9.12.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Message style in the figures are also corrected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

9.12.2.3.2	SEALDD and VAL coordination measurement based on PIN

Figure 9.12.2.3.2-1 illustrates the procedure of SEALDD and VAL coordination Tethering link measurement based on PIN.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE or PINE has already been registered in PIN server;

3.	The PEMC on the 3GPP device has already obtained the tethered device information from PIN server;

4.	There is business agreement between the VAL provider and the SEALDD provider, which requests the VAL client to respond to ICMP Ping packet.

Figure 9.12.2.3.2-1: SEALDD and VAL coordination measurement based on PIN procedure

1.	The PIN is successfully created and in use. The 3GPP UE acts as PEGC and PEMC, and the Tethered XR devices acts as a PINE. During the PIN creation, the detailed information of the tethered device has been provided to PEMC on 3GPP UE from PINAPP server as defined in clause 8.5.2, 3GPP TS 23.542 [19].

2.	The regular data transmission connection is established, with the information received in step1.

The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2.

3.	The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server as defined in clause 9.7. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time.

4.	The SEALDD server starts the transmission quality measurement as defined in clause 9.7.2.3. The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server.

5.	The SEALDD client, acting as a PINE, gets the information of the tethered device from the PEMC over PIN-1, as defined in the clause 8.5.8 of 3GPP TS 23.542 [19], to identify the tethering link.

6.	The SEALDD client on the 3GPP UE interacts with XR client on the tethered UE to do the measurement based on ICMP ping protocol.

7.	The SEALDD client sends the report to the SEALDD server using the transmission quality measurement notification.

8.	The SEALDD server reports the data transmission quality measurement results to the VAL server via the notification message.

9.12.2.3.3	SEALDD measurement based on PIN

Figure 9.12.2.3.3-1 illustrates the procedures of SEALDD measurement based on PIN.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE or PINE has already been registered in PIN server;

3.	The PEMC on the 3GPP device has already obtained the tethered device information from PIN server;

Figure 9.12.2.3.3-1: SEALDD measurement based on PIN procedure

1.	The PIN is successfully created and in use. The 3GPP UE acts as PEGC and PEMC, and the Tethered XR devices acts as a PINE. During the PIN creation, the detailed information of the tethered device has been provided to PEMC on 3GPP UE from PINAPP server as defined in clause 8.5.2, 3GPP TS 23.542 [19].

2.	The regular data transmission connection is established, with the information received in step1.

The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2.

Optionally, the SEALDD client gets the tethered device information form PEMC(e.g., the MAC address, PINE Address, Port number.) and sends the direct data transmission connection request to the tethered UE using the information form PEMC to establish a SEALDD-UUc connection between the 3GPP UE and tethered device as defined in clause 9.12.2.3.4.

3.	The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server as defined in clause 9.7. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time.

4.	The SEALDD transmission quality measurement is defined in clause 9.7.2.3. The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server.

5.	The SEALDD client gets the information of the tethered device from the PEMC over PIN-1, as defined in the clause 8.5.8 of 3GPP TS 23.542 [19], to identify the tethering link.

6.	The SEALDD client on the 3GPP UE interacts with SEALDD client on the tethered UE to do the measurement based on ICMP ping protocol, or uses monitoring packet in established tethering link SEALDD connection.

7.	The SEALDD client sends the report to the SEALDD server using the transmission quality measurement notification.

8.	The SEALDD server sends the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the SEALDD enabled data transmission quality measurement notification as defined in clause 9.7.3.3.

9.12.2.3.4	SEALDD-UUc Connection establishment between the SEALDD client on 3GPP UE and tethered device

Depicted in figure 9.12.2.3.4-1 is the procedure for establishment of SEALDD-UUc connection between the SEALDD client on 3GPP UE and tethered device.

Figure 9.12.2.3.4-1: Establishment of SEALDD-UUc Connection between the SEALDD client on 3GPP UE and tethered device

1.	SEALDD client on 3GPP UE sends a SEALDD-UUc Connection Request to SEALDD client on tethered UE. This message includes the UE identity, SEALDD-UUc Data transmission connection information(e.g., address/port allocated).

2.	SEALDD client on Tethered UE initiates the procedure for mutual authentication. The successful completion of the authentication procedure completes the establishment of the SEALDD-UUc Connection.

3.	SEALDD client on tethered UE sends a SEALDD-UUc Connection Response to SEALDD client on 3GPP UE.