3GPP TSG-SA WG6 Meeting #65	S6-250110

Athens, Greece, 17th – 21st February 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0133 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Message name style alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, SA6 adopts the message name style with all lower case word. But not all the messages in TS 23.433 follows this rule. It is proposed to align this message style. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the message style. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment of message style impact the quality of the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2.1, 9.6.3.5, 9.6.3.6, 9.6.3.7 9.11.2.1, 9.11.2.2, 9.11.2.3, 9.11.3.1, 9.11.3.2, 9.11.3.3, 9.11.3.4, 9.11.3.5, 9.11.3.6, 9.11.3.7, 9.11.4.1, 9.11.4.2, 9.11.4.3, 9.11.4.4, 9.11.4.5, 9.12.2.3.4, 9.12.3.5, 9.12.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

9.6.2.1	SEALDD context transfer

Figure 9.6.2.1-1 describes SEALDD context transfer procedure in pull (step 1a and 2a) or push (step 1b and 2b) operation.

Pre-conditions:

1.	For pull operation, the old SEALDD server endpoint is available in the new SEALDD server

2.	For push operation, the new SEALDD server endpoint is available in the old SEALDD server.

Figure 9.6.2.1-1: SEALDD context transfer

The transferred SEALDD context includes the service subscription information created upon VAL server's interaction for requesting SEALDD transmission service (e.g. step 1 and 2 in Figure 9.2.2.1-1), and also the SEALDD client and SEALDD server communication tunnel management information (e.g. UE IP address) which is created upon SEALDD client's interaction for requesting data transmission (e.g. step 5 and 6 in Figure 9.2.2.1-1). If new SEALDD server supports transportation layer service continuity, additional transport layer context (e.g. TCP/TLS/QUIC) is transferred from old SEALDD server to new SEALDD server. If the new SEALDD Server is not provisioned with the VAL server configured SEALDD policy, then it can pull from the Old SEALDD server via the pull operation. The push operation does not support the transfer of the VAL server configured SEALDD policy between Old and New SEALDD server.

The new SEALDD server, after receiving the SEALDD context from the old SEALDD server, allocates IP address and port for SEALDD-UU user plane communication. The new SEALDD server also sends back to the old SEALDD server with the allocated endpoint information If operation is push. Then the old SEALDD server can request 5GC to perform IP replacement procedure, as defined in clause 6.3.3 of 3GPP 23.548 [8]. The request includes the traffic descriptor of old SEALDD server (i.e., SEALDD-UU address/port), the traffic descriptor of new SEALDD server (i.e., SEALDD-UU address/port) and/or the target DNAI.

Optionally, after receiving the SEALDD context with the SEALDD-UU endpoint from the old SEALDD server, the new SEALDD server can request 5GC to perform IP replacement procedure.

NOTE:	The TCP/TLS/QUIC context transfer is required to support the SEALDD server relocation with IP replacement procedure, as described in clause 9.6.2.2.

To improve the seamless SEALDD relocation, the old SEALDD server may stop the downlink data transmission before pushing the SEALDD context to the new SEALDD server, the context data may include content breakpoint information, as defined in clause 9.6.3.1. After the SEALDD client connects to the new SEALDD server, the new SEALDD server transmits the downlink traffic to the SEALDD client based on content breakpoint information.

# /**************************** Next changes **********************/

9.6.3.5	SEALDD inform ACR event request

Table 9.6.3.5-1 describes the information flow from the VAL server to the SEALDD server to inform the ACR event, for ACR coordination purpose.

Table 9.6.3.5-1: SEALDD inform ACR event request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |

9.6.3.6	SEALDD inform ACR event response

Table 9.6.3.6-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD Inform ACR event request.

Table 9.6.3.6-1: SEALDD inform ACR event response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |

9.6.3.7	SEALDD inform ACR event notification

Table 9.6.3.7-1 describes the information flow from the SEALDD server to the VAL server for notifying ACR event related information.

Table 9.6.3.7-1: SEALDD inform ACR event notifiation

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD flow transfer result | M | Success or failure. |

# /**************************** Next changes **********************/

9.11.2.1	SEALDD background data transfer subscription

Figure 9.11.2.1-1 illustrates the procedure for SEALDD background data transfer subscription.

Figure 9.11.2.1-1: SEALDD background data transfer

1.	The VAL server sends a SEALDD background data transfer subscription request to the SEALDD server. The request includes the VAL service ID, a list of VAL UE IDs or VAL group ID, the data volume per UE and may also include the desired time window, the desired area information (e.g., when a UE enters a geographical area), Policy Selection Guidance.

2.	The SEALDD server authorizes the request from the VAL server.

3.	The SEALDD server starts the BDT configuration negotiation with the SEAL NRM server as described in clause 14.3.13 of TS 23.434 [4].

NOTE:	If the NRM server is an external control plane functionality for the SEALDD server, then the SEALDD server uses step 3, otherwise SEALDD server can use procedure defined in 3GPP TS 23.502 [6] clause 4.16.7.2.

4.	The SEALDD server sends the SEALDD background data transfer subscription response, containing the result of the operation which includes the result and may include the BDT subscription identifier.

5.	If the SEALDD server determines that the negotiated BDT policy applies to ongoing sessions, the SEALDD server performs Network resource adaptation negotiation with the SEAL NRM server as described in clause 14.3.3.3 of TS 23.434 [4]. Policies are set for the BDT data transfer using the previously negotiated BDT Reference ID. This step may be performed simultaneously with step 7.

6.	If the SEALDD server determines that the negotiated BDT policy applies to future sessions, the SEALDD server performs the Set BDT policy for future sessions as specified in clause 4.15.6.8 of 3GPP TS 23.502 [6].

7.	The VAL server uses clause 9.5.2 procedure to send the BDT data to the SEALDD server for DL centric direction. During the negotiated transmission time window, the SEALDD server sends the BDT data to SEALDD client(s) and monitors the quality, e.g., latency, jitter, bitrate, packet loss rate, of the transmission. The SEALDD client forwards the BDT data received to the VAL client.

Equivalently, for UL centric direction, during the negotiated transmission window, the SEALDD client(s) sends the BDT data to the SEALDD server. The SEALDD server forwards the BDT data received to the VAL server.

8.	The SEALDD server may notify the VAL server about:

i.	the delivery result, by sending the SEALDD background data transfer notification including the BDT subscription identifier and optionally the list of VAL UEs which received/sent the BDT data, the executed time window and quality of the BDT data transmission based on the measurements in step 5; or.

ii.	the renogiated granted time window or the removal of BDT policy, if there is any change in the network impacting the current BDT policy and the SEAL NRM server performs BDT policy re-selection as described in clause 14.3.13.3 of 3GPP TS 23.434 [4].

9.11.2.2	SEALDD background data transfer subscription update

Figure 9.11.2.2-1 illustrates the procedure for SEALDD background data transfer subscription update from VAL server to the SEALDD server.

Pre-condition:

-	The SEALDD background data transfer subscription procedure has been performed.

Figure 9.11.2.2-1: SEALDD background data transfer subscription update

1.	The VAL server can trigger a BDT update by sending a SEALDD background data transfer subscription update request. The request contains BDT subscription identifier and may contain e.g. updated desired time window, desired area information, Policy Selection Guidance, updated data volume per UE.

2.	The SEALDD server starts a new BDT configuration negotiation with the SEAL NRM server as described in clause 14.3.13.5 of TS 23.434 [4].

NOTE:	If the NRM server is an external control plane functionality for the SEALDD server, then the SEALDD server uses step 3 otherwise SEALDD server can directly use procedure defined in 3GPP TS 23.502 [6] clause 4.16.7.2

3.	The SEALDD server sends the SEALDD background data transfer subscription update response to the VAL server, containing result and the BDT subscription identifier, and, if updated, the granted time window.

4.	If the SEALDD server determines that the negotiated BDT policy applies to ongoing sessions, the SEALDD may perform Network resource adaptation negotiation with the SEAL NRM server as described in clause 14.3.3.3 of TS 23.434 [4] for the updated BDT subscription. This step may occur simultaneously with the delivery of the BDT data.

5.	If the SEALDD server determines that the negotiated BDT policy applies to future sessions, the SEALDD server may perform the Set BDT policy for future sessions as specified in clause 4.15.6.8 of 3GPP TS 23.502 [6].

9.11.2.3	SEALDD background data transfer unsubscribe

Figure 9.11.2.3-1 illustrates the procedure for SEALDD background data transfer unsubscription from the VAL server to the SEALDD server.

Pre-condition:

-	The SEALDD background data transfer subscription procedure has been performed.

Figure 9.11.2.3-1: SEALDD background data transfer unsubscribe

1.	The VAL server may trigger a BDT deletion by sending a SEALDD background data transfer unsubscribe request to the SEALDD server. The request contains the BDT subscription identifier.

2.	The SEALDD server requests the deletion of the BDT configuration to the SEAL NRM server as described in clause 14.3.13.6 of TS 23.434 [4] and deletes any stored application data related to the BDT subscription identifier.

3.	The SEALDD server sends the SEALDD background data transfer unsubscribe response to the VAL server, containing result of the operation.

9.11.3	Information flows

9.11.3.1	SEALDD background data transfer subscription request

Table 9.11.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD background data transfer request.

Table 9.11.3.1-1: SEALDD background data transfer subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | Identity of the VAL service for which the background data transfer is requested. |
| List of VAL UE IDs | O (See NOTE) | List of VAL UE IDs for which the transfer policy applies. |
| VAL group ID | O (See NOTE) | VAL group ID for which the transfer policy applies. |
| Desired time window | O | Desired time window for the background data transfer. |
| Data volume per UE | M | Expected data volume for the background data transfer. |
| Desired area information | O | Desired geographical area for the background data transfer. |
| Policy Selection Guidance | O | List that includes guidance in selecting from multiple transfer policies provided by underlying network. Possible values include: ”lowest cost”, “highest throughput given maximum cost of X”, etc. If not included, local and ASP-provided policies decide one among multiple transfer policies. |
| NOTE: One of these IEs shall be present in the message. |  |  |

9.11.3.2	SEALDD background data transfer subscription response

Table 9.11.3.2-1 describes the information flow from the SEALDD server to the VAL server.

Table 9.11.3.2-1: SEALDD background data transfer subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure. |
| subscription identifier | O | Indicates the background data transfer subscription identifier. Applicable for successful result. |
| Granted time window | O | Granted time window for the background data transfer. Applicable for successful result. |

9.11.3.3	SEALDD background data transfer notification

Table 9.11.3.2-1 describes the information flow from the SEALDD server to the VAL server.

Table 9.11.3.3-1: SEALDD Background data transfer notification

| Information element | Status | Description |
| --- | --- | --- |
| subscription identifier | M | Indicates the background data transfer subscription identifier |
| List of VAL UE IDs | O | List of VAL UE IDs or VAL group ID. |
| Executed time window | O | The actual time window the BDT data was transferred. |
| Granted time window | O | If BDT renegotiation takes place, and the previously provided granted time window changed, the current granted time window for the background data transfer. |
| BDT transmission quality | O | Indicates the transmission quality (e.g., QoE) when transferring the BDT data between SEALDD client and SEALDD server. |

9.11.3.4	SEALDD background data transfer update subscription request

Table 9.11.3.4-1 describes the information flow from the VAL server to the SEALDD server for sending the SEALDD background data transfer update subscription request.

Table 9.11.3.4-1: SEALDD background data transfer update subscription request

| Information element | Status | Description |
| --- | --- | --- |
| subscription identifier | M | Indicates the background data transfer subscription identifier. |
| Desired time window | O (See NOTE) | Desired time window for the background data transfer. |
| Desired area information | O (See NOTE) | Desired geographical area for the background data transfer. |
| Policy Selection Guidance | O (See NOTE) | List that includes guidance in selecting from multiple transfer policies provided by underlying network. Possible values include: ”lowest cost”, “highest throughput given maximum cost of X”, etc. |
| Data volume per UE | O (See NOTE) | Updated expected data volume for the background data transfer. |
| NOTE: At least one of these IEs shall be present in the message. |  |  |

9.11.3.5	SEALDD background data transfer update subscription response

Table 9.11.3.5-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD background data transfer update subscription request.

Table 9.11.3.5-1: SEALDD background data transfer update subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure. |
| Subscription identifier | O | Indicates the background data transfer subscription identifier. Applicable for successful result. |
| Granted time window | O | Granted time window for the background data transfer. Applicable for successful result. |

9.11.3.6	SEALDD background data transfer unsubscribe request

Table 9.11.3.6-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD background data transfer unsubscribe request.

Table 9.11.3.6-1: SEALDD background data transfer unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| subscription identifier | M | Indicates the background data transfer subscription identifier. |

9.11.3.7	SEALDD background data transfer unsubscribe response

Table 9.11.3.7-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD Background data transfer unsubscribe response.

Table 9.11.3.7-1: SEALDD background data transfer unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the BDT unsubscribe operation. |

9.11.4	APIs

9.11.4.1	General

Table 9.11.4.1-1 illustrates the APIs exposed by SEALDD server for background data transfer.

Table 9.11.4.1-1: List of SEALDD server APIs for background data transfer

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_BDT | Subscribe | Subscribe/Notify | VAL server |
|  | Notify | Subscribe/Notify | VAL server |
|  | Update | Subscribe/Notify | VAL server |
|  | Unsubscribe | Subscribe/Notify | VAL server |

9.11.4.2	Sdd_BDT_Subscribe operation

API operation name: Sdd_BDT_Subscribe

Description: The consumer requests for a background data transfer service.

Inputs: See clause 9.11.3.1.

Outputs: See clause 9.11.3.2.

See clause 9.11.2.1 for details of usage of this operation.

9.11.4.3	Sdd_BDT_Notify operation

API operation name: Sdd_BDT_Notify

Description: The consumer is notified with background data transfer result.

Inputs: See clause 9.11.3.3.

Outputs: None.

See clause 9.11.2.1 for details of usage of this operation.

9.11.4.4	Sdd_BDT_Update operation

API operation name: Sdd_BDT_Update

Description: The consumer requests to update the SEALDD background data transfer service.

Inputs: See clause 9.11.3.4.

Outputs: See clause 9.11.3.5.

See clause 9.11.2.2 for details of usage of this operation.

9.11.4.5	Sdd_BDT_Unsubscribe operation

API operation name: Sdd_BDT_Unsubscribe

Description: The consumer requests to unsubscribe the SEALDD background data transfer service.

Inputs: See clause 9.11.3.6.

Outputs: See clause 9.11.3.7.

See clause 9.11.2.3 for details of usage of this operation.

# /**************************** Next changes **********************/

9.12.2.3.4	SEALDD-UUc Connection establishment between the SEALDD client on 3GPP UE and tethered device

Depicted in figure 9.12.2.3.4-1 is the procedure for establishment of SEALDD-UUc connection between the SEALDD client on 3GPP UE and tethered device.

Figure 9.12.2.3.4-1: Establishment of SEALDD-UUc connection between the SEALDD client on 3GPP UE and tethered device

1.	SEALDD client on 3GPP UE sends a SEALDD-UUc connection request to SEALDD client on tethered UE. This message includes the UE identity, SEALDD-UUc data transmission connection information(e.g., address/port allocated).

2.	SEALDD client on Tethered UE initiates the procedure for mutual authentication. The successful completion of the authentication procedure completes the establishment of the SEALDD-UUc connection.

3.	SEALDD client on tethered UE sends a SEALDD-UUc connection response to SEALDD client on 3GPP UE.

# /**************************** Next changes **********************/

9.12.3.5	SEALDD-UUc connection request

Table 9.12.3.5-1 describes the information flow from the SEALDD client on 3GPP UE to SEALDD client on tethered UE for SEALDD-UUc connection request.

Table 9.12.3.5-1: SEALDD-UUc connection request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor’s ID or PIN ID | M | Identity of the requestor SEALDD client on 3GPP UE, or the PIN ID of the requestor. |
| SEALDD-UUc data transmission connection information | M | SEALDD-UUc data transmission connection information(e.g., address/port allocated) |

9.12.3.6	SEALDD-UUc connection response

Table 9.12.3.6-1 describes the information flow from the SEALDD client on 3GPP UE to SEALDD client on tethered UE for SEALDD-UUc ronnection response.

Table 9.12.3.6-1: SEALDD-UUc connection Response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-UUc data transmission connection information | O | SEALDD-UUc data transmission connection information(e.g., address/port allocated) |