3GPP TSG-SA WG6 Meeting #65	S6-250114

Athens, Greece, 17th – 21st February 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0084 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve ENs in clause 10.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The EN “Editor's Note: The corresponding service profiles in the MC system for LMR user homed to IWF are FFS.” can be directly removed as the LMR user homed to IWF will act as a MC service user, and will be configured with the MC service profile as the other MC service users homed to the MC system. - The following editor’s note “Editor's Note: How the MC service system authorizes the actions of users homed in the IWF is FFS.” in clause 10.14.1 can be directly removed as each procedures in clause 10.14.3 has a similar and detailed editor’s note “Editor's note: How the MC service server checks the authorization of the IWF to perform functional alias related actions is FFS.”. - The “Editor's note: How the MC service server checks the authorization of the IWF to perform functional alias related actions is FFS.” can be directly removed as the IWF message e.g., IWF functional alias activation request, contains the MC service ID which represents the identity of the requesting MC service user can be authorized by the MC service service. i.e., such MC service ID has the corresponding user profile stored at the MC service server which allows the MC service ID to check the authorization. - The ” Editor's note: How the MC service server checks whether a user homed in the IWF is an "authorized" user is FFS.” can be directly removed as each MC service ID used by the IWF represents a MC service user and has to performed the identity authentication and service authorization before initiating the service procedures like affiliation, call request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the editor’s notes in clause 10.14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid editor’s note in the TS will cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.1, 10.14.3.2, 10.14.3.3, 10.14.3.4, 10.14.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

10.14.1	General

LMR users homed in the IWF shall have the ability to enable, apply, or disable a functional alias in the MC system for the use in communication with MC service users.

The functional alias feature is not a requirement in 3GPP TS 22.179 [3] and is therefore an optional feature for systems that support 3GPP TS 22.179 [3].

# /****************************2nd changes **********************/

10.14.3.2	User homed in the IWF retrieves active functional alias(es) for a certain MC service user

An user homed in the IWF can request the active functional alias(es) for a certain MC service user.

Figure 10.14.3.2-1 below illustrates the active functional alias list query for a certain MC service user.

Figure 10.14.3.2-1: IWF active functional alias list query

1.	The user homed in the IWF requests a list of active functional aliases for a certain MC service ID from the MC service server by sending an IWF functional alias information query request encompassing the MC service ID or the functional alias of the queried user.

2.	The MC service server checks whether the querying user homed in the IWF is authorized to perform the query. If authorized, then the MC service server retrieves the requested functional alias information based on the corresponding MC service ID or the MC service IDs based on the functional alias.

3.	The MC service server sends an IWF functional alias information query response including the active functional alias or MC service ID information to the user homed in the IWF.

10.14.3.3	User homed in the IWF activates functional alias(es) within an MC system

The procedure for the user homed in the IWF activates functional alias(es) within an MC system is illustrated in figure 10.14.3.3-1.

Pre-conditions:

1.	The IWF has already been provisioned (statically or dynamically) with the functional alias(es) information that the user homed in the IWF is allowed to activate.

2.	MC service server has retrieved the user subscription and functional alias policy e.g. which user(s) are authorized to activate to what functional alias, priority, and other configuration data.

Figure 10.14.3.3-1: IWF functional alias activation procedure within an MC system

1.	The user homed in the IWF requests the MC service server to activate a functional alias or a set of functional aliases.

2.	The MC service server checks if there are any conflicts with active functional alias(es).

3.	If the user homed in the IWF is authorised to activate the requested functional alias(es) then the MC service server stores the functional alias(es) status of the requested functional alias(es).

If a certain functional alias(es) can be simultaneously active for multiple users and the upper limit of number of simultaneous MC service users is not reached, the MC service shall activate the functional alias(es) for the user homed in the IWF and inform all other user(s) with sharing the same functional alias(es) (step 5). If the limit of number of simultaneous users is reached or the functional alias is not allowed to be shared, the request is rejected, and the IWF is notified (step 4).

Editor's note:	How the MC service server checks whether a user homed in the IWF is an "authorized" user is FFS.

If the functional alias(es) is (are) already used by another user(s), an authorized user homed in the IWF gets an offer to take over the functional alias from the user currently using the functional alias(es).

4.	MC service server sends an IWF functional alias(es) activation response to the user homed in the IWF.

5.	The MC service server informs all other MC service user(s) and/or IWF sharing the same functional alias(es).

10.14.3.4	User homed in the IWF de-activates functional alias(es) within an MC system

The procedure for the user homed in the IWF de-activates functional alias(es) within an MC system is illustrated in figure 10.14.3.4-1.

When a user homed in the IWF does not want to use a functional alias(es) anymore, then the user homed in the IWF can de-activate functional alias(es).

Pre-conditions:

1.	MC service server has already subscribed to the functional alias(es) information from the functional alias management server and has stored the data of the functional alias(es) a user homed in the IWF has activated.

Figure 10.14.3.4-1: IWF functional alias de-activation procedure within an MC system

1.	The user homed in the IWF requests the MC service server to de-activate a functional alias or a set of functional aliases.

2.	Based on the MC service user subscription and stored functional alias policy, the MC service server checks if the user homed in the IWF is authorized to de-activate from the requested functional alias(es) and if the user homed in the IWF has activated to the requested functional alias(es).

3.	If the user homed in the IWF is authorized to de-activate from the requested functional alias(es) then the MC service server updates the functional alias activation status of the user homed in the IWF.

4.	MC service server provides to the user homed in the IWF the functional alias de-activation response.

5.	The MC service server informs all other MC service user(s) and/or users homed in the IWF sharing the same functional alias(es).

10.14.3.5	User homed in the IWF takes over functional alias(es) within an MC system

The procedure for the user homed to IWF takes over functional alias(es) within an MC system is illustrated in figure 10.14.3.5-1.

During functional alias(es) activation, if the functional alias(es) is (are) already used by another MC service user(s), an authorized user homed in the IWF can get an offer to take over the functional alias(es) from the MC service user currently using the functional alias(es).

Pre-conditions:

1.	MC service client 1 has performed the functional alias(es) activation procedure.

2.	As result of the functional alias(es) activation procedure, the user homed in the IWF is aware which functional alias(es) are already used but can be taken over.

3.	The user homed in the IWF decides to take over a functional alias.

Figure 10.14.3.5-1: IWF functional alias taking over procedure within an MC system

1.	The user homed in the IWF requests the MC service server to take over a functional alias by sending an IWF functional alias take over request.

2.	The MC service server checks if there are any conflicts taking over the functional alias.

3.	If the user homed in the IWF is authorised to take over the requested functional alias then the MC service server sends a functional alias revoke notification to inform MC service client 1 that the functional alias has been revoked and is not any longer active for the user of MC service client 1.

4.	The MC service server stores the functional alias status of the requested functional alias.

5.	MC service server sends an IWF functional alias take over response to the user homed in the IWF.

6.	The MC service server informs all other MC service user(s) sharing the same functional alias, of the take over by sending a functional alias status notification.

7.	The MC service server informs all user(s) homed in the IWF sharing the same functional alias of the take over by sending an IWF functional alias status notification.