3GPP TSG-SA WG6 Meeting #65	S6-250117

Athens, Greece, 17th – 21st February 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0144 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Some editorial changes to 23.289 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some editorial issues, grammar issues and title style issue need to fix. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix the editorial, grammar and title style issues in the TS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS quality is bad. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5, 7.3.2.7, 7.3.3.2.3, 7.3.3.7.2.1, 7.3.3.7.2.2, 7.3.3.9.1, 7.3.3.10.3, 7.3.3.11.2.1, 7.3.3.11.2.2, 7.3.3.11.2.2.1, 7.3.3.11.2.2.2, 7.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The title style of clause 7.3.3.11.2.1, 7.3.3.11.2.2, 7.3.3.11.2.2.1, 7.3.3.11.2.2.2 are corrected with only change marks. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

6.2.5	UE 1

UE 1 is an MC service UE in on-network mode supporting data connectivity and application(s) related to one or more MC services over the 5GS, or a UE that acts as 5G ProSe UE-to-network relay, or both of the above. It is composed of the corresponding MC service functional entities described in clause 5.

# /**************************** 2nd changes **********************/

7.3.2.7	UnMapGroupFromSessionStream

Table 7.3.2.7-1 describes the information flow to disconnect an MC group communication call from an MBS session. It is sent from the MC service server to the MC service client, where the MC service can either be MCPTT, MCVideo or MCData services.

Table 7.3.2.7-1: UnMapGroupFromSessionStream

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT group ID | M | This element identifies the MC service group related to a group call to be dissociated over the MBS session. The MC service group ID is either MCPTT group ID, MCVideo group ID, or MCData group ID. |
| Media stream identifier | M | This element identifies the media stream of the SDP, which is no longer used for the group call within the MBS session. In case separate media streams are used for audio and video stream in MCVideo services, a separate identifier is optinally used to identify the audio stream, which is no longer used. |
| MBS session ID | O | Indicating the MBS session ID, if the information is sent over another MBS session or unicast path |

# /**************************** Next changes **********************/

7.3.3.2.3	Procedure for updating MBS resources with dynamic PCC rule

The procedure shown in figure 7.3.3.2.3-1 presents an MBS session update procedure triggered by the MC service server to the 5GC, either directly or via NEF/MBSF. Based on the required updates to be done, the MC service server needs to interact with the MB-SMF to update the MBS service area and multicast activity status, with the PCF to update the required QoS requirements, or sequentially both to update all the above, as indicated in 3GPP TS 23.247 [15].

Pre-conditions:

-	The MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	The MC service server has obtained the required information related to the MB-SMF, either locally configured or during initial session configuration.

-	The MBS session is created with certain service requirements and optionally with a certain broadcast/multicast service area. The MBS session is announced to be associated with the MC service group for group communication purposes.

Figure 7.3.3.2.3-1: MBS session update with dynamic PCC.

1.	An MBS session is established as described in 3GPP TS 23.247 [15] (either a multicast or a broadcast MBS session), and associated with a certain active MC group for group communication purposes. In the case of a multicast MBS session, the MC service clients have already joined the MBS session.

2.	The MC service server, based on the update requirements (i.e., MBS service area and/or the multicast MBS session activity status as well as the service requirements), perform the MBS session update with PCC procedure towards the 5GS as described in 3GPP TS 23.247 [15] and/or the MBS session activate/deactivate procedure as described in clause 7.3.3.4.

In case the of untrusted MC service server, if the MBS service area is partially accepted by the 5GC, the reduced MBS service area information is grouped and provided by the NEF/MBSF in the response. Hence, the MC service server should send a new MBS session creation request (as described in clause 7.3.3.1.3) per grouped MC service area. The MC service server may utilize a unicast session if any MBS service area is not supported by any MB-SMF.

NOTE 1:	The updated service area information is required for local MBS and for broadcast MBS services.

3.	The MC service server may initiate an MBS session announcement towards the MC service clients associated with the ongoing MBS session in order to announce the updated information if required, e.g., the updated service area or SDP information.

NOTE 2:	The updated service area information is required for local MBS and for broadcast MBS services.

4.	The MC service server sends an MapGroupToSessionStream over the MBS session providing the required information to receive the media related to the established MC service group communication.

5.	The MC service clients process the received information over the MapGroupToSessionStream in order to receive the associated MC media over the specific MBS session stream.

6.	MC service client 1 sends media to the MC service server over unicast to be distributed for the established group communication.

7.	The MC service server distributes the MC media to the MC services clients 2 to n over the indicated streams.

# /**************************** Next changes **********************/

7.3.3.7.2.1	MBS Session coordination independent of media

The procedure in this sub clause applies to both multicast and broadcast MBS session. The principle and pre-condition are similar with eMBMS bearer coordination as defined in 3GPP TS 23.280 [3].

This procedure is used when two or more MC service servers are serving users in the same area and are configured to share 5G MBS sessions for that specific area. The MC service servers may be of the same kind or different kind. The MC service servers are not participating in the same group call, which means that each MC service server transmit media independently of each other.

Pre-conditions:

-	All MC service servers are configured with the contact information of those MC service servers that are configured to take the MBS session control role.

Figure 7.3.3.7.2.1-1: Multiple server MBS procedure independent of media.

1.	The MC service server 1 evaluates whether MBS transmission is desired for each service area in which MC service group members are located, based upon the locations, affiliation status and other factors.

2.	The MC service server 1 determines whether another MC service server has already established an MBS session with coverage for the MBS service area where MBS transmissions are desired. To do this, the MC service server 1 consults a pre-configured list of MC service servers and sends them a discover MBS session request. This request may be sent to several MC service servers.

NOTE:	MC service servers of the same type can be configured to discover MBS sessions from a single server. The single server then becomes a centralized entity for MBS session control for the MC service. Similarly, all MC service servers of all types can be configured to discover MBS sessions from a single server. The single server then becomes a centralized MBS session controller for all MC services.

3.	The MC service server 2 (MBS session control role) responds with a discover MBS session response indicating whether there is an MBS session available in the specific MBS service area with the requested bandwidth. The discover MBS session response message includes the MBS session ID of the MBS session that is shared between the MC service servers. If the MBS session of interest has insufficient bandwidth, the polling MC service server 1 may resort to unicast, or may allocate another MBS session for the congested area. If a duplicate MBS session is allocated for the same area, the MBS session should not be shared with other servers and may be torn down as soon as the congestion on the original MBS session clears up, in order to conserve resources.

For any MBS service areas not covered by another MC service server, the MC service server 1 prepares to distribute media to those MBS service areas by setting up an MBS session. The MBS session set up by the MC service server 1 may then become available for other MC service servers (controlling role) for other MC service groups.

4.	4a.The MC service server 1 performs the MBS session announcement towards MC client 1, as well as MBS notification handling, according to the relevant procedures specified in this specification. In case of multicast MBS sessions, the MC service UE subsequently initiates a UE session join towards 5GC, and may send a UE session join notification to MC service server 1 indicating it has successfully joined the multicast MBS session under consideration.

4.b If the MC service server 2 is authorized to receive MBS related location information from the users utilizing the services from MC service server 1, the MC service server 2 may optionally do the MBS session announcement and handling of the notifications on behalf of MC service server 1. The notifications shall in this case be sent to both MC service server 1 and MC service server 2. In case of multicast MBS sessions, the MC service UE subsequently initiates a UE session join towards 5GC, and may send a UE session join notification to MC service server 2.

5.	The MC service server 1 sends a media distribution request to the MC service server 2 (MBS session control role). The media distribution request is sent to reserve the specified capacity in the MBS session.

6.	MC service server 2 (MBS session control role) sends a media distribution response to the MC service server 1 indicating whether the request can be supported and supplies details about the MBS session.

7.	The MC service server 1 establishes a group communication session via the MBS session, informing MBS session connected MC service clients 1 and 2 that a group communication session is about to start on the MBS session. This step is equivalent to MapGroupToSessionStream.

8.	MC service client 2 sends media on the uplink to the MC service server 1.

9.	The MC service server 1 forwards the media to MC service server 2 (MBS session control role).

10.	The MC service server 2 (MBS session control role) distributes the media to MC service client 1 with MBS capabilities over the MBS session.

11.	The MC service server 1 sends a media distribution release request, informing the MC service server 2 (MBS session control role) to request the MC service server 2 (MBS session control role) to release the capacity that was reserved in step 5.

12.	The MC service server 2 (MBS session control role) responds to the request by sending a media distribution release response.

7.3.3.7.2.2	MBS session coordination within one group call

The procedure in this subclause applies to both multicast MBS session and broadcast MBS session.

The principle is similar to eMBMS bearer coordination within one group call as the following:

-	It may be used when two MC service servers are serving users in the same area and are configured to share MBS sessions for that specific area. The MC service servers are of the same kind, and the MC service servers may participate in the same group call, and by that have a need to deliver the same content.

Pre-conditions:

-	All MC service servers are configured with the contact information of those MC service servers that are configured to take the MBS session control role.

Figure 7.3.3.7.2.2-1: Multiple server MBS procedure within one group call

1.	The MC service server 1 evaluates whether MBS based transmission is desired for each service area in which MC service group members are located, based upon the locations, affiliation status and other factors.

2.	The MC service server 1 determines whether another MC service server has already established an MBS session with coverage for the MBS service area where MBS based transmission is desired. To do this, the MC service server 1 consults a pre-configured list of MC service servers and sends them a discover MBS session request. This request may be sent to several MC service servers.

NOTE 1:	MC service servers of the same type can be configured to discover MBS sessions from a single server. The single server then becomes a centralized entity for MBS session control for the MC service. Similarly, all MC service servers of all types can be configured to discover MBS sessions from a single server. The single server then becomes a centralized MBS session controller for all MC services.

3.	The MC service server 2 (MBS session control role) responds with a discover MBS session response indicating whether there is an MBS session available in the specific MBS service area with the requested bandwidth. The discover MBS session response message includes the ID of the MBS session that is shared between the MC service servers. If the MBS session of interest has insufficient bandwidth, the polling MC service server 1 may resort to unicast, or may allocate another MBS session for the congested area. If a duplicate MBS session is allocated for the same area, the MBS session should not be shared with other servers and may be torn down as soon as the congestion on the original MBS session clears up, in order to conserve resources.

For any MBS service areas not covered by another MC service server, the MC service server 1 prepares to distribute media to those MBS service areas by setting up an MBS session. The MBS session created by the MC service server 1 may then become available for other MC service servers (controlling role) for other MC service groups.

4a.	The MC service server 1 performs the MBS session announcement as well as the MBS listening reporting according to the relevant procedures specified in this specification. In case of multicast MBS sessions, the MC service UE(s) subsequently initiate a UE session join towards 5GC, and may send a UE session join notification to MC service server 1 indicating it has successfully joined the multicast MBS session under consideration.

4b.	If the MC service server 2 is authorized to receive MBS related location information from the users utilizing the services from MC service server 1, the MC service server 2 may optionally do the MBS session announcement and handling the listening reports on behalf of MC service server 1. Listening reports shall in this case be sent to both MC service server 1 and MC service server 2. In case of multicast MBS sessions, the MC service UE(s) subsequently initiate a UE session join towards 5GC, and may send a UE session join notification to MC service server 2.

NOTE 2:	Steps 1-4 are also performed by MC service server 3, but is not shown in the procedure to make it easier to read.

5.	The MC service client 2 initiates a group call that is subject for MBS transmission. In this scenario there are more than one MC service server (i.e., MC service server 1 and MC service server 3) that serve MC service clients that are affiliated to the group, and by that should receive the media in the group call.

6a.	The MC service server 1 sends a media distribution request to the MC service server 2 (MBS session control role). The media distribution request includes the MC group identifier. This indicates that the media distribution request is used for this specific group call.

6b.	The MC service server 3 sends a media distribution request to the MC service server 2 (MBS session control role). The media distribution request includes the MC group identifier. This indicates that the media distribution request is used for this specific group call.

7a.	The MC service server 2 (MBS session control role) sends a media distribution response to the MC service server 1 indicating whether the request can be supported and supplies details about the MBS session. This also includes details on which media stream should be used for transmitting the media on the MBS session. This information is used in the MapGroupToSessionStream message sent by the MC service server when setting up the group call.

7b.	The MC service server 2 (MBS session control role) sends a media distribution response to the MC service server 3 indicating that the group call is already transmitted on the MBS session by another MC service server. Based on the information, the MC service server 3 could decide to not transmit media if media is already transmitted.

8a.	The media is sent from the MC service client 2 to MC service server 1, which is the participating server for the MC service group of the group call.

8b.	The media is forwarded to all MC service servers that are serving users that take part in the group call.

NOTE 3:	The figure above does not visualize the participating server for the MC service group and controlling server for the MC service group. The media is sent to all participating servers for the MC service group which are the servers that decide on unicast or MBS transmission.

9.	The MC service server 1 forwards the media to MC service server 2 (MBS session control role).

10.	The MC service server 2 (MBS session control role) distributes the media to MC service client 1 with MBS capabilities via MBS session.

11.	The MC service server 1 sends a media distribution release request, to request the MC service server 2 (MBS session control role) to release the capacity that was reserved in step 5. The media distribution release request shall only be sent when the group call is terminated.

12.	The MC service server 2 (MBS session control role) responds to the request by sending a media distribution release response.

# /**************************** Next changes **********************/

7.3.3.9.1	General

When working in transport only mode, the MC service server guides the MC service clients throughout the MC media transmission for switching between the LTE and 5G systems. For this purpose, the location management client sends location related information to the location management server, similar to the one defined in 3GPP TS 23.280 [3], which is triggered due to its location change – in this case due to Radio Access Technology (RAT) change, to inform the server hence the latter provides guidance related to how to receive the MC services after the location update.

The procedures cover both the deployment scenarios with/without MBSF/MBSTF. The procedures specify four inter-system switching related scenarios as follows:

1.	Inter-system switching from 5G MBS session to LTE eMBMS bearer, as described in clause 7.3.3.9.2

2.	Inter-system switching from 5G MBS session to LTE unicast bearer, as described in clause 7.3.3.9.3

3.	Inter-system switching from LTE eMBMS to 5G MBS session, as described in clause 7.3.3.9.4

4.	Inter-system switching from LTE eMBMS to 5G unicast PDU session, as described in clause 7.3.3.9.5

In all the inter-system switching related scenarios described in clause 7.3.3.9.2, 7.3.3.9.3, 7.3.3.9.4 and 7.3.3.9.5, the functional entity that resides in 5GS may be NEF, or MBSF, or MB-SMF for session creation and together with PCF or PCC related interaction.

NOTE:	There will be a service interrupt when the MC service server performs path switch between 5G and LTE bearers or sessions.

# /**************************** Next changes **********************/

7.3.3.10.3	Enhanced MCPTT group call setup procedure with 5MBS

Enhanced MCPTT group call setup procedure with eMBMS bearer is specified in clause 10.10.6 of 3GPP TS 23.379 [6]. When using mission critical services over 5G MBS, the application paging message transported over 5G MBS session should also be supported.

The related procedures and information flow are as specified in 3GPP TS 23.379 [6], with only difference of using of 5G MBS session for the transmission of application paging message.

# /**************************** Next changes **********************/

7.3.3.11.2	Call connect and disconnect over MBS session procedures

7.3.3.11.2.1	General

MBS session can be used for MCVideo group calls. One MBS session may not be permanently associated to one specific group or group call.

NOTE:	It is implementation-specific that one MBS session can be re-assigned to different groups, or is associated to only one group.

The procedure in clause 7.3.3.11.2.2 requires that the group session is setup before the media transmission start. This eliminates the need for the receiving clients to continuously use a unicast session. Prior to this the MBS session is activated and announced to the MCVideo clients.

7.3.3.11.2.2	Procedure

7.3.3.11.2.2.1	Group call connect over MBS session

Figure 7.3.3.11.2.2.1-1 presents the procedure for a group communication, in specific, video call connects over either broadcast or multicast MBS sessions.

Pre-conditions:

-	The MCVideo clients 1 to n are attached to the 5GS, registered and affiliated to the same MCVideo group X.

-	The MCVideo server has directly performed (or via NEF/MBSF) an MB-SMF discovery and selection, unless the corresponding information is locally configured.

-	The MCVideo server has decided to use an MBS session for the MCVideo services associated to the MCVideo group X.

Figure 7.3.3.11.2.2.1-1: Group call connect on MBS session

1.	An MBS session is configured with the required QoS requirements, announced and established.

NOTE:	In case of broadcast MBS sessions, the session is established as part of session configuration procedures as described in 3GPP TS 23.247 [15]. In case of multicast MBS sessions, the session is established as specified in step 2.

2.	In the case of multicast MBS sessions, the MCVideo UEs initiate a UE session join towards the 5GC based on the session information provided to the MCVideo UE during the session announcement step. This step is essential in order to receive the corresponding MC media. The multicast MBS session is hence established once the request of the first initiated UE session join is accepted as indicated in 3GPP TS 23.247 [15].

3.	MCVideo client 1 initiates a MCVideo group communication, namely a video group call, by sending an initial transmit media request over a unicast PDU session towards the MCVideo server.

4.	The MCVideo server sends a MapGroupToSessionStream including the necessary stream information for the MCVideo clients 2 to n to receive the MC media related to the group call which is taken place within the associated MBS session.

5.	The MCVideo clients 2 to n process the MapGroupToSessionStream information and may send a MapGroupToSessionStream Ack back to the MCVideo server if required.

6.	The MCVideo server grants MCVideo client 1 the right to transmit media over the associated MBS session and sends a transmit media grated message to client 1 over a unicast PDU session.

7.	A media transmission notification message is sent from the MCVideo server to the MC clients 2 to n over the associated MBS session. It indicates the MCVideo ID of the transmitting client, i.e., MCVideo ID of service client 1 and the associated MC group ID.

8.	MCVideo clients 2 to n process the notification message and get informed about the necessary information of the video group call.

9.	The MCVideo clients 2 to n may send a receive media request to the MCVideo server, to indicate the reception of media over the associated MBS session. As a result, the MCVideo server sends a corresponding response message.

10.	MCVideo client 1 sends the MC media over a unicast PDU session towards the MCVideo server.

11.	The MCVideo server sends the MC media over the indicated stream within the associated MBS session to the MCVideo clients 2 to n.

7.3.3.11.2.2.2	Group call disconnect from MBS session

Figure 7.3.3.11.2.2.2-1 presents the procedure for a group call disconnect over broadcast and multicast MBS sessions.

Figure 7.3.3.11.2.2.2-1: Group call disconnect from MBS session

1.	The MC group call is taking place over the associated MBS session. MCVideo client 1 is sending the MC media over a unicast PDU session to the MCVideo server.

2.	The MCVideo delivers the MC media over the associated broadcast or multicast MBS session to MCVideo clients 2 to n.

3.	After the MC media transmission is over, i.e., no further media to be sent over the group communication, the MCVideo server sends an UnMapGroupFromSessionStream to MCVideo clients 2 to n to indicate the de-association of the group call from the corresponding MBS session stream.

4.	The MCVideo server may release the MBS session as described in clause 7.3.3.3.2.

# /**************************** Next changes **********************/

7.4.3	Network notifications of EPS interworking related events

In the case of MC services provided over a 5GS supporting EPS interworking, the MC service server may not be able to identify whether an MC service UE is registered on 5GS or EPS. Therefore, the MC service server can subscribe/unsubscribe to receive notifications of specific events from the network. These notifications can be related to PDU sessions/PDN connections to which MC service sessions are bounded or related to a specific MC service UE or group of MC service UEs.

Such notifications can be provided from the PCF (via N5 or Rx) for specific events, e.g. access network information notification and change of access type. If SCEF+NEF is deployed, notifications can also be provided from SCEF+NEF for specific events, e.g. core network (CN) type change. These event procedures are described in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], and 3GPP TS 23.503 [9].

The procedure in figure 7.4.3-1 describes a high level subscription/notification procedure between the MC service server and the network.

Pre-conditions:

-	MC service UE is a 5GS/EPS capable UE.

-	MC service UE is registered to a 5GS supporting EPS interworking for the support of MC services.

Figure 7.4.3-1: Network notifications of EPS interworking related events

1.	MC service UE is registered to the MC service server over the 5GS. MC service UE has established a PDU session(s) associated to MC service sessions.

2.	MC service server subscribes to receive notifications from the network associated to MC service UE related to EPS interworking events. For instance, the MC service server may subscribe to notifications of events from the PCF associated to the MC service UE, e.g. access network information notification and change of access type, as specified in 3GPP TS 23.503 [9].

If SCEF+NEF is deployed, the MC service server may also subscribe to notifications of events from SCEF+NEF associated to the MC service UE, e.g. CN type change, as specified in 3GPP TS 23.501 [7] and 3GPP TS 23.502 [10].

3.	The 5GS supporting EPS interworking decides that the MC service UE should be handed over to EPS, e.g. due to radio conditions. 5GS handles associated PDU sessions of the MC service UE to establish corresponding PDN connections in EPS, i.e. required default and dedicated EPS bearers associated to MC service sessions.

NOTE 1:	The interworking procedures with EPS are described in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], and 3GPP TS 23.503 [9].

4.	MC service UE is registered on EPS. MC services continue being provided to the MC service UE over EPS via the established PDN connections (default and dedicated bearers associated to corresponding MC service sessions) in step 2.

5.	The network provides notifications to the MC service server of subscribed events associated to the MC service UE.

NOTE 2:	Step 5 can occur before or in parallel to step 4.

Figure 7.3.3.11.2.2.1-1: Group call connect on MBS session

1.	An MBS session is configured with the required QoS requirements, announced and established.

NOTE:	In case of broadcast MBS sessions, the session is established as part of session configuration procedures as described in 3GPP TS 23.247 [15]. In case of multicast MBS sessions, the session is established as specified in step 2.

2.	In the case of multicast MBS sessions, the MCVideo UEs initiate a UE session join towards the 5GC based on the session information provided to the MCVideo UE during the session announcement step. This step is essential in order to receive the corresponding MC media. The multicast MBS session is hence established once the request of the first initiated UE session join is accepted as indicated in 3GPP TS 23.247 [15].

3.	MCVideo client 1 initiates a MCVideo group communication, namely a video group call, by sending an initial transmit media request over a unicast PDU session towards the MCVideo server.

4.	The MCVideo server sends a MapGroupToSessionStream including the necessary stream information for the MCVideo clients 2 to n to receive the MC media related to the group call which is taken place within the associated MBS session.

5.	The MCVideo clients 2 to n process the MapGroupToSessionStream information and may send a MapGroupToSessionStream Ack back to the MCVideo server if required.

6.	The MCVideo server grants MCVideo client 1 the right to transmit media over the associated MBS session and sends a transmit media grated message to client 1 over a unicast PDU session.

7.	A media transmission notification message is sent from the MCVideo server to the MC clients 2 to n over the associated MBS session. It indicates the MCVideo ID of the transmitting client, i.e., MCVideo ID of service client 1 and the associated MC group ID.

8.	MCVideo clients 2 to n process the notification message and get informed about the necessary information of the video group call.

9.	The MCVideo clients 2 to n may send a receive media request to the MCVideo server, to indicate the reception of media over the associated MBS session. As a result, the MCVideo server sends a corresponding response message.

10.	MCVideo client 1 sends the MC media over a unicast PDU session towards the MCVideo server.

11.	The MCVideo server sends the MC media over the indicated stream within the associated MBS session to the MCVideo clients 2 to n.