3GPP TSG-SA WG6 Meeting #65	S6-250118

Athens, Greece, 17th – 21st February 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0240 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial changes to TS23.281 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some grammar, typo issues needs to correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the grammar, typo issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bad TS quality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.2.2.2, 6.2.2.3.6. 7.4.2.5.2, 7.4.2.5.3, 7.5.2.3, 7.5.3.4.2, 7.5.3.4.2, 7.7.1.3.2A, 7.7.1.3.2B, 7.7.1.3.4, 7.7.2.2.1, 7.7.2.2.2, 7.7.2.2.3, 7.7.2.2.4, 7.7.2.2.5, 7.7.2.2.6, 7.7.2.2.7, 7.19.2.20, 7.19.2.20a, 7.19.3.1.6, 7.19.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

MCVideo client: An instance of an MC service client that provides the client application function for the MCVideo service.

MCVideo group: An MC service group confgured for MCVideo service.

MCVideo group affiliation: An MC service group affiliation for MCVideo.

MCVideo group de-affiliation: An MC service group de-affiliation for MCVideo.

MCVideo group home system: The mission critical system where the MCVideo group is defined.

MCVideo group host server: The MCVideo server within a mission critical system which provides centralised support for MCVideo services of an MCVideo group defined in a MCVideo group home system.

MCVideo ID: An instance of an MC service ID within the MCVideo service.

MCVideo server: An instance of an MC service server that provides the server application function for the MCVideo service.

MCVideo service: A video communication service supporting applications for mission critical organizations and mission critical applications for other businesses and organizations (e.g., utilities, railways) with strong security, high availability, reliability and priority handling.

MCVideo system: The collection of applications, services, and enabling capabilities required to provide Mission Critical video for a Mission Critical Organization.

MCVideo UE: An MC service UE that can be used to participate in MCVideo services.

MCVideo user: An MC service user who is authorized for MCVideo services via an MCVideo UE.

Transmission control: Video transmitting control mechanism in an MCVideo service that determines which participants have the authority to transmit video, and determines the onward downlink video transmission during a video call.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.280 [2] apply:

Mission Critical

Mission Critical Applications

Mission Critical Service

Mission Critical Organization

Mission Critical System

For the purposes of the present document, the following terms given in 3GPP TS 22.179 [15] apply

Group-broadcast group

User-broadcast group

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.281 [3] apply:

Real Time

Real Time Video

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.280 [6] apply:

Ad hoc Group Communication

MC service client

MC service group

MC service group affiliation

MC service group de-affiliation

MC service group home system

MC service group host

MC service ID

MC service server

Pre-arranged group

# /****************************Next changes **********************/

6.2.2.2	Common services core

The description of the common services core entities is contained in common functional architecture for MC services over LTE in 3GPP TS 23.280 [6].

# /****************************Next changes **********************/

6.2.2.3.6	Transmission control server

This functional entity provides support for centralised transmission control for on-network and distributed transmission control for off-network operation. It may schedule transmission requests according to uplink criteria from different transmission control participants, send a notification to all transmission control participants to allow them to receive the video according to downlink criteria if the transmission request is granted, and provide queuing in cases of contention. Transmission control applies to all MCVideo communications including group call and private call. For on-network operation, this functional entity is located with the MCVideo server, however, the transmission control server may use different IP address. For off-network operation, this functional entity is located in the UE.

# /****************************Next changes **********************/

7.4.2.5.2	Remotely initiated video push – call setup

The procedure describes the case where an authorized MCVideo user is initiating a remotely initiated video push call from source MCVideo user to a destination MCVideo user. Only the source user is allowed to transmit video to the destination user.

Procedures in figure 7.4.2.5.2-1 are the basic signalling control plane procedures for the MCVideo client remotely initiating establishment of MCVideo private call between the source and destination MCVideo users.

Figure 7.4.2.5.2-1: Remotely initiated video push – call setup

1.	MCVideo user on MCVideo client 3 initiates remote video push from MCVideo client 1 to MCVideo client 2, by sending a remote video push request to the MCVideo server for establishing a one-to-one video push call between MCVideo client 1 and MCVideo client 2. The remote video push call request contains MCVideo ID of source (MCVideo client 1) and destination (MCVideo client 2) users

2.	MCVideo server checks whether the MCVideo user at MCVideo client 3 is authorized to remotely initiate video push call between the source user and the destination user. MCVideo server also checks if there is any on-going call between the source user and the destination user.

3.	If MCVideo client 3 is authorized and there is no on-ongoing private call between source user and destination user, the MCVideo server sends the remote video push request message to MCVideo client 1 including the information of MCVideo ID of source and destination users.

4.	The MCVideo client 1 accepts the remote video push request and establishes a one-to-one video push call between MCVideo client 1 and MCVideo client 2 as described in the subclause 7.4.2.3.2. The video is transmitted from MCVideo client 1 to MCVideo Client 2.

5.	The MCVideo client 1 sends a remote video push response message to MCVideo server indicating success or failure of call establishment between MCVideo client 1 and MCVideo client 2.

6.	Upon receiving the remote video push response from MCVideo client 1, the MCVideo server informs the MCVideo client 3 about success or failure of the call establishment by sending a remote video push response message.

NOTE:	Steps 5 and 6 can occur before step 4.

7.4.2.5.3	Remotely initiated video push – call release by authorized user

The procedure describes the case where an authorized MCVideo user is releasing a remotely initiated video push call.

Procedures in figure 7.4.2.5.3-1 are the basic signalling control plane procedures for the MCVideo client releasing the remotely initiated video push call between the source and destination users.

Figure 7.4.2.5.3-1: Remotely initiated video push – call release by authorized user

1.	MCVideo user on MCVideo client 3 initiates release of remotely initiated video push call between MCVideo client 1 and MCVideo client 2, by sending a remote video push release request to the MCVideo server for releasing a one-to-one video push call between MCVideo client 1 and MCVideo client 2. The remote video push release request contains MCVideo ID of source user (MCVideo client 1) and destination user (MCVideo client 2).

2.	MCVideo server checks whether the MCVideo user at MCVideo client 3 is authorized to release the remotely initiated video push call between MCVideo client 1 and MCVideo client 2, and whether there is any on-going one-to-one video push call between MCVideo client 1 and MCVideo client 2.

3.	MCVideo server sends the remote video push release request to MCVideo client 1 for releasing the on-going one-to-one video push call between MCVideo client 1 and MCVideo client 2.

4.	The MCVideo client 1 accepts the remote video push release request and performs the release procedure for the one-to-one video push call between MCVideo client 1 and MCVideo client 2 as described in the subclause 7.4.2.3.3.

5.	The MCVideo client 1 sends a remote video push release response to the MCVideo server indicating the release of the one-to-one video push call between MCVideo client 1 and MCVideo client 2.

6.	Upon receiving the remote video push release response from MCVideo client 1, the MCVideo server informs the MCVideo client 3 about the one-to-one video push call release by sending a remote video push release response message.

NOTE:	Steps 5 and 6 can occur before step 4.

# /****************************Next changes **********************/

7.5.2.3	Update MCVideo capabilities information at the MCVideo server

The procedure for updating the MCVideo capabilities information at the MCVideo server is described in figure 7.5.2.3-1.

Figure 7.5.2.3-1: Update MCVideo capabilities information at the MCVideo server

1.	The MCVideo client sends an update MCVideo capabilities info request message to MCVideo server to update the capabilities information available at the MCVideo client

2.	The MCVideo server stores the updated MCVideo capabilities information received from the MCVideo client.

3.	The MCVideo server provides an update MCVideo capabilities info response indicating success or failure.

# /****************************Next changes **********************/

7.5.3.4.2	Request clients with particular capabilities

Figure 7.5.3.4.2-1 describes procedures for a mechanism to request other MCVideo clients having particular characteristics to share their capabilities.

MCVideo client sends a capability request message to other MCVideo clients with search criteria. The search criteria may include MCVideo user ID or MCVideo client ID or a set of capabilities or a particular category of capabilities, or a mix of such criterions, etc.

Upon receiving the capability request message, the MCVideo client that fulfils the search criteria responds to the received capability request message with a capability announcement message.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCVideo client 1.

2.	MCVideo client 1 to MCVideo client N are members of the same MCVideo group.

Figure 7.5.3.4.2-1: Request clients with particular capabilities

1.	MCVideo client 1 sends a capability request message with search criteria to request MCVideo clients of the MCVideo group, with particular characteristics, to share their capabilities.

2.	Upon receiving a capability request message with search criteria, all MCVideo clients which fulfil the search criteria respond with a Capability announcement message, which contains its capability information with other relevant information.

NOTE 1:	If the capability request message does not contain a search criterion, all the MCVideo clients that receive the Capability request message respond with a capability announcement message.

3.	MCVideo clients, upon receiving a capability announcement message from another MCVideo client, cache its presence and capability information along with other relevant information.

NOTE 2:	Capability announcement may include specific category tags (e.g. certain video camera), video capabilities of the client, location, camera angles, camera orientation, etc.

7.5.3.4.3	Request capabilities from a particular client

Figure 7.5.3.4.3-1 describes procedures for a mechanism to request a particular MCVideo client in proximity, for its capabilities.

MCVideo client sends a capability request message to the other MCVideo client.

Upon receiving the capability request message, the MCVideo client responds to the received capability request message by sending a capability announcement message to the sender of the capability request message.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo client 2 is pre-configured in MCVideo client 1.

2.	MCVideo client 1 has discovered MCVideo client 2 in proximity using ProSe Discovery procedures.

Figure 7.5.3.4.3-1: Request capabilities from a particular client

1.	MCVideo client 1 sends a capability request message towards MCVideo client 2.

2.	Upon receiving the capability request message, the MCVideo client 2 responds with a capability announcement message, which contains its capability information with other relevant information.

3.	MCVideo client 1, upon receiving a capability announcement message, caches MCVideo client 2's presence and capability information along with other relevant information.

NOTE:	Capability announcement may include specific category tags (e.g. certain video camera), video capabilities of the client, location, camera angles, camera orientation, etc.

# /****************************Next changes **********************/

7.5.3.5.2	Request activity status of group members

Figure 7.5.3.5.2-1 describes procedures to request activity status of other MCVideo clients having particular characteristics.

MCVideo client sends an activity status request message to other MCVideo clients with search criteria. The search criteria may include MCVideo user ID or MCVideo client ID or a set of capabilities or a particular category of capabilities, or a mix of such criterions, etc.

Upon receiving the activity status request message, the MCVideo client that fulfils the search criteria responds to the received activity status request message with an activity status announcement message.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCVideo client 1.

2.	MCVideo client 1 to MCVideo client N are members of the same MCVideo group.

Figure 7.5.3.5.2-1: Request activity status of group members

1.	MCVideo client 1 sends an activity status request message with search criteria to request MCVideo clients of the MCVideo group, with particular characteristics, to share their activity status.

2.	Upon receiving an activity status request message with search criteria, all MCVideo clients which fulfil the search criteria respond with an activity status announcement message, which contains its activity status information.

NOTE:	If the activity status request message does not contain a search criterion, all the MCVideo clients that receive the activity status request message respond with an activity status announcement message.

3.	MCVideo clients, upon receiving an activity status announcement message from another MCVideo client, cache its presence and activity status information.

# /****************************Next changes **********************/

7.7.1.3.2A	End media reception – receiving user initiated

Figure 7.7.1.3.2A-1 describes the procedure for the receiving user-initiated media reception end. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A has been granted the permission to transmit media.

Figure 7.7.1.3.2A-1: End media reception – receiving user initiated

1.	The MCVideo user determines to end a receiving video stream.

2.	The user sends a media reception end request including the video stream to be ended to the transmission control server.

3.	The transmission control server returns a media reception end response.

4.	Upon receiving the media reception end request, the transmission control server stops sending the video stream selected to the transmission control participant B.

5.	If the end media reception at transmission control participant B causes the video transmission continuation condition to fail, then the transmission control server initiates the media transmission end procedure as described in subclause 7.7.1.3.6.2.

7.7.1.3.2B	End media reception – transmission control server initiated

Figure 7.7.1.3.2B-1 describes the procedure for the transmission control server-initiated media reception end. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A has been granted the permission to transmit media.

Figure 7.7.1.3.2B-1: End media reception – transmission control server initiated

1.	The transmission control server determines to end a video stream transmitting to transmission control participant B according to events or configurations.

2.	The transmission control server sends a media reception end request including the video stream to be ended to the transmission control participant B.

3.	The MCVideo user is notified about the media reception end.

4.	The transmission control participants B returns a media reception end response.

5.	The transmission control server stops sending the video stream selected to the transmission control participant B.

6.	If the end media reception at transmission control participants B causes the continuation conditions to fail, then the transmission control server initiates media transmission end procedure as described in subclause 7.7.1.3.6.2.

# /****************************Next changes **********************/

7.7.1.3.4	Queue position during an MCVideo session

Figure 7.7.1.3.4-1 describes the procedure where the transmission control is conducted for the MCVideo session already established between MCVideo clients (with transmit media granted to transmission control participant B) and server (with a revoke determination at transmission control server). Only two UEs involved in the session are shown for the simplicity.

Figure 7.7.1.3.4-1: Queue status during an MCVideo session

1.	It is assumed that transmission control participant B has been granted permission to transmit media and is transmitting video media. There are several other transmission control participants (including transmission control participant A) requesting video transmission which get queued at the transmission control server.

2.	Transmission control participant A would like to know its current position in the video transmission queue.

3.	Transmission control participant A sends a queue position request message to the transmission control server.

4.	Transmission control server determines the current queue position of transmission control participant A from the video transmission queue.

5.	Transmission control server responds with the current position in queue position info message.

6.	User at transmission control participant A is informed about the current queue position.

# /****************************Next changes **********************/

7.7.2.2.1	Transmission request

Table 7.7.2.2.1-1 describes the information flow for the transmission request sent by a transmission participant to request for the transmission permission. This information flow is sent in unicast or broadcast.

Table 7.7.2.2.1-1: Transmission request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting the transmission permission |
| Transmission priority | M | Priority of the request |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

7.7.2.2.2	Transmission granted

Table 7.7.2.2.2-1 describes the information flow for the transmission granted sent by the transmission arbitrator, to indicate that a request for transmission is granted and media may be transmitted. This information flow is sent in unicast or broadcast.

Table 7.7.2.2.2-1: Transmission granted

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose client is acting as transmission arbitrator |
| MCVideo ID | M | Identity of the user that has been granted transmission permission |
| Duration | M | The time for which the granted party is allowed to transmit |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| MCVideo ID of subsequent arbitrator | O | Subsequent transmission arbitrator's identity |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission participant is required |

7.7.2.2.3	Transmission release

Table 7.7.2.2.3-1 describes the information flow for transmission release sent by the transmission participant, to indicate that the media transmission is complete and transmission permission is released. This information flow is sent in unicast or broadcast.

Table 7.7.2.2.3-1: Transmission release

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of user releasing transmission |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

7.7.2.2.4	Transmission rejected

Table 7.7.2.2.4-1 describes the information flow for transmission rejected sent by the transmission arbitrator, to indicate that a request for the transmission is rejected. This information flows is sent in unicast or broadcast.

Table 7.7.2.2.4-1: Transmission rejected

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose client is acting as transmission arbitrator |
| MCVideo ID of rejected party | M | Identity of user whose transmission request has been rejected |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Rejection cause | O | Indicates the cause for transmission rejection |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission participant is required |

7.7.2.2.5	Transmission revoked

Table 7.7.2.2.5-1 describes the information flow for transmission revoked sent by the transmission arbitrator, to indicate that the transmission permission, that was earlier granted, is revoked. This information flow is sent in unicast or broadcast.

Table 7.7.2.2.5-1: Transmission revoked

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose client is acting as transmission arbitrator |
| MCVideo ID | M | Identity of user whose transmission permission has been revoked |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission participant is required |

7.7.2.2.6	Transmission arbitration taken

Table 7.7.2.2.6-1 describes the information flow for transmission taken sent by the transmission participant, to indicate that the transmission participant has taken the responsibility of transmission arbitration. This information flow is sent in unicast or broadcast.

Table 7.7.2.2.6-1: Transmission arbitration taken

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the MCVideo user taking responsibility of transmission arbitration |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Permission to request the transmission | O | Indicates whether receiving parties are allowed to request the transmission or not (e.g. broadcast call). |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission participant is required |

7.7.2.2.7	Transmission arbitration release

Table 7.7.2.2.7-1 describes the information flow for transmission arbitration release sent by the transmission arbitrator, to indicate that the responsibility of transmission arbitration is released. This information flow is sent in unicast or broadcast.

Table 7.7.2.2.7-1: Transmission arbitration release

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose client is acting as transmission arbitrator |
| MCVideo ID | O | Identity of the user whose client is being delegated transmission arbitrator function |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

# /****************************Next changes **********************/

7.19.2.20	Modify ad hoc group call criteria request (MCVideo client – MCVideo server)

Table 7.19.2.20-1 describes the information flow Modify ad hoc group call criteria request from the MCVideo client to the MCVideo server.

Table 7.19.2.20-1: Modify ad hoc group call criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the equesting MCVideo user who is authorized to modify the ad hoc group criteria |
| Functional alias | O | The functional alias of the requesting MCVideo user who is authorized to modify the ad hoc group criteria |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can belocation-based criteria to invite participants in a particular area |

7.19.2.20a	Modify ad hoc group call criteria request (MCVideo server – MCVideo server)

Table 7.19.2.20a-1 describes the information flow Modify ad hoc group call criteria request from the MCVideo server to the MCVideo server.

Table 7.19.2.20a-1: Modify ad hoc group call criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be location-based criteria to invite participants in a particular area |

# /****************************Next changes **********************/

##### 7.19.3.1.6	Modification of ad hoc group call criteria by an authorized user

Figure 7.19.3.1.6-1 below illustrates the modification of ad hoc group call criteria procedure by an authorized user.

Pre-conditions:

1.	The MCVideo user at the MCVideo client 1 is authorized to modify the criteria.

2.	Both the MCVideo server and the MCVideo client 1 are aware of the criteria related to the ongoing ad hoc group call.

Figure 7.19.3.1.6-1: Modification of ad hoc group call criteria by the authorized user

1.	The MCVideo user at the MCVideo client 1 is authorized and requests to modify the criteria for determining the list of participants. The MCVideo client 1 sends the modify ad hoc group call criteria request to the MCVideo server which contains the updated criteria to determine the list of participants.

2.	The MCVideo server verifies whether the MCVideo client 1 is authorized to modify the criteria which determines the list of participants during on-going ad hoc group video call. The MCVideo server determines the list of ad hoc group call participants based on the criteria provided and the added/removed participant by the authorized user as marked in clause 7.19.3.1.4.

If MCVideo client 3 satisfies the new criteria to be removed and

- has already been added by an authorized user, the MCVideo server does not remove it from the call and skips the step 4 to step 6.

- has not been already added by an authorized user, the procedure continues with step 4.

If MCVideo client 2 satisfies the new criteria to be added and

- has already been removed by an authorized user, the MCVideo server does not add it back to the call and skips the step 7 to step 9.

- has not been already removed by an authorized user, the procedure continues with step 4.

Depending on the criteria provided and based on local policy, the MCVideo server may modify the content of the criteria received in step 1 to determine the list of participants.

3.	The MCVideo server sends modify ad hoc group call criteria response containing the call resulting criteria used by the MCVideo server to determine the list of participants to be invited to the MCVideo client 1.

4.	The MCVideo server sends the ad hoc group call leave request to the MCVideo client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCVideo client 3 notifies the user of the ad hoc group call leave request.

6.	The MCVideo client 3 sends the ad hoc group call leave response to the MCVideo server.

7.	The MCVideo server sends the ad hoc group call request towards MCVideo client 2.

NOTE 1:	Steps 7 to 9 can occur at any time following step 3.

8.	The receiving MCVideo client 2 notifies the user about the incoming ad hoc group call.

9.	The MCVideo client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCVideo server. The response may also contain a functional alias of the responding MCVideo user, which is verified (valid and activated for the user) by the MCVideo server. The MCVideo server considers the MCVideo user as implicitly affiliated to the ad hoc group.

10.	The MCVideo server may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call. All ad hoc group call notify messages contain the call resulting criteria used by the MCVideo server to determine the list of participants to be invited.

11.	The MCVideo server may notify the participants about the change in the participants list of on-going ad hoc group call. All ad hoc group call notify messages contain the call resulting criteria used by the MCVideo server to determine the list of participants to be invited.

The MCVideo server continuously checks whether other MCVideo clients meet or if participating MCVideo clients no longer meet the criteria for the ad hoc group emergency call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# /****************************Next changes **********************/

7.19.3.2.5	Procedure for ad hoc group call release by MCVideo server – Participants list provided by the Initiator

This procedure focuses on the case where an MCVideo server initiates the termination of an ongoing MCVideo ad hoc group call for all the participants of that group call, since at least one of the termination conditions are met e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCVideo group members are not present.

Procedures in figure 7.19.3.2.5-1 are the signalling control plane procedures for the MCVideo server initiating termination of an ongoing MCVideo ad hoc group call.

Pre-condition:

-	The MCVideo client 1 and MCVideo client 2 belong to primary MC system. The MCVideo client 3 belongs to partner MC system.

Figure 7.19.3.2.5-1: Ad hoc group call release

1.	The MCVideo users on MCVideo client 1, client 2 and client 3 are already part of the ongoing ad hoc group call (e.g. as a result of ad hoc group call setup as specified in clause 7.19.3.2.4).

2.	The MCVideo server would like to release the ad hoc group call which is ongoing e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCVideo group members are not present.

3.	The MCVideo server identifies the participants of the ongoing ad hoc group call and generates ad hoc group call release request to release ongoing session.

4.	The MCVideo server sends an ad hoc group call release request via SIP core towards each participant of the ongoing group call. If the participants belong to partner MC system, then ad hoc group call release request is routed to the MCVideo clients of the partner MC system via the partner MCVideo server.

5.	The MCVideo users are notified about the release of the ad hoc group call.

6.	The MCVideo client(s) receiving ad hoc group call release request, send acknowledgement towards the MCVideo server by sending a ad hoc group call release response.

7.	The MCVideo client 1, client 2 and client 3 have successfully released the transmission control and media plane resources associated with the ad hoc group call that is terminated. The MCVideo servers remove the ad hoc group information from the dynamic data held in the MCVideo servers and the ad hoc group ceases to exist, otherwise if the ad hoc group call follows an ad hoc group emergency alert the MCVideo servers keep the dynamic data and the ad hoc group continues to exist.