3GPP TSG-SA WG6 Meeting #65	S6-250122

Athens, Greece, 17th – 21st February 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0637 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN in clause 7.5.2.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following EN “Editor's note: Whether SIP-2 and SIP-3 reference points are used for non-subscription/notification signalling messages is FFS.” in clause 7.5.2.24 can be removed as clause 10.9.3.10 has specified several non-subscription/notificaiton interactions between CSC servers in different MC system. When the MC gateway server is deployed, such signallings has to be routed via the MC gateway server. In another aspect, the CSC server will use the HTTP for the non-subscription/notification signalling messages. So, the EN can be removed by adding a new sentence indicating the HTTP is used for non-subscription/notification signalling messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN in clause 7.5.2.24 and add description about using HTTP for non-subscription/notification signalling messages |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid editor’s note |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

7.5.2.24	Reference point CSC-23 (between location management server and MC gateway server)

The CSC-23 reference point, which exists between location management server and MC gateway server, is used to handle location management related signalling.

The CSC-23 reference point uses SIP-2 and SIP-3 reference points for transport and routing of subscription/notification related signalling.

The CSC-23 reference point uses the HTTP-1, HTTP-2 and HTTP-3 reference points for transport and routing of non-subscription/notification related signalling.