3GPP TSG-SA WG6 Meeting #65	S6-250131

Athens, Greece 17th – 21st February 2025	(revision of S6-25xxxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new solution to KI on NRM XR API

Spec:	3GPP TR 23.700-35 v0.2.0

Agenda item:	8.9

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to a new solution to the gaps identified in clause 4.1.

2. Reason for Change

As described in the corresponding discussion paper in “Service API principle” and “New API service design guideline”.

5xxxx.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-35 v0.2.0.

* * * First Change * * * *

8.a	Solution #x: XR session service

8.a.1	Solution description

This solution proposes to address the gaps identified in clause 4.1 regarding the support XR specific service at NRM.

It is proposed to introduce a new set of NRM APIs dedicated to XR service based on the following principles:

-	The new NRM APIs are designed per application layer requirements as described in clause 8.4.

- A single service API serves a single service;

- The input parameters of each service API are minimum and necessary to achieve the desired benefit corresponding to the application requirement(s);

- The name of the service, service APIs, input parameters are familiar to the XR application community.

The list of new NRM XR service API are as follows:

-	XR session with guaranteed bitrate API. This API enables the VAL server to request XR related network resource and handlings for XR service, e.g., guaranteed bitrate.

-	XR session monitoring API. This API enables the VAL server to subscribe the interested performance measurement regarding the XR session including UL packet delay, DL packet delay -trip delay, congestion (i.e., a percentage of congestion level), DL and/or UL data rate, packet delay variation and get the corresponding notifications.

-	XR session with video frame/slice guarantee API. This API enables the VAL server to enjoy transmission guarantee per video frame/slice(s) granularity, especially in a limited network resource case where the network applies differentiated handling to the e.g., XR frame/Slice(s) based on the PDU set handing information provided by the VAL server.

-	XR session with L4S ECN marking API: This API enables the VAL server to obtain the L4S ECN marking added by the network in the IP packet for further congestion control and flow control at the application layer.

8.a.1.1	Procedures

8.a.1.1.1	XR session with guaranteed bitrate API

8.a.1.1.1.1	XR session with guaranteed bitrate create

The figure 8.a.1.1.1.1-1 illustrates the high-level procedure of VAL server invoking the NRM XR session with guaranteed bandwidth API to create a network session with requested bitrate requirements.

Pre-condition:

- The application traffic delivery is handled by VAL server and VAL client at the application layer.

Figure 8.a.1.1.1-1: XR session with guaranteed bitrate service create procedure

1.	The VAL server determines to request the network XR session for XR service. It sends the XR network session with guaranteed bitrate create request to the NRM server. In the request, the following parameters are included:

-	Bitrate parameters: This is used to request the desired bitrate, and it contains the requested maximum bitrate, requested guaranteed bitrate and requested packet error rate.

The following parameters are optionally included per XR application requirements:

-	Alternative set of bitrate parameters: This is the alternative bitrate parameters the XR service may accept if the primary bitrate parameters cannot be fulfilled by the network.

2.	Based on the received request, the NRM further invokes the Nnef_AFsessionWithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO domain)  or Npcf+policyAuthorization_create request (if the NRM server is owned by MNO). The NRM server selects the corresponding features and set the input parameters of the network API based on the parameters received in step 1.

3.	The NEF or PCF process the NRM server’s request.

4.	The NEF or PCF returns the corresponding Nnef_AFsessionWithQoS_create response or Npcf+policyAuthorization_create response.

5.	The NRM server sends the XR session with guaranteed bitrate create response with result to the VAL server.

8.a.1.1.1.2	XR session with guaranteed bitrate update

Compared with the XR session with guaranteed bitrate update procedure and information flows, the only differences are the message name in the update procedure is XR network session with guaranteed bitrate update request and response. Accordingly, the NRM server invokes the Nnef_AFsessionWithQoS_update or Npcf+policyAuthorization_update.

8.a.1.1.1.3	XR session with guaranteed bitrate cancel

The figure 8.a.1.1.3-1 illustrates the high-level procedure of VAL server invoking the NRM XR session with guaranteed bandwidth API to cancel the requested bitrate.

Figure 8.a.1.1.3-1: XR session with guaranteed bitrate cancel procedure

1.	The VAL server determines to cancel the requested bitrate for XR service. It sends the XR network session with guaranteed bitrate cancel request to the NRM server.

2.	Based on the received request, the NRM further invokes the Nnef_AFsessionWithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO domain)  or Npcf+policyAuthorization_delete request (if the NRM server is owned by MNO).

3.	The NEF or PCF process the NRM server’s request.

4.	The NEF or PCF returns the corresponding Nnef_AFsessionWithQoS_delete response or Npcf+policyAuthorization_delete response.

5.	The NRM server sends the XR network session with guaranteed bitrate cancel response with result to the VAL server.

8.a.1.1.2	XR session monitoring API

8.a.1.1.2.1	XR session monitoring subscribe and notification

The figure 8.a.1.1.2.1-1 illustrates the high-level procedure of VAL server invoking the NRM XR session monitoring API to subscribe the interested performance measurement regarding the XR network session and get the notifications.

Pre-condition:

- The application traffic delivery is handled by VAL server and VAL client at the application layer.

Figure 8.a.1.1.2.1-1: XR session monitoring subscribe and notification

1.	The VAL server invokes the XR session monitoring API by sending XR session monitoring subscribe request to the NRM server. Service data flow description, expected measurement parameters are provided in the request. Reporting settings e.g., frequency (event triggered, periodic), reporting period.

2.	Upon receiving the request,  the NRM server further interacts with the NEF or PCF to initiate the QoS monitoring procedure as defined in 3GPP TS 23.501 [23.501], 3GPP TS 23.502 [23.502], 3GPP TS 23.503 [23.503].

3.	The NRM server returns the XR session monitoring subscribe response to the VAL server.

4.	When the triggers are met at the 3GPP system, the QoS monitoring unification or QNC notification is send to the NRM server.

5.	The NRM server further sends XR session monitoring notification to the VAL server. The measured parameters are included. If alternative set of bitrate parameters are enabled, a QNC unification may be received in step 4, the notification also includes the bitrate parameters no longer be satisfied and the alternative bitrate parameters being fulfilled.

NOTE:	Step 4 and step 5 may occur multiple times.

8.a.1.1.2.2	XR session monitoring subscribe update

The VAL server is able to update the existing XR monitoring subscription. The figure 8.a.1.1.2.2-1 illustrates the high-level procedure of VAL server invoking the NRM XR session monitoring API to update the existing subscription of the interested performance measurement regarding the XR network session and get the notifications.

Figure 8.a.1.1.2.2-1: XR session monitoring subscribe update and notification

1.	The VAL server sends the XR session monitoring subscribe update request to the NRM server. Subscription ID, updated measurement parameters, Reporting settings are provided in the request.

2.	Upon receiving the request,  the NRM server further interacts with the NEF or PCF to update the QoS monitoring parameters as defined in 3GPP TS 23.501 [23.501], 3GPP TS 23.502 [23.502], 3GPP TS 23.503 [23.503].

3.	The NRM server returns the XR session monitoring subscribe update response to the VAL server.

When the updated triggers are met, similar step 4 and step 5 as clause 8.a.1.1.2.1 are performed.

8.a.1.1.2.3	XR session monitoring unsubscribe

The VAL server is able to cancel the existing XR monitoring subscription. The figure 8.a.1.1.2.3-1 illustrates the high-level procedure of VAL server invoking the NRM XR session monitoring API to unsubscribe the existing subscription of the interested performance measurement regarding the XR network session.

Figure 8.a.1.1.2.3-1: XR session monitoring subscribe update and notification

1.	The VAL server sends the XR session monitoring unsubscribe request to the NRM server. Subscription ID is included.

2.	The NRM server further interacts with the 3GPP core network to cancel the QoS monitoring subscription accordingly.

3.	The NRM server sends XR session monitoring unsubscribe response to the VAL server.

8.a.1.1.3	XR session with video frame/slice guarantee  API

8.a.1.1.3.1	XR session with video frame/slice guarantee create

The figure 8.a.1.1.3.1-1 illustrates the high-level procedure of VAL server invoking the NRM XR session with video frame/slice guarantee API to create a network session with requested video from/slice(s) level requirements.

Figure 8.a.1.1.1-1: XR session with video frame/slice guarantee create procedure

1.	The VAL server sends the XR session with video frame/slice guarantee create request to the NRM server. In the request, the following parameters are included:

-	Video frame/slice guarantee parameters: This is the desired video frame/slice level transmission requirements, and it contains the requested network delay budget, error rate;

-	Protocol description indicating the transport protocol used by the service data flow.

The following parameters are optionally included per XR application requirements:

-	Alternative set of frame/slice guarantee parameters: This is the alternative frame/slice guarantee parameters the XR service may accept if the primary frame/slice guarantee parameters cannot be fulfilled by the network.

2.	Based on the received request, the NRM further invokes the Nnef_AFsessionWithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO domain)  or Npcf+policyAuthorization_create request (if the NRM server is owned by MNO). The NRM server selects the corresponding features and set the input parameters of the network API based on the parameters received in step 1.

3.	The NEF or PCF process the NRM server’s request.

4.	The NEF or PCF returns the corresponding Nnef_AFsessionWithQoS_create response or Npcf+policyAuthorization_create response.

5.	The NRM server sends the XR session with video frame/slice guarantee create response with result to the VAL server.

8.a.1.1.3.2	XR session with video frame/slice guarantee update

Compared with the XR session with video frame/slice guarantee create procedure and information flows, the only differences are the message name in the update procedure is XR session with video frame/slice guarantee update request and response. Accordingly, the NRM server invokes the Nnef_AFsessionWithQoS_update or Npcf+policyAuthorization_update.

8.a.1.1.3.3	XR session with video frame/slice guarantee cancel

The figure 8.a.1.3.3-1 illustrates the high-level procedure of VAL server invoking the NRM XR session with video frame/slice guarantee API to cancel the video frame/slice guarantee from the network session.

Pre-condition:

Figure 8.a.1.3.3-1: XR session with video frame/slice guarantee cancel procedure

1.	The VAL server determines to cancel the video frame/slice(s) guarantee for XR service. It sends the XR network session with video frame/slice guarantee cancel request to the NRM server.

2.	Based on the received request, the NRM further invokes the Nnef_AFsessionWithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO domain)  or Npcf+policyAuthorization_delete request (if the NRM server is owned by MNO). The NEF or PCF process the NRM server’s request. The NEF or PCF returns the corresponding Nnef_AFsessionWithQoS_delete response or Npcf+policyAuthorization_delete response.

3.	The NRM server sends the XR network session with video frame/slice guarantee cancel response with result to the VAL server.

8.a.1.1.4	XR session with L4S ECN marking API

8.a.1.1.4.1	XR session with XR session with L4S ECN marking enable

The figure 8.a.1.1.4.1-1 illustrates the high-level procedure of VAL server invoking the NRM XR session with L4S ECN marking API to request the network to apply the L4S ECN marking for the service data flow.

Figure 8.a.1.4.1-1: XR session with L4S ECN marking enable procedure

1.	The VAL server sends the XR session with L4S ECN marking enable request to the NRM server. The service data flow description indicating the service data flow to be marked is included.

2.	Based on the received request, the NRM further invokes the Nnef_AFsessionWithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO domain)  or Npcf+policyAuthorization_create request (if the NRM server is owned by MNO). The NRM server selects the corresponding features and set the input parameters of the network API based on the parameters received in step 1. The NEF or PCF process the NRM server’s request. The NEF or PCF returns the corresponding Nnef_AFsessionWithQoS_create response or Npcf+policyAuthorization_create response.

3.	The NRM server sends the XR session with L4S ECN marking enable response with result to the VAL server.

8.a.1.1.4.2	XR session with XR session with L4S ECN marking cancel

The figure 8.a.1.1.4.2-1 illustrates the high-level procedure of VAL server invoking the NRM XR session with L4S ECN marking API to cancel the L4S ECN marking for the service data flow.

Figure 8.a.1.4.1-1: XR session with L4S ECN marking enable procedure

1.	The VAL server sends the XR session with L4S ECN marking cancel request to the NRM server. The service data flow description that the L4S ECN marking has enabled is included.

2.	Based on the received request, the NRM further invokes the Nnef_AFsessionWithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO domain)  or Npcf+policyAuthorization_update request (if the NRM server is owned by MNO). The NRM server selects the corresponding features and set the input parameters of the network API based on the parameters received in step 1. The NEF or PCF process the NRM server’s request. The NEF or PCF returns the corresponding Nnef_AFsessionWithQoS_create response or Npcf+policyAuthorization_create response.

3.	The NRM server sends the XR session with L4S ECN marking cancel response with result to the VAL server.

8.a.1.2	Information flows

8.a.1.2.1	XR network session with guaranteed bitrate create/update request

Table 8.a.1.2.1-1 describes the information flow from VAL server to NRM server for unicast resources create/update.

Table 8.a.1.2.1-1: XR network session with guaranteed bitrate create/update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE. |
| Bitrate parameters | M | Indicates the bitrates requirements of the XR service. It contains the requested maximum bitrate, requested guaranteed bitrate and requested packet error rate. |
| Alternative bitrate parameters set | O | Indicates the alternative bitrates requirements of the XR service. It contains the requested maximum bitrate, requested guaranteed bitrate and requested packet error rate. |

8.a.1.2.2	XR network session with guaranteed bitrate create/update response

Table 8.a.1.2.2-1 describes the information flow from NRM server to VAL server for XR network session with guaranteed bitrate create/update response.

Table 8.a.1.2.2-1: XR network session with guaranteed bitrate create response

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE whose network session is to be released. |

8.a.1.2.3	XR network session with guaranteed bitrate delete request

Table 8.a.1.2.3-1 describes the information flow from NRM server to VAL server for XR network session with guaranteed bitrate delete request.

Table 8.a.1.2.3-1: XR network session with guaranteed bitrate delete request

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource request operation. |
| Cause | O | Indicates the failure cause |

8.a.1.2.4	XR network session with guaranteed bitrate delete response

Table 8.a.1.2.4-1 describes the information flow from NRM server to VAL server for XR network session with guaranteed bitrate delete response.

Table 8.a.1.2.4-1: XR network session with guaranteed bitrate delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource request operation. |
| Cause | O | Indicates the failure cause |

8.a.1.3	NRM XR service and APIs

8.a.1.3.1	XR session with guaranteed bitrate API

Editor’s note:	The detail of this clause is FFS.

8.a.1.3.2	XR session monitoring API

Editor’s note:	The detail of this clause is FFS.

8.a.1.3.3	XR session with video frame/slice guarantee  API

Editor’s note:	The detail of this clause is FFS.

8.a.1.3.4	XR session with L4S ECN marking API

Editor’s note:	The detail of this clause is FFS.

8.a.2	Impacts on architecture, services, entities

This solution has no architecture impacts.

This solution creates new NRM XR service and APIs at NRM.

8.a.3	Solution evaluation

This solution provides an NRM XR service complied with the friendly API design principle.