3GPP TSG-SA6 Meeting #65	S6-250133

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0013 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolutions in TS 23.482 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The AIML_App work has been completed; however some ENs need resolution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | There are multiple ENs resolved in the following: - In Tables 8.3.3.1-1,8.6.3.2-1and 8.6.3.6-1 there are ENs for addional IEs which are FFS. Such ENs can be removed since there work is finalized. - In Table 8.11.4.1-1 (ML model information storage request), an EN related to adding security credentials is resolved by adding a new IE applicable to the case where the requestor is VAL / external entity. - In clause 8.18 the EN is converted to NOTE since it needs proper study in future release before adding a new procedure. - In clause 8.22 there is an EN for adding the unsubscribe and subscribe update procedures. However, such procedures require to study the rationale behind updating or terminating a subscription and from implementation point of view the feature is not incomplete since there is a time validity for the subscription. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs will not be resolved in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8.3.3.1, clause 8.6.3.2, clause 8.6.3.6, clause 8.11.4.1, clause 8.18.2 , clause 8.22.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.3.3	Information flows

#### 8.3.3.1	ML model training request

Table 8.3.3.1-1 describes the information flow from the VAL server to the AIMLE server for requesting the ML model training.

Table 8.3.3.1-1: ML model training request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Training type | M | Identifies whether the VFL or HFL training to be performed. |
| List of member clients | O (NOTE 1) | List of member clients to be utilized for training the ML model. |
| Member selection criteria | O (NOTE 1) | Identifies the criteria that needs to be continuously monitored for selecting the member clients (e.g., AIMLE clients in a particular location, member availability duration, new member clients registering with the required capabilities). |
| ML model information | O (NOTE 2) | Identifies the ML model that has to be distributed to the selected member clients for training. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model requirement information | O (NOTE 2) | Identifies the requirement for selecting a model to be trained and this information contains the filtering criteria for selecting the model as specified in Table 8.11.4.1-2. |
| Members update notification | O | Indicates whether requestor needs to be notified whenever there is update related to new member clients selected or de-selected. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| NOTE 1: At least one of these IE shall be present. NOTE 2: At least one of these IE shall be present. |  |  |

* * * Next Change * * * *

### 8.6.3	Information flows

#### 8.6.3.1	General

The following information flows are specified for supporting AI/ML task transfer. In the following clauses 8.6.3.2 to 8.6.3.9, the source AI/ML member and the target AI/ML member could be e.g., VAL UE, AIMLE Client.

#### 8.6.3.2	AI/ML task transfer assist request

Table 8.6.3.2-1 describes information elements for the AI/ML task transfer assist request from the source AI/ML member to the AIMLE Server.

Table 8.6.3.2-1: AI/ML task transfer assist request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL UE (the source AI/ML member). |
| VAL service ID | O | The identifier of the VAL service for which the assistance information is requested. |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| > AI/ML model remaining training requirement (NOTE) | O | Indicate the requirement for AI/ML model training including, required remaining training resource, required remaining training number of iterations, |
| > AI/ML intermediate information (NOTE) | O | Indicate the AI/ML intermediate information for intermediate AI/ML operation, including AI/ML intermediate model, AI/ML intermediate model used training time, used training resource, used training number of iterations. |
| Time validity | O | The time validity of the request. |
| NOTE: The IE only present when AI/ML Task Type is ML model training. |  |  |

#### 8.6.3.3	AI/ML task transfer assist response

Table 8.6.3.3-1 describes information elements for the AI/ML task transfer assist response from the AIMLE Server to the source AI/ML member.

Table 8.6.3.3-1: AI/ML task transfer assist response

| Information element | Status | Description |
| --- | --- | --- |
| Transfer Mode | O | Indication the transfer mode (e.g. direct transfer). |
| Target AI/ML member(s) | M | The identifier of the VAL UE (the target AI/ML member). |
| Assistance Information | M | Assistance information for the AI/ML task transfer (e.g. time window for the transfer). |

#### 8.6.3.4	AI/ML task transfer request

Table 8.6.3.4-1 describes information elements for the AI/ML task transfer request from the AIMLE Server to the target AI/ML member.

Table 8.6.3.4-1: AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL Server. |
| VAL UE ID | M | The identifier of the VAL UE (the source AI/ML member). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| AI/ML Task Transfer Time | O | Information on time or time window for the AI/ML task transfer. |
| Time validity | O | The time validity of the request. |

#### 8.6.3.5	AI/ML task transfer response

Table 8.6.3.5-1 describes information elements for the AI/ML task transfer response from the target AI/ML member to the AIMLE Server.

Table 8.6.3.5-1: AI/ML task transfer response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AI/ML Task Transfer Time | O | Information on time or time window for the AI/ML task transfer. |

#### 8.6.3.6	Direct AI/ML task transfer request

Table 8.6.3.6-1 describes information elements for the direct AI/ML task transfer request from the source AI/ML member to the target AI/ML member.

Table 8.6.3.6-1: Direct AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL UE (the source AI/ML member). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| AI/ML Task Transfer Time | O | Information on time or time window for the AI/ML task transfer. |
| Time validity | O | The time validity of the request. |

* * * Next Change * * * *

### 8.11.4	Information flows

#### 8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository or from the AIMLE client/VAL server to the AIMLE server as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | The identity of the requestor performing the request. |
| Security Credentials | M (NOTE 1) | Security credentials of the requestor performing the request. |
| ML model | O (NOTE 2) | The ML model to be stored in the ML repository. |
| ML model address | O (NOTE 2) | Represents the ML model address that can be used by Model repository to download the ML model. |
| ML model information | M | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE 1: This information is needed if the requestor is at the VAL service provider domain. NOTE 2: At least one of these information elements shall be provided. |  |  |

* * * Next Change * * * *

##### 8.14.2.5.2	Subscribe

Figure 8.14.2.5.2-1 illustrates the procedure for an AIMLE client or VAL server to subscribe with the AIMLE server to be notified of events related to split AI/ML operation.

Pre-conditions:

1.	The AIMLE client or VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

2.	The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.14.2.5.2-1: Split operation subscription

1.	The requestor (e.g., AIMLE client or VAL server) sends a split operation subscribe request to the AIMLE server. The request includes information defined in Table 8.14.3.12-1.

a.	The requestor (e.g., AIMLE client or VAL server) may include the "split operation pipeline information" event to indicate the AIMLE server to notify the requestor when a split operation profile is created or an existing pipeline is updated (e.g., node is added or removed, stage order or ML models are modified, etc.) or deleted and satisfy the discovery filters of the subscription request.

b.	The requestor (e.g., AIMLE client or VAL server) may include the " split operation assistance information" event to indicate the AIMLE server to notify the requestor when the AIMLE server determines assistance information related to the split operation usage.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server creates the subscription and stores the subscription information.

3.	The AIMLE server sends a split operation subscribe response to the requestor. If the AIMLE server has created the subscription, the response includes an indication of success, the subscription identity and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the split operation subscription expires. If the AIMLE server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

If the subscription is for "assistance information", the AIMLE Server considers the assistance information included in the subscription request and can perform the following:

The AIMLE Server aggregates the collected assistance information from NEF, NWDAF, and/or ADAES to generate assistance information, e.g. Time (time point(s) or time window(s)) to deliver the task or data for the split operation. Or the AIMLE server performs inference using ML model to generate assistance information, e.g. achievable QoS with current configuration for task or data delivery, or suggestion of QoS for task or data delivery.

-	For assistance information from NEF, the AIMLE Server, acting as AF, sends a request to NEF for assistance information (e.g. PDTQ on recommended time windows for AI/ML operations with QoS, QoS monitoring) as described in clause 4.16.15 of TS 23.502 [3]. The request may include the information of the AI/ML task/ML model/data (e.g. size of the task or ML model, or data volume), information of the receiving nodes, requirement on the delivery/distribution (e.g. time budget, time critical or not, QoS), maximum time for complete the distribution/delivery.

-	For assistance information from NWDAF, the AIMLE Server can send a request or subscription request to NWDAF for analytics (e.g. E2E data volume transfer time, DN performance, Network performance, UE mobility). The details parameters in the requests to NWDAF for analytics are given in 3GPP TS 23.288 [2].

-	For assistance information from ADAES, the AIMLE Server can send a request or subscription request to ADAES for analytics (e.g. slice-specific application performance analytics, UE-to-UE application performance analytics). The details parameters in the requests to ADAES for analytics are given in 3GPP TS 23.436 [4].

* * * Next Change * * * *

## 8.18	Support Vertical FL

This clause describes procedure for supporting VFL among application layer multiple UEs.

### 8.18.1	General

The following clauses specify procedures, information flows and APIs to support VFL among Application Layer multiple UEs.

### 8.18.2	Procedure for supporting VFL

Pre-conditions:

1.	VFL members registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server.

2.	The datasets of each of the UEs (where the AIMLE Clients (as VFL members) are deployed on) belong to more than one different data domains.

3.	The VAL server has successfully subscribed/registered with the AIMLE server for model training notifications.

Figure 8.18.2-1: Procedure for supporting VFL

1.	An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between different data domains), the procedure continues to step 2.

2.	[Optional] The AIMLE server retrieves the indicated ML model in step 1 using the ML model retrieval procedure as described in clause 8.1. If the retrieved model is already trained and meets the machine learning requirements requested by the consumer, the procedure continues to step 7.

3.	If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the VFL training as described step 4 of the clause 8.13.2. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the VFL training.

4.	The AIMLE Server gets VFL members information (the VFL members information may be included in the request in step 1 (Table 8.3.3.1-1) or be obtained through step 3. The AIMLE Server interacts with the VFL members (AIMLE Clients which are deployed on UEs) for each data domain for ML model training capability evaluation as described in clause 8.19.2.

5.	The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 4 and parameters received in step 1 (Table 8.3.3.1-1). The AIMLE Server determines VFL members for each data domain. The criteria used by the AIMLE Server include:

-	Available data and minimum number of data samples for the same sample among the VFL members.

-	Feature alignment of the sample/datasets with data labels among the VFL members.

-	Available time of the VFL members for support the VFL training operations.

-	Capability and minimum number of the VFL members for the VFL training operations.

-	AIML model information for the VFL members and for the AIMLE Server.

6.	The AIMLE Server coordinates the selected VFL members for VFL training. During VFL training process, the VFL members send intermediate results to the AIMLE Server, and the AIMLE Server responds to the VFL members with the updated information (e.g. gradients). The information from the AIMLE server can be used to update the model parameters maintained at each VFL member for the different data domains.

6a.	The AIMLE server may report to the VAL server with the training status, that includes intermediate training results, The VAL server may adjust its request on the ML model training. If the VAL Server is providing data labels to complete the training, the VAL Server sends updated training parameters for the AIMLE Server to distribute to the VFL members. The updated training parameters apply for models of VFL members associated with each data domain.

NOTE: How AIMLE Server coordinates the selected VFL members to perform VFL training is out of scope for this release.

7.	The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 6 are repeated for the next training round.

* * * Next Change * * * *

## 8.22	ML model performance monitoring

### 8.22.1	General

The following clauses specify procedures, information flows, and APIs for ML model performance monitoring and potential degradation detection.