3GPP TSG-SA WG6 Meeting #65	S6-250139

Athens, Greece 17th – 21st February 2025

Source:	Samsung

Title:	Pseudo-CR on Location Retrieval API enhancements

Spec:	3GPP TR 23.700-35 V0.2.0

Agenda item:	8.9

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

SEAL location management service defined in TS 23.434 supports retrieval of location information of one or more users. It supports one time fetch (On demand) or subscription based location retrieval. The location information could be utilized by applications running on the client or on the application server. It is required to amend the procedures and information flows in TS 23.434 to specify that application clients can also fetch the location information of other users via SEAL LMC.

2. Reason for Change

Update the location retrieval procedures to specify that it can be utilized by the application clients also.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 V0.2.0.

* * * First Change * * * *

## 4.x.	Location Management service #X

### 4.x.1	Analysis

SEAL location management service defined in TS 23.434 supports retrieval of location information of one or more users. It supports one time fetch (On demand) or subscription based location retrieval. The location information could be utilized by applications running on the client or on the application server. It is required to amend the procedures and information flows in TS 23.434 to specify that application clients can also fetch the location information of other users via SEAL LMC.

### 4.x.2	Identified gaps

Gap 4.x.2-1 : How to clarify that on-demand retrieval of location information is applicable to VAL clients also.

* * * Next Change * * * *

## 8.X	Solution X: Updates to the On demand usage of location information

### 8.x.1	Solution description

This solution is to address the “Gap 4.x.2-1: How to clarify that on-demand retrieval of location information is applicable to VAL clients also.” Required changes that need to applied to TS 23.434 are highlighted.

#### 8.x.1.1	Changes to TS 23.434

### 9.3.9	On-demand usage of location information procedure

The VAL server can request UE location information at any time by sending a location information request to the location management server, which may trigger location management server to immediately send the location report.

Figure 9.3.9-1 illustrates the high level procedure of on demand usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE.

Pre-condition:

-	The LM Server has confirmed the target UE is associated with other UEs and they are sharing the same location.

-	The LM Server has obtained the UE’s location who is associated with the target UE.

Figure 9.3.9-1: On-demand usage of location information procedure

Replace the above Figure 9.3.9-1 with the below figure

1.	VAL server/Location Management client sends a location information request to the location management server to request locations for one or more VAL UEs or VAL user(s) which may include the location QoS that contains the location accuracy, response Time and QoS class as defined in clause 4.1b of TS 23.273 [50].

2.	The location management server checks if there is stored and valid UE location report (e.g. the timer for the stored UE location report is not expired) for the target UE. If available, it reuses the stored UE location information and reports it to the VAL server. If there isn’t, it acquires the latest location of the UEs being requested, by triggering an on-demand location report procedure as described in subclause 9.3.4, or from PLMN operator, and checks if the obtained location of the VAL UE could meet the location QoS requirements (if any).

If the LM Server identified that target UE is associated with other UE(s) and they are sharing the same location as specified in clause of 9.3.23.2 or 9.3.23.3, it determines to reuse the associated UE's location data instead of obtaining the target UE's location data directly. The LM Server may select the associated UE(s) based on the requested LCS QoS, and the operator's policies, etc. But if the LM Server doesn’t store the associated UE's location data or the stored location data is invalid (e.g. the valid timer for the confirmation of sharing location information is expired, or the verify location sharing response is negative), the LM Server will ask the target UE to report the UE location data as described in clause 9.3.4.

3.	Then, location management server immediately sends the location information report including the latest location information of associated UE(s) or acquired of target one or more VAL users or VAL UEs. The location management server may report the location to the VAL server considering the location information received via non-3GPP positioning technologies (e.g. GNSS, Bluetooth), for instance, to improve the location accuracy.

4.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 4 can be skipped if not needed.

#### 9.3.2.3	Location information request

Table 9.3.2.3-1 describes the information flow from the VAL server/Location management client to the location management server and/or from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |
| Requested location information | O | Identifies what location information is requested |
| Requested location access type (NOTE 4) | O (NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Requested positioning method | O (NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. If the requested VAL user/UE is not the receiving VAL user/UE hosting the location management client, the UE positioning method includes PC5 SEAL LM, BT, WiFi. |
| Location QoS | O (NOTE 3) | Definition of the location Quality of Service for which the location information is requested (see NOTE 2). |
| Velocity requested indication | O (NOTE 5) | It indicates whether the velocity of the requested VAL users/UEs is needed. |
| List of association IDs | O (NOTE 6) | Identifies list of associations with other UEs may share the location information when they are at the same location. |
| NOTE 1: This e element is only applicable for request sent from the location management server to the location management client. NOTE 2: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 3: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 4: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. NOTE 5: The definition of the velocity refers to 3GPP TS 23.271 [60]. NOTE 6: The association IDs are known to the UE based on offline methods and are out of scope of this specification. The requestor can be part of multiple associations and one association can have two or more UEs. |  |  |

* * * End Change * * * *