3GPP TSG-SA WG6 Meeting #65	S6-250142

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Samsung

Title:	Pseudo-CR on Location Verification API

Spec:	3GPP TR 23.700-35 V0.2.0

Agenda item:	8.9

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

Location feature is one of the most widely used feature in most of the applications. TS 23.434 supports the location management service and offers various functionalities to the application layer to consume. Location verification is a feature which provides the application layer the ability to verify the location of a certain end-user device within the area specified. CAMARA project exposes the location verification API and GSMA OPAG realizes the support for the location verification.

SEAL LMS supports confirm Location verification procedure specified in the clause 9.3.26. Existing procedure of confirm Location verification mandates the subscription procedure by the VAL server which might be a overhead. Whether the UE is allowed to utilize the confirm location verification API can be done through an authorization mechanism and need not be tied up with the subscription procedure. VAL servers can subscribe if they want to monitor the usage of the APIs and which UE utilizes it.

Proposal :

It is proposed to decouple the subscription mechanism from the usage of confirm location verification API.

2. Reason for Change

As discussed in the above section it is necessary to support Location verification API decoupled from the confirm location subscription.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 V0.2.0.

* * * First Change * * * *

## 4.x.	Location Management service #X

### 4.x.1	Analysis

Location feature is one of the most widely used feature in most of the applications. TS 23.434 supports the location management service and offers various functionalities to the application layer to consume. Currently location management service do suport location verification feature the feature is coupled with the VAL server subscription for the confirm location service subscription (Clause 9.3.26.3 of TS 23.434).

Location verification is a feaure which enables the application layer to verify the location of a certain end-user device within the area specified. It is a demanding feature in order to address various usecases and some are listed below :

- Fraud protection, to ensure a given end-user is located in the location area claimed for financial transactions.

- Verification of GPS coordinates reported by the application on a device, to ensure the GPS was not faked, e.g. for content delivery with regional restrictions.

- Contextual-based advertising, to trigger advertising after verifying the device is in the area of interest.

Hence it is important to study how the location verification feature can be supported irrespecitive of the confirm location service subscription.

### 4.x.2	Identified gaps

Gap 4-x-2-1: How to enhance SEAL Location management service to support the location verification feature decoupled from confirm location service subscription.

* * * Next Change * * * *

## 8.X	Solution X: Updates to the Confirm Location verification feature

### 8.x.1	Solution description

This solution is to address the” Gap 4-x-2-1: How to enhance SEAL Location management service to support the location verification feature decoupled from confirm location service subscription”. Required changes that need to applied to the clause 9.3.26 of TS 23.434 are as below:

1. Delete the yellow highlighted text from TS 23.434.

2. Incorporate the green highlighted text into TS 23.434.

#### 8.x.1.1	Changes to clause 9.3.26.2 of TS 23.434

### 9.3.26	Verify UE location procedure

#### 9.3.26.1	General

Verify UE location could be used by multiple applications that need confirmed location to provide their service. The UE application logic would first confirm customer location before accepting customer requests for service in a specific location.

This procedure provides the VAL server application with subscription for verification service and supports verification of UE provided location by the LMS. Only UEs using application with such subscription can use the confirm location verification from LMS. Sharing of UE location to the network is only possible with user consent. The network provided location is not directly shared but only verification is provided to the requesting LM client. The verified location is for internal UE usage.

#### 9.3.26.2	Confirm location service subscription

The application is running on a VAL server. The application would like to use confirm location service and subscribes to LMS for it using the following procedure:

Figure 9.3.26.2-1 illustrates the procedure for confirm location service subscription. This procedure can be used by the VAL server to get notified whenever SEAL LMC utilizes the confirm location verification service.

Figure 9.3.26.2-1: Procedure of confirm location service subscription

1.	The VAL server subscribes to LMS to receive notification every time UE using specific application is requesting confirmation of location.

2.	The LMS shall check if the VAL server is authorized to subscribe for the confirm location service notifications for a specific application identifier.

3.	If the VAL server and specific application are authorized to subscribe for confirm location service, the LMS creates subscription based on provided parameters in the request.

4.	The LMS sends to VAL server confirm location service subscription response.

#### 9.3.26.3	Confirm location verification

The location management client (LMC)/application in the UE gets from non-3GPP positioning (e.g. GNSS) in the UE its location. The application in the UE wants to check if this is really the correct location.

Pre-conditions:

1.	The LMC is authorized to discover and to use Confirm Location API provided by the LMS;

2.	LMS is authorized to use Nnef Event Exposure API, based on SLA with NEF/5GS of MNO;

3.	It is assumed there is user consent given in the UE for exposing location information from UE to LMS.

Figure 9.3.26.3-1: Procedure of confirm location verification

Replace the above figure 9.3.26.3-1 with the below in TS 23.434

1.	The UE sends Confirm Location request from the LMC towards LMS. It contains the location measured from the UE (non-3GPP location information, e.g. GNSS). It contains latitude and longitude, accuracy, timestamp, carrier name, cell id (known to the UE from the network signalling), application identifier.

2.	 The LMS shall check if there is subscription (based on VAL subscription in clause 6.12.2.1) for confirm location service with the specific application identifier received in step1. If subscription is present, step 3 is performed. If subscription is not present the LMS replies back to LMC with confirm location status unknown (step 4). The LMS shall check if the Location management client is authorized to request confirm location verification and if authorized step 3 is performed and if not authorized the LMS sends confirm location report with status unknown (Step 4)

3.	LMS requests UE location from 5GC/LMF via NEF (LMS send determine Location operation via NEF (N33 interface) and using Nlmf_Location API defined in 3GPP TS 29.572 [8] – parameters available from the location reporting are described in the TS). It also requests accuracy to be a parameter present in the response. LMS can also perform a location information request to GMLC directly or via NEF (as defined in 6.1. of 3GPP TS 23.273 [5]), acting as AF. LMS can also perform a location information request to 3rd party location servers via LM-3P interface. LMS can also use the T8 interface to obtain UE location. Or the LMS can have already the UE location based on the methods described in 3GPP TS 23.434 [9]. The received response contains the UE location from the network. LMS compares the location provided from the network with the location provided from the LMC. LMS can use event exposure using NEF according to table 4.15.3.1-1 of TS 23.502 to obtain user location information from AMF.

4.	LMS sends confirm location report to LMC.

5.	The LMS sends notification towards the VAL server with the UE identifier and application using the service.

* * * End Change * * * *