3GPP TSG-SA WG6 Meeting #65	S6-250143

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	China Mobile

Title:	pCR on clause 5 Architectural requirements

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR is proposed to add the clause 5 Architectural requirements.

2. Reason for Change

This pCR is proposed to add the clause 5 Architectural requirements.

4. Proposal

This pCR is proposed to add the clause 5 Architectural requirements.

* * * First Change * * * *

5	Architectural requirements

5.1	General

This clause specifies architectural requirements for MMTel Enabler.

5.2	Data Channel Application management requirements

5.2.1	Description

This clause specifies the Data Channel Application management requirements for MMTel Enabler.

5.2.2	Requirements

[AR-5.2.2-a] The MMTel Enabler shall provide capability for MMTel service provider to upload, update and delete the Data Channel applications on DCAR via MMTel Enabler .

[AR-5.2.2-b] The MMTel Enabler shall provide capability for MMTel service provider to configure, update and deleted the Data Channel application profile on DCAR via MMTel Enabler server.

[AR-5.2.2-c] The MMTel Enabler shall provide capability for MMTel service provider to get the Data Channel application profile information.

[AR-5.2.2-d] The MMTel Enabler shall provide capability to control the downloading of data channel applications on UE by providing data channel application profile to UE.

5.3	MMTel service enablement requirements

5.3.1	Description

This clause specifies the MMTel service enablement requirements for MMTel Enabler.

5.3.2	Requirements

[AR-5.3.2-a] The MMTel Enabler shall provide capability for application without IMS capabilities to establish, modify or terminate a call with an Application supporting MMTel service.

[AR-5.3.2-b] The MMTel Enabler shall provide capability for application providers/Vertical service provider to use MMTel service including establish, modify or terminate a Third-Party Call.