3GPP TSG-SA WG6 Meeting #65	S6-250149

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	China Mobile

Title:	pCR on update of 8.4.2 third party call

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR is  proposed to update the procedure and add the information flows of third party call.

2. Reason for Change

This pCR is  proposed to update the procedure and add the information flows of third party call.

4. Proposal

This pCR is  proposed to update the procedure and add the information flows of third party call.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

[5]	GSMA NG.134: "IMS Data Channel, Version 1.0".

[6]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL)".

[7]	OMA-TS-REST_NetAPI_ThirdPartyCall-V1_0-20130212-C: "RESTful Network API for Third Party Call".

[291984]	3GPP TS 29.198-04: "Open Service Access (OSA) Application Programming Interface (API); Part 4: Call control Service Capability Feature (SCF)"

[7989]	IETF RFC 7989 (October 2016): "End-to-End Session Identification in IP-Based Multimedia Communication Networks".

[291984-2]	3GPP TS 29.198-04-02: "Open Service Access (OSA) Application Programming Interface (API); Part 4: Call control; Sub-part 2: Generic call control Service Capability Feature (SCF)"

[291984-4]	3GPP TS 29.198-04-04: "Open Service Access (OSA) Application Programming Interface (API); Part 4: Call control; Sub-part 4: Multimedia call control Service Capability Feature (SCF)"

[291983]	3GPP TS 29.198-03: "Open Service Access (OSA) Application Programming Interface (API); Part 3: Framework"

[OMAComm]	OMA-TS-REST_NetAPI_Common-V1_0-20180116-A: "Common definitions for RESTful Network APIs".

* * * Next Change * * * *

8.4.2	Third-Party Call service API with Data Channel capability

8.4.2.1	General

The MMTel Enabler Server exposes the Third-Party Call service API with Data Channel capability to the Vertical service provider in order to support establishing a call between two UEs.

8.4.2.2	Procedure

8.4.2.2.1	Establishing a Third-Party Call with Data Channel capability

Figure 8.4.2.2.1-1 illustrates the operation to establish a Third-Party Call with Data Channel capability.

Pre-conditions:

1.	The Vertical service provider is authorized to use Third-Party Call service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use the IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

3.	Based on the deployment of MMTel Enabler server as specified in clause 6.2.2, the MMTel Enabler Server is authorized to use OMA THIRD PARTY CALL API as specified in OMA-TS-REST_NetAPI_ThirdPartyCall [7] or OSA Call Control APIs as specified in 3GPP TS 29.198-04 [291984].

Figure 8.4.2.2.1-1: Establishing a Third-Party Call with Data Channel capability

1.	The Vertical service provider sends a Third-Party Call with DC capability establishment request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.4.2.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then:

If the OMA Enabler is supported in the PLMN network (Option 1),

a)	the MMTel Enabler Server utilizes the OMA Third Party Call API [7]  to establish a Third-Party Call session with audio or video media as specified in OMA-TS-REST_NetAPI_ThirdPartyCall;

b)	 the OMA Enabler utilizes the OSA Application Initiated Call procedure as specified in clause 4.3 of 3GPP TS 29.198-04 [291984] to establish the call between two parties; and

c)	the MMTel Enabler Server obtains the OMA API call Session ID corresponding to the IMS session established in step a.

NOTE 1:	The call session ID attribute in the OMA Third Party Call API is generated by the entity implementing the OMA APIs, which is different from the IMS session Id. In this case, the MMTel Enabler Server handles the call session ID obtained from OMA Third Party Call API.

NOTE 2	when the Third-Party Call session is established, the IMS entity which sets up calls to both call parties can act as the B2BUA as specified in RFC 7989 [7989] and obtain the End-to-End Session id of the IMS Session as defined in RFC 7989 [7989]. The IMS entity can notify this IMS Session ID to the OMA enabler/MMTel Enabler server in the API response or API notification.

If MMTel Enabler Server supports OMA OSE (Option 2) or interact with IMS by using OSA APIs directly (Option 3),

a’)	the MMTel Enabler Server utilizes the OSA Application Initiated Call procedure as specified in clause 4.3 of 3GPP TS 29.198-04 [291984] to establish the call between two parties and obtains the Session ID corresponding to the IMS session.

3.		the MMTel Enabler Server utilizes the DC update capability of the IMS core network to add DC media to the existing Third-Party Call Session as specified in 3GPP TS 23.228 [3],

the IMS core network returns the update result.

4.	The MMTel Enabler Server sends a Third-Party Call with DC capability establishment response to the Vertical service provider. The response message includes information elements as specified in clause 8.4.2.3.2.

NOTE:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.

Editor's note 1:	The flow will need to be updated once SA2 work is done to specify NEF exposure of IMS session management (Nnef_ImsSessionManagement).

Editor's note 2:	The requirements to deploy the MMTel Enabler in a trusted 3rd party call service provider domain is FFS.

8.4.2.3	Information flows

8.4.2.3.1	Third-Party Call with DC capability establishment request

Table 8.4.2.3.1-1 describes information elements for the Third-Party Call with DC capability establishment request from the Application Server to the MMTel Enabler server.

Table 8.3.2.3.1-1: Information elements in Third-Party Call with DC capability establishment request

| Information element | Status | Description | Related OMA API Element | Related OSA API Element |
| --- | --- | --- | --- | --- |
| Originating ID (see NOTE) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7] | The targetAddress in the first routeReq() method as specified in 3GPP TS 29.198-04 [291984-2] |
| Terminating ID (see NOTE) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7] | The targetAddress in the second routeReq() method as specified in 3GPP TS 29.198-04 [291984-2] |
| Media information | M | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7] | The TpMediaStreamRequest in the media stream in mediaStreamAllow() method as specified in 3GPP TS 29.198-04 [291984-4] |
| DC media information | O | Identifier of whether DC media is expected to be used in this call | N/A | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call | N/A | N/A |
| Notification information | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. | The callbackReference of the CallSessionInformation specified in OMA Third Party Call API [7] | The appInterface in the setCallback() method as specified in 3GPP TS 29.198-03 [291983] |
| NOTE: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain,. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |  |

Editor's note:	Whether more information elements are needed to be  included in the request/response are FFS and needs to be aligned with the parameters in the existing OMA APIs.

8.4.2.3.2	Third-Party Call with DC capability establishment response

Table 8.4.2.3.2-1 describes information elements for the Third-Party Call with DC capability establishment response from the Application Server to the MMTel Enabler server.

Table 8.3.2.3.2-1: Information elements in Third-Party Call with DC capability establishment response

| Information element | Status | Description | Related OMA API Element | Related OSA API Element |
| --- | --- | --- | --- | --- |
| Call establishment result | M | Indication if the Call establishment is success or failure. | The HTTP response codes of the created call session as specified in OMA REST_NetAPI_Common [OMAComm] | If expected returns is received and no exception is raised, the method is considered as success, otherwise is considered as failure. |
| Session ID (see NOTE 1) | O | The identifier of the call session. | The callSessionId as specified in OMA Third Party Call API [7] | The CallSessionID in the returned TpCallIdentifier of createCall() method as specified in 3GPP TS 29.198-04 [291984-2] |
| Failure Cause (see NOTE 2) | O | The reason for failure | The Policy Exception and Service Exception fault as specified in OMA Third Party Call API [7] | The TpCommonExceptions raised in the createCall() method as specified in 3GPP TS 29.198-04 [291984-2] |
| Originating ID (see NOTE 3) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7] | The targetAddress in the first routeReq() method as specified in 3GPP TS 29.198-04 [291984-2] |
| Terminating ID (see NOTE 3) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7] | The targetAddress in the second routeReq() method as specified in 3GPP TS 29.198-04 [291984-2] |
| Media information | M | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7] | The TpMediaStreamRequest in the media stream in mediaStreamAllow() method as specified in 3GPP TS 29.198-04 [291984-4] |
| DC media information | O | Identifier of whether DC media is expected to be used in this call | N/A | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call | N/A | N/A |
| NOTE 1: This IE shall only be present when the Call establishment result is Success. The callSessionId in OMA API and OSA API are generated by the entity implementing the APIs. In this case, the MMTel Enabler Server handles the call session ID obtained from the related APIs. In some implementation, the IMS entity which sets up calls to both call parties can act as the B2BUA as specified in RFC 7989 [7989] and obtain the End-to-End Session id of the IMS Session as defined in RFC 7989 [7989]. The IMS entity can notify this IMS Session ID to the entity implementing the OMA or OSA APIs, and then notifies to the MMTel Enabler Server. NOTE 2: This IE shall only be present when the Call establishment result is Failure. NOTE 3: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain,. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |  |