3GPP TSG-SA WG6 Meeting #65	S6-250150

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	China Mobile

Title:	pCR on update the reference points

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR is proposed to update the reference points.

2. Reason for Change

This pCR is proposed to update the reference points.

4. Proposal

This pCR is proposed to update the reference points.

* * * First Change * * * *

6.4	Reference points

6.4.1	General

The reference points for MMTel Enabler are described in the following subclauses.

6.4.2	MMTel-1 (between the MMTel Enabler client and the MMTel Enabler server)

The MMTel-1 reference point supports the interactions between a MMTel Enabler client and the corresponding MMTel Enabler server. This reference point supports:

-	the MMTel Enabler client to download the DC application profile from MMTel Enabler Server;

-	the MMTel Enabler server notifies the MMTel Enabler client in the UE to send application request messages to the MMTel Enabler server to request the profiles of data channel applications; and

-	the MMTel Enabler server sends the profiles to the UE directly after the UE subscribes to the profile notification

6.4.3	MMTel-2 (between the MMTel Enabler server and the Application Server)

The interactions between the Application Server and the underlying network are provided by the MMTel Enabler server via MMTel-2 reference point to ensure the security access of the underlying network. This reference point supports:

-	the Application Server to discover MMTel service APIs, to authenticate, to get authorization and to communicate with the MMTel service; and

-	to provide the MMTel calling API with Data Channel capability to the Applications to use the MMTel service.

6.4.4	MMTel-3 (between the MMTel Enabler server and the Controlling Application Server)

The interactions between the MMTel Enabler server and the Controlling Application Server are referred to MMTel-3 reference point. This reference point supports:

-	the Controlling Application Server to manage, e.g. configure, update, delete, the DC application and profile on DCAR via MMTel Enabler server; and

-	the Controlling Application Server to get the DC application profile information from the MMTel Enabler Server.

6.4.5	MMTel-4 (between the MMTel Enabler client and the DCMTSI client)

The interaction between MMTel Enabler Client and DCMTSI Client.are referred to MMTel-4 reference point. This reference point is internal interaction in the dialler application on the UE and implementation specific.

NOTE:	The functionalities of the MMTel-4 reference point is out of scope of the present document.

6.4.6	App-1 (between the Application Client and the Application Server)

The App-1reference point supports the communication between Application Client and Application Server to provide the Data Channel Application specific logic.

NOTE:	The functionalities of the App-1 reference point is out of scope of the present document.

6.4.1	SEAL-C

The following SEAL-C reference points for MMTel enablement service are supported:

-	NM-C reference point for notification management as specified in 3GPP TS 23.434 [6];

6.4.2	SEAL-S

The following SEAL-S reference points for MMTel enablement service are supported:

-	NM-S reference point for notification management as specified in 3GPP TS 23.434 [6];

6.4.3	SEAL-UU

The following SEAL-UU reference points for MMTel enablement service are supported:

-	NM-UU reference point for notification management as specified in 3GPP TS 23.434 [6];