3GPP TSG-SA WG6 Meeting #65	S6-250152

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0219 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove EN in clause 8.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EN in clause 8.1.4: Editor's note: Whether the enhancement is addressed by VAL UE configuration request and response are FFS. Is discussed in SEAL_Ph2 in S6-231026 and the conclusion is that there is no such requirement for other VAL services. Therefore this feature will not be added to SEAL configuration service now and the EN in MSGin5G can be removed without more action. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to remove EN in clause 8.1.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary EN exists in the text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.1.4	MSGin5G UE bulk configuration over MSGin5G-6 reference point

When more than one MSGin5G UEs, which are constrained devices, which support an MSGin5G Client, get the MSGin5G Service configuration information via the MSGin5G Gateway UE, the MSGin5G Gateway UE may decide to use bulk configuration procedure specified in this clause based on service policy.

Pre-conditions:

1.	The constrained MSGin5G UE has discovered and selected an MSGin5G Gateway UE as specified in clause 8.2.8 and connected to the serving network via the MSGin5G Gateway UE successfully.

2.	The MSGin5G Gateway UE has been configured by using the procedure specified in clause 8.1.2 of the present document.

The constrained MSGin5G UE sends the Get VAL UE configuration request as specified in clause 8.1.2 of the present document to MSGin5G Gateway UE. If the MSGin5G Gateway UE decides to use bulk configuration based on the service policy, the MSGin5G Gateway UE sends a Get VAL UE configuration response including the information elements as listed in table 8.1.4-1 as a part of VAL UE configuration data to the constrained MSGin5G UEs before bulk configuration is performed. This Get VAL UE configuration response is used to inform each constrained MSGin5G UE that their Get VAL UE configuration requests will be handled later.

NOTE:	How to decide that bulk configuration is used is implementation specific and out of scope.

Table 8.1.4-1: Information in the Get VAL UE configuration response to constrained device before bulk configuration is performed

| Information element | Status | Description |
| --- | --- | --- |
| Maximum configuration time | M | The maximum wait time for the bulk configuration request to be sent to the MSGin5G Server. |

In the MSGin5G UE bulk configuration procedure, the MSGin5G Gateway UE acts as Configuration management client specified in clause 8.1.2 of the present document. In addition to the information elements listed in table 8.1.2-1 of the present document and clause 11.3.2.1 of 3GPP TS 23.434 [5], the Get VAL UE configuration request used for bulk configuration also includes the information elements as listed in table 8.1.4-2.

Table 8.1.4-2: additional Information in the Get VAL UE configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Bulk configuration flag | M | Indicates this request is an enhanced Get VAL UE configuration request used for MSGin5G UE bulk configuration |
| List of MSGin5G UE IDs | M | List of MSGin5G UE ID of MSGin5G UEs which needed to be configured in this bulk configuration request |

In addition to the information elements listed in table 8.1.2-2 of the present document, the Get VAL UE configuration response used for bulk configuration also includes the information elements as listed in table 8.1.4-3 as a part of VAL UE configuration data as specified in clause 11.3.2.2 of 3GPP TS 23.434 [5].

Table 8.1.4-3: additional Information in the Get VAL UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| List of MSGin5G UE configuration information | M | Each element in this list contains information as specified in Table 8.1.2-2. The configuration information included in this IE is used to complete the configuration of the constrained MSGin5G UEs. |

Upon receiving the Get VAL UE configuration response used for bulk configuration, the MSGin5G Gateway UE includes the MSGin5G UE configuration information in the List of MSGin5G UE configuration information into multiple individual Get VAL UE configuration responses and sends them to the corresponding constrained MSGin5G UEs.