3GPP TSG-SA WG6 Meeting #65	S6-250154

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0220 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove EN in clause 8.7.5.2 and 8.8.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EN in clause 8.7.5.2: Editor's Note: TS 33.501 does not specify how the connection is secured. The EN in clause 8.8.4.2: Editor's Note: How a secure connection between two MSGin5G Servers is to be established is FFS. The connection between MSGin5G Servers in different PLMNs can be considered as connection between NFs in different PLMNs. Therefore the Network Domain Security (NDS); IP network layer security specified in TS33.210 can apply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to remove EN in clause 8.7.5.2 and 8.8.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary EN exists in the text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.7.5.2, 8.8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.262: "Message Service within the 5G System".

[3]	GSMA PRD RCC.07: "RCC.07 Rich Communication Suite 9.0 Advanced Communications Services and Client Specification".

[4]	OMA OMA-ERELD-LightweightM2M-V1_1-20180612-C: "Enabler Release Definition for LightweightM2M".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[9]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[10]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[11]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[12]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[13]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[14]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[15]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[16]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[17]	3GPP TS 32.240: "Charging architecture and principles".

[18]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[19]	3GPP TS 23.303: "Proximity-based services (ProSe)".

[20]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[21]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[22]	3GPP TS33.210: "Network Domain Security (NDS); IP network layer security".

* * * Next Change * * * *

8.7.5.2	Inter-PLMN message exchange procedure

Pre-condition:

1.	The Message Sender (e.g. MSGin5G Client 1 in MSGin5G UE 1) is registered to the MSGin5G Server 1 in one PLMN.

2.	The Message Receiver (e.g. MSGin5G Client 2 in MSGin5G UE 2) is registered to the MSGin5G Server 2 in another PLMN.

3.	MSGin5G Server 1 and MSGin5G Server 2 have established a secured connection based on NDS/IP specified in 3GPP TS 33.210[22].

Figure 8.7.5.2-1 shows message delivery between MSGin5G endpoints in different PLMNs, where the Message Sender is registered in MSGin5G Server 1 and the Message Receiver is registered in MSGin5G Server 2.

Figure 8.7.5.2-1: Message delivery between MSGin5G UEs in different PLMNs

1.	The Message Sender sends an MSGin5G message request to MSGin5G Server 1 in PLMN 1 as specified in clause 8.3.2.

2.	The MSGin5G Server 1 analyses the target UE Service ID and determines that the message is targeted to the Message Receiver in PLMN 2, authenticates that the Message Sender is allowed to send a message to the Message Receiver, and then the MSGin5G Server 1 forwards the MSGin5G message request to MSGin5G Server 2 in PLMN 2. The MSGin5G message request contains the Information Elements as specified in table 8.3.3-1.

3.	MSGin5G Server 2 forwards the MSGin5G message request to the Message Receiver as specified in clause 8.3.3.

4-6.	If the message delivery status report is requested, the Message Receiver sends a message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.

* * * Next Change * * * *

8.8.4.2	Messaging Topic list subscription

Before the subscribing of Messaging Topic(s), the MSGin5G Server 1 should obtain the available Messaging Topic list on the MSGin5G Server 2 to determine whether to forward the Messaging Topic subscription request to MSGin5G Server 2, to subscribe to the Messaging Topic on behalf of all MSGin5G UE/Application Server served by it on MSGin5G Server 2, or to create the subscription on Server 1 as specified in clause 8.8.1 if the subscription does not exist on Server 2.

Figure 8.8.4.2-1 shows the MSGin5G Server 1 subscribing to Messaging Topic list on the MSGin5G Server 2.

NOTE 1:	If the MSGin5G Server 1 and MSGin5G Server 2 are located in the same PLMN, the synchronization of Messaging Topic list between MSGin5G Servers may also be implementation specific.

Pre-conditions:

1.	MSGin5G Server 1 and MSGin5G Server 2 have established a secured connection based on NDS/IP specified in 3GPP TS 33.210[22].

Figure 8.8.4.2-1: MSGin5G Server 1 subscribes to Messaging Topic list on the MSGin5G Server 2

1.	The MSGin5G Server 1 sends a Messaging Topic list subscription request to the MSGin5G Server 2. The request includes the information elements listed in Table 8.8.4.2-1.

Table 8.8.4.2-1: Messaging Topic list subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Originating MSGin5G Server ID | M | The MSGin5G Server which requests the Messaging Topic list. |
| Recipient MSGin5G Server ID | M | The MSGin5G Server which holds the Messaging Topic list. |
| Security credentials | O | Security information required by the MSGin5G Server 2 and is left for implementation. |
| Expiration | O | The date and time when the subscription expires. If this IE is included, the value of it should be larger than 0. If this IE is not included, the expiration time is subject to operator policy. |

2.	Upon receiving the Messaging Topic list subscription request, the MSGin5G Server 2 validates this request and may verify the security credentials.

3.	The MSGin5G Server 2 checks the locally stored Messaging Topic list subscription(s).

a)	If the MSGin5G Server 1's subscription has already been created, the MSGin5G Server 2 updates the validity time of this subscription.

b)	If the MSGin5G Server 1's subscription has not been created, the MSGin5G Server 2 creates the subscription.

4.	The MSGin5G Server 2 sends a Messaging Topic list Subscription response to MSGin5G Server 1. The response includes the information listed in Table 8.8.4.2-2.

Table 8.8.4.2-2: Messaging Topic list Subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | Indicates whether the subscription was successfully added or deleted on the MSGin5G Server 2. |
| Expiration | O | The validity date and time of this Messaging Topic list subscription set by the MSGin5G Server 2. |

5.	The MSGin5G Server 2 checks whether Messaging Topic list notification is needed, e.g. whether the MSGin5G Server 1 subscribes the Messaging Topic list on MSGin5G Server 2 for the first time, or the local Messaging Topic(s) on the MSGin5G Server 2 are updated, e.g. new Messaging Topic(s) has been created or existing Messaging Topic(s) has been deleted.

NOTE 2:	If the MSGin5G Server 1 has previously unsubscribed the Messaging Topic list on MSGin5G Server 2, the MSGin5G Server 2 should consider that the MSGin5G Server 1 subscribes the Messaging Topic list on MSGin5G Server 2 for the first time when the MSGin5G Server 1 subscribes the Messaging Topic list again.

6.	If Messaging Topic list notification is needed, the MSGin5G Server 2 sends a Messaging Topic list notification to MSGin5G Server 1. The notification includes the information listed in Table 8.8.4.2-3.

Table 8.8.4.2-3: Messaging Topic list notification

| Information element | Status | Description |
| --- | --- | --- |
| Expiration | O | The new validity date and time of this subscription set by the MSGin5G Server 2. |
| >Messaging Topic list | M | A list of Messaging Topic(s) that exists on the MSGin5G Server 2. If the MSGin5G Server 1 subscribes the Messaging Topic list on MSGin5G Server 2 for the first time, the MSGin5G Server 2 should include all Messaging Topic(s) that exist on the MSGin5G Server 2 in this Messaging Topic list, else the MSGin5G Server 2 includes the deviation of Messaging Topic(s) since the last notification, Each element in this list contains information as specified in Table 8.8.4.2-4. Based on service policy, the MSGin5G Server 2 may only include a part of Messaging Topic(s) in the notification which are allowed to be subscribed by MSGin5G Server 1. |

Table 8.8.4.2-4: Individual Messaging Topic

| Information element | Status | Description |
| --- | --- | --- |
| Messaging Topic | M | Unique identifier of this Messaging Topic. |
| Update status | M | Identifies the Messaging Topic is newly created on the MSGin5G Server 2, or newly deleted on the MSGin5G Server 2 |

7.	Upon receiving the Messaging Topic list notification, the MSGin5G Server 1 updates the locally stored Messaging Topic list:

a)	if the Update status of a Messaging Topic is Created, the MSGin5G Server 1 adds the Messaging Topic to the locally stored Messaging Topic list; and

b)	if the Update status of a Messaging Topic is Deleted,

i)	if the Messaging Topic exists on MSGin5G Server 1, the MSGin5G Server 1 removes the Messaging Topic from the locally stored Messaging Topic list; and

ii)	if the Messaging Topic does not exist on MSGin5G Server 1, the MSGin5G Server 1 ignores this Messaging Topic update.

NOTE 3:	The MSGin5G Server should not send Messaging Topic list notification to other MSGin5G Servers if its locally stored Messaging Topic list is updated by receiving a Messaging Topic list notification.