3GPP TSG-SA WG6 Meeting #65	S6-250164

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on business relationship involving SEAL client

Spec:	3GPP TR 23.700-35 v0.2.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yanmei Yang yangyanmei@huawei.com

- Introduction

This contribution is to propose to provide the solution for the Gap of Incomplete business relationship for SEAL service.

2. Reason for Change

As it is mentioned in the Gap#5, the existing business relationship in the specification TS 23.434 is not completed to reflect all the potential deployments case, especially it does not involve all the stakeholder e.g., the SEAL client providers.

This contribution is proposed to add the related descirption.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v0.2.0.

* * * First Change * * * *

8.X	Solution: business relationship involving SEAL Client provider

8.x.1	Solution description

VAL service provider may get SEAL services by establishing the service agreement with SEAL server service provider directly as figure 8.x.1-1, or indirectly based on the service agreement between VAL service provider and SEAL client service provider, and SEAL client service provider and SEAL server service provider as figure 8.x.1-2.

As figure 8.x.1-1 shows, some SEAL clients are required to provide terminal side information/resource to support some functions of SEAL service serve. Thus, SEAL server services provider may need to implement SEAL clients, to enable the SEAL server services business toward to VAL service provider.

As figure 8.x.1-2 shows, SEAL client may support the function of invoking the API from SEAL service server, then re-exposing them to Application Clients as the local API or SDK service. In this case, VAL service provider who own Application Clients needs to setup service agreement with SEAL client service provider and SEAL client provider should also setup service agreement with SEAL server service provider. Some SEAL client may also provide function based on UE model capabilities, which may potential requires the PLMN business agreement between SEAL client and PLMN operator.

Figure 8.x.1-1: Business relationships for VAL services (SEAL client provided by SEAL server provider)

Figure 8.x.1-2: Business relationships for VAL services

The VAL user belongs to a VAL service provider based on a VAL service agreement between the VAL user and the VAL service provider. The VAL service provider can have VAL service agreements with several VAL users. The VAL user can have VAL service agreements with several VAL service providers.

The SEAL service provider and the home PLMN operator can be part of the same organization, in which case the business relationship between the two is internal to a single organization.

The VAL service provider can have SEAL provider arrangements with multiple SEAL providers and the SEAL provider can have PLMN operator service arrangements with multiple home PLMN operators. The SEAL provider and the VAL service provider or the home PLMN operator can be part of the same organization, in which case the business relationship between the two is internal to a single organization.

The home PLMN operator can have PLMN operator service arrangements with multiple VAL service providers and the VAL service provider can have PLMN operator service arrangements with multiple home PLMN operators. As part of the PLMN operator service arrangement between the VAL service provider and the home PLMN operator, PLMN subscription arrangements can be provided which allows the VAL UEs to register with home PLMN operator network.

The home PLMN operator can have PLMN roaming agreements with multiple visited PLMN operators and the visited PLMN operator can have PLMN roaming agreements with multiple home PLMN operators.

8.x.2	Evaluation

* * * Next Change * * * *

5.5	Generic Gap#5: Incomplete business relationship for SEAL service

5.5.1	Analysis

In 3GPP TS 23.434 [23.434] clause 5 describes the business relationship between different stakeholders. In clause 5 the SEAL service provider is quite not clear and complete (e.g., the role of seal service provider, seal clients/server provider). For example, the existing business relationship in the specification does not involve all the stakeholder e.g., the SEAL client providers.

The SEAL client may be potentially provided by different software providers, e.g., OS, 3rd party middleware provider, application provider, or substituted by other alternatives to serve different business relationship.

5.5.2	Identified gaps

Gap#5-5-2-1: Identify the further enhancements (like role of seal client provider) for the seal service provider described in clause 5 based on the different seal service deployment options in the context of the overall SEAL business relationship.

* * * End of  Change * * * *