3GPP TSG-SA WG6 Meeting #64	S6-250185

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245372)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0624 | rev | 2 | Current version: | 19.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC gateway UE fixes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Currently, there is no peer application functional entity in MC gateway UE to handle the signaling from MC client as described in clause 11.5, i.e., the location management and MBMS management related messages. The functional model should be updated. - There is no “GW MC service ID” defined in the TS but used in clause 11.5. The MC service ID is used to identify a particular MC service user. The wrong Identity should be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - the functional model is updated with a new functional entity in the gateway UE and two reference points between the Gateway UE and the clients in non-3GPP device. - the identity used in clause 11.5.2 and 11.5.3 was corrected to MC service ID of the MC service user using the MC service client hosted within the non-3GPP device - the procedures were updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect functional model and identifier usage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1, 11.2.2, (new) 11.2.2.aa, (new) 11.2.2.bb, (new) 11.2.4, (new) 11.2.4.1, (new) 11.2.4.2, 11.5.2.2.1, 11.5.2.2.2, 11.5.2.2.3, 11.5.2.3, 11.5.2.3.1, 11.5.2.3.2, 11.5.2.3.3, 11.5.3.2.1, 11.5.3.2.2, 11.5.3.2.3, 11.5.3.2.4, 11.5.3.2.5, 11.5.3.2.6, 11.5.3.2.7, 11.5.3.2.8, 11.5.3.3, 11.5.3.3.1, 11.5.3.3.2, 11.5.3.3.3, 11.5.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ 1st changes *************************/

### 11.2.1	Functional model

The authentication and authorisation of the MC clients hosted on the MC gateway UE follows the procedures described in 3GPP TS 33.180 [25]. Upon successful authentication and authorisation the MC clients can access a MC server via an MC gateway UE.

The MC gateway UE supports the authentication and authorisation of the MC client residing at the non-3GPP device by forwarding the unmodified application layer signalling between the MC client and the corresponding MC server. The authentication and authorisation of the MC client in the non-3GPP device follows the procedures described in 3GPP TS 33.180 [25].

NOTE 1:	The authentication and authorisation between the MC gateway UE and the non-3GPP device is out of scope of 3GPP.

Figure 11.2.1-1 represents the functional model for the application plane for utilizing MC gateway UE to enable MC services to non-3GPP devices that can host an MC client.

Figure 11.2.1-2 represents the functional model for the signalling plane for utilizing MC gateway UE to enable MC services to non-3GPP devices that can host an MC client.

Figure 11.2.1-3 represents the functional model for the application plane for non-3GPP devices that cannot host an MC client.

Figure 11.2.1-1: Functional model of MC gateway UE application plane with MC client hosted in the non- 3GPP device

Figure 11.2.1-2: Functional model of MC gateway UE signalling plane with MC client hosted in the non- 3GPP device

Figure 11.2.1-3: Functional model of MC gateway UE application plane with MC client hosted in the MC gateway UE

The functional model for the signalling control plane as described in figure 7.3.1-2 is applicable to figure 11.2.1-3.

# /************************ Next changes *************************/

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1]. Not all definitions are used in this document.

Accuracy: Reflects the uncertainty of the location at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

ACM: Administrative Configuration Management, which enables the exchange of administrative configuration data between interconnected MC systems.

ACMC: Administrative Configuration Management Client, client entity which initiates administrative configuration exchange request to an interconnected partner MC system and which could make decision on such request received from an ACMC of a partner MC system.

ACMS: Administrative Configuration Management Server, server entity which receives administrative configuration exchange requests from an ACMC belonging to the same primary MC system or via an ACMS of an interconnected partner MC system.

Active MC service user profile: The MC service user profile that is currently used by an MC service client of an MC service user while receiving MC service.

Ad hoc Group Communication: The combining of a multiplicity of MC service users into a group for the duration of a communication. When the communication is released, the group no longer exists. If the communication is associated with an alert, then the group continues to exist until the alert is also cancelled.

Ad hoc Group emergency alert: The combining of a multiplicity of MC service users into a group for sending an emergency alert. When the alert is cancelled, the group no longer exists. If the alert is associated with a communication, then the group continues to exist until the communication is also cancelled.

Altitude: Third dimension for the geographical coordinates at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Bearing: Direction at the moment of location measurement, e.g. see 3GPP TS 25.305 [30].

Chat group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Group members must join the pre-established group call to participate.

ECGI: E-UTRAN Cell Global Identifier, which is used to identify cells globally, where the ECGI is constructed from the Mobile Country Code (MCC), Mobile Network Code (MNC) and the E-UTRAN Cell Identifier (ECI).

Interconnection: A means of communication between MC systems whereby MC service users obtaining MC service from one MC system can communicate with MC service users who are obtaining MC service from one or more other MC systems.

Interconnection group: An MC service group that is configured to allow inclusion of MC service group members who are MC service users from partner MC system(s).

LCS network: The 3GPP network that provides location service as defined in 3GPP TS 23.271 [29].

Location: The current physical location of the MC service UE.

MBMS SAI: Multimedia Broadcast Multicast Service Area Identity which is mapped to the MBMS service area.

MC gateway server: A server providing topology hiding for MC service interconnection with a partner MC system, where that partner MC system is in a different trust domain.

MC service: A generic name for any one of the three mission critical services: either MCPTT, or MCVideo, or MCData.

MC service affiliated group member: An MC service user who has indicated an interest in a particular MC service group and has been accepted to participate in MC service group communication for that MC service group.

MC service client: A generic name for the client application function of a specific MC service. MC service client could be replaced by MCPTT client, or MCVideo client, or MCData client depending on the context.

MC service group: A defined set of MC service users with associated communication dispositions (e.g. media restrictions, default priority and commencement directions) configured for the use with one or more MC services.

MC service group affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is determined.

MC service group call: A mechanism by which an MC service user can make a one-to-many MC service(s) transmission to other users that are members of MC service group(s).

MC service group de-affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is removed.

MC service group home system: The MC system where the MC service group is defined.

MC service group host MC service server: The MC service server within an MC system which provides centralised support for a particular MC service of an MC service group defined in a MC service group home system.

MC service group member: An MC service user, whose MC service ID is listed in a particular MC service group.

MC service ID: A generic name for the user ID of a mission critical user within a specific MC service. MC service ID could be replaced by MCPTT ID, MCVideo ID, or MCData ID depending on the context.

MC service server: A generic name for the server application function of a specific MC service. MC service server could be replaced by MCPTT server, MCVideo server, or MCData server depending on the context.

MC service user: An authorized user, who can use an MC service UE to participate in one or more MC services.

MC service user profile: The set of information associated to an MC service user that allows that user to employ one or more MC services in a given role and from a given MC service UE.

MC service UE: A UE that can be used to participate in one or more MC services and recording admin and replay services.

MC service UE label: A generic name for identification of a specific MC service UE.

MC system: The collection of applications, services, and enabling capabilities required to provide a single mission critical service or multiple mission critical services to one or more mission critical organizations.

MC user: A user, identified by an MC ID, who, after authorization, obtains mission critical service(s).

Migration: A means for an MC Service user to obtain MC service directly from a partner MC system.

Partner MC system: Allied MC system that provides MC services to an MC service user based on the MC service user profiles that are defined in the primary MC system of that MC service user.

Preconfigured MC service group: an MC service group used only for regrouping that has been configured in advance of a group or user regrouping operation to serve as the source of regroup group configuration.

Pre-arranged group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Affiliated group members are invited when the group communication is setup.

Pre-selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile through configuration, and applicable for an authenticated MC service user upon MC service authorization.

Primary MC system: MC system where the MC service user profiles of an MC service user are defined.

Recording admin UE: A UE that can be used to configure targets for recording. Recording admin UE can be a mobile device or a (computer) workstation.

Recording admin and replay service user profile: A set of parameters defining the authorizations for a recording administrator and/or replay user.

Recording administrator: A user, identified by MCRec ID, who, after authorization, is able to set and modify target users and target groups for recording.

Recording admin client: A client application that a recording administrator can use to set target users and target groups for recording.

Recording server: A server that is able to log the metadata and record the media of MCPTT, MCData, and MCVideo group communications and private communications. This server is also able to retrieve the logged/recorded data when requested by an authorized replay user.

Replay user: A user, identified by an MCRec ID, who, after authorization, is able to fetch and replay recorded MCPTT, MCData, and MCVideo metadata and media of MC service group communications and MC service private communications under the user’s authority.

Replay UE: A user equipment that can be used to fetch and replay recorded MCPTT, MCData, and MCVideo metadata and media of MC service group communications and MC service private communications under the UE user’s authority. The Replay UE can be a mobile device or a (computer) workstation.

Replay client: A client application that an authorized replay user can use to fetch and replay recorded metadata and media from a recording server.

Requested Priority: A value for use in a MC service group or MC private communication that, if accepted, is used by the MCX service server to temporarily replace the priority level that is predefined in the MC service group or MC service user profile. This value is used in combination with other factors to determine the application priority for the requested communication.

Selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile for an MC service upon request by an MC service user.

Serving MC service server: The MC service server which is providing MC service to an MC service client.

NOTE 1:	There is one serving MC service server for each MC service, which can be the primary MC service server of the MC service user of the MC service client, or can be a partner MC service server to which the MC service user has migrated.

Serving MC system: The MC system which is providing MC service to an MC user.

NOTE 2:	The MC system can be the primary MC system of the MC service user, or can be a partner MC system to which the MC service user has migrated.

Speed: Movement at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Time of measurement: Date and time expressed with a certain precision to reflect the moment of the location measurement.

For the purposes of the present document, the following terms given in 3GPP TS 22.280 [3] apply

Mission Critical

Mission Critical Applications

Mission Critical Organization

Mission Critical Service

Functional alias

For the purposes of the present document, the following terms given in 3GPP TS 22.179 [2] apply

Multi-talker control

Group-broadcast group

For the purposes of the present document, the following terms related to a MC gateway UE function apply

MC gateway UE:  A UE with functionality that enables access to the MC service for non-3GPP devices.

MC client: Aggregates a set of clients (i.e. Group management client, Configuration management client, Identity management client, Key management client, Location management client and MC service client).

MC server: Aggregates a set of servers (i.e. Group management server, Configuration management server, Identity management server, Key management server, Location management server and MC service server) which serves the MC client accordingly.

Non-3GPP device: A device that enables connectivity towards an MC gateway UE using an access method not specified by 3GPP. A subset of these devices can host an MC client specified by 3GPP.

GW Client: Functional entity residing on the non-3GPP UE for supporting the location management and MBMS transmission for the MC client.

GW Server: Functional entity residing on the MC gateway UE for supporting the location management and MBMS transmission for the MC clients residing on the non-3GPP UE.

# /************************ Next changes *************************/

### 11.2.2	Reference points

#### 11.2.2.aa	Reference points GW-local (between the GW client and the GW server)

The reference points GW-local is between the GW Client and GW Server. It supports the following functionalities:

- MBMS related signalling exchange (e.g., MBMS listening status report, MBMS announcement) to enable the MC gateway UE to receive the MBMS data for the MC service user using the non-3GPP device.

- Location management related signalling exchange (e.g., location configuration trigger, location report) to enable the MC client residing on the non 3GPP device to handle the 3GPP network related location information and triggers.

# /************************ Next changes *************************/

##### 11.5.2.2.1	MC GW location reporting configuration

Table 11.5.2.2.1-1 describes the information flow from the MC client, which resides on a non-3GPP device, to the MC gateway UE for the location reporting configuration.

Table 11.5.2.2.1-1: MC GW location reporting configuration

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report (see NOTE 2) |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent |
| NOTE 1: If none of the information elements is present, this represents a cancellation for location reporting, if configured. NOTE 2: The triggering criteria contains only the events related to the 3GPP access network. |  |  |

##### 11.5.2.2.2	MC GW location information report

Table 11.5.2.2.2-1 describes the information flow from the MC gateway UE to the MC client residing on a non-3GPP device for the location information reporting.

Table 11.5.2.2.2-1: MC GW location information report

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| Triggering event | O | Identity of the event that triggered the sending of the report |
| Location information (see NOTE) | O | Location information of the MC gateway UE |
| NOTE: The following location information elements which are related to 3GPP access network shall be present (configurable): Serving and neighbouring ECGI, MBMS SAIs, MBMSfnArea, PLMN ID. |  |  |

##### 11.5.2.2.3	MC GW location information request

Table 11.5.2.2.3-1 describes the information flow from the MC client residing on a non-3GPP device to the MC gateway UE for requesting an immediate location information report.

Table 11.5.2.2.3-1: MC GW location information request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |

# /************************ Next changes *************************/

##### 11.5.3.2.1	MC GW MBMS bearer announcement

Table 11.5.3.2.1-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of MBMS bearer announcement received by the MC Client from the MC Service server.

Table 11.5.3.2.1-1: MC GW MBMS bearer announcement

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC client. |
| TMGI | M | TMGI information |
| List of service area identifier | M | A list of service area identifier for the applicable MBMS broadcast area. |
| Frequency | O | Identification of frequency if multi carrier support is provided |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this bearer (e.g. codec, protocol id, FEC information) |
| Monitoring state | O | The monitoring state is used to control if the client is actively monitoring the MBMS bearer quality or not. |
| ROHC information | O | Indicate the usage of ROHC and provide the parameters of the ROHC channel to signal to the ROHC decoder. |

# /************************ Next changes *************************/

##### 11.5.3.2.3	MC GW MapGroupToBearer request

Table 11.5.3.2.3-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of MapGroupToBearer message received from the MC service server.

Table 11.5.3.2.3-1: MC GW MapGroupToBearer request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user. |
| MCPTT group ID | M | This element identifies the MCPTT group, in which the call is started. |
| Media stream identifier | M | This element identifies the media stream of the SDP used for the group call (e.g. MBMS subchannel). |
| TMGI | M | The MBMS bearer identifier. |

# /************************ Next changes *************************/

##### 11.5.3.2.7	MC GW UnMapGroupToBearer request

Table 11.5.3.2.7-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of UnMapGroupToBearer message received from the MC service server.

Table 11.5.3.2.7-1: MC GW UnMapGroupToBearer request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user. |
| MCPTT group ID | M | This element identifies the MC service group related to a group call to be dissociated over the MBS session. |
| Media stream identifier | M | This element identifies the media stream of the SDP, which is no longer used for the group call within the MBMS session. |
| TMGI | M | The MBMS bearer identifier. |

# /************************ Next changes *************************/

##### 11.5.3.3.2	Procedure for handling MapGroupToBearer message

Whenever the MC client detects that traffic received from MC service server is MapGroupToBearer message and if the MC client participates in the group session or communication identified by the MapGroupToBearer message then it should inform the details contained in the MapGroupToBearer message to MC gateway UE. When the association of group call, MBMS bearer and the MC service ID of the MC client is known to the MC gateway UE, it can forward the traffic received over MBMS bearer accordingly.

Figure 11.5.3.3.2-1 illustrates the procedure for handling the MapGroupToBearer message by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.3.3.2-1: Handling of MapGroupToBearer message

1.	The MC service server sends a MapGroupToBearer message over a previously activated MBMS bearer to all users that will receive the call over an MBMS bearer. The MapGroupToBearer message includes association information between the group call and MBMS bearer. The MapGroupToBearer message includes MC service group ID and information about the media stream identifier of the activated MBMS bearer and may include the identifier (i.e. the TMGI) of the MBMS bearer broadcasting the call.

2.	If the MC client is participating in the MC group communication identified by the MapGroupToBearer message, it sends the details contained in the MapGroupToBearer message to the MC gateway UE through MC GW MapGroupToBearer request message.

3.	The MC gateway UE on receiving the MC GW MapGroupToBearer Request message from the MC client it maintains the association between the MC Service ID and the corresponding MBMS sub channel.

4.	The MC gateway UE sends the MC GW MapGroupToBearer response message to the MC client which contains the details of the non-3GPP transport resources related parameters. The MC gateway UE may choose to send the details of existing communication channel information as part of non-3GPP transport resources establishment parameters IE if existing transport resources can be reused.

5.	The MC client establishes the communication channel with the MC gateway UE based on the parameters received in step 4 to receive the MC service group communication data from the MC gateway UE, if these parameters are not referring to any of the already established transport resources. The MC Gateway UE forwards the MC service group communication data it received over the MBMS bearer from the MC service server to the MC client over this transport resources.

6.	The MC service server sends the downlink media for the group communication session over the MBMS bearer.

7.	The MC gateway UE checks which MC clients should receive the media of the MC group communication based on Step 3.

8.	The MC gateway UE forwards the downlink media to the intended MC clients over the transport resources established as in step 5.

##### 11.5.3.3.2A	Procedure for handling UnmapGroupFromBearer message

Whenever the MC client receives UnmapGroupFromBearer message and if the MC client participates in the group session or communication identified by the UnmapGroupFromBearer message then it should inform the details contained in the UnmapGroupFromBearer message to MC gateway UE. When the association of group call, MBMS bearer and the MC GW service ID of the MC client is known to the MC gateway UE, it can remove the association and stop forwarding of the traffic received over MBMS bearer accordingly.

Figure 11.5.3.3.2A-1 illustrates the procedure for handling the UnmapGroupFromBearer message by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.3.3.2A-1: Handling of UnmapGroupFromBearer message

1.	An MCPTT group call is ongoing; the MC gateway UE forwards the downlink media to the intended MC client over the transport resources established.

2.	MCPTT server has determined to disconnect the call over the MBMS bearer for the MC client.

3.	An UnmapGroupFromBearer message is sent by the MCPTT server to MC client (if in MBMS coverage area) on MBMS bearer(s).

4.	If the MC client is participating in the MC group communication identified by the UnmapGroupFromBearer message, it sends the details contained in the UnmapGroupFromBearer message to the MC gateway UE through MC GW UnMapGroupToBearer request message.

5.	The MC gateway UE on receiving the MC GW UnMapGroupToBearer Request message from the MC client it removes the association maintained between the MC Service ID and the corresponding MBMS sub channel.

6.	The MC gateway UE sends the MC GW MapGroupToBearer response message to the MC client.