3GPP TSG-SA WG6 Meeting #65	S6-250186

Athens, Greece, 17th – 21st February 2025	(revision of S6-250xyz)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0136 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mode of reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the LS reply to CT1, S6-245501, the “mode of reporting” was changed from “Regular or irregular” to “periodic or event triggered”. CR S6-245683 updated clause 9.2.3.10, however, it missed the correction in the procedure for SEALDD connection status report configuration. In addtion, in table 9.2.3.10 the mode of reporting is defined as optional, what leaves unspecified for the SEALDD client whether it should configure periodic or event triggered reporting method. It needs to be clarified that event triggered is the default reporting method. The reporting interval is not optional, but conditional to the periodic reporting method. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “Regular” and “irregular” in mode of reporting are changed to “periodical” and “event triggered”. The event triggered reporting method is defined as default one when no reporting method is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unclear description on “mode of reporting” remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.6, 9.2.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.2.2.6	SEALDD connection status procedure

Figure 9.2.2.6-1 illustrates the procedure for SEALDD connection status from the VAL server to the SEALDD server.

Figure 9.2.2.6-1: SEALDD connection status procedure

1.	The VAL server sends the SEALDD connection status subscribe request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE identity, and the SEALDD client connection status check periodicity under "SEALDD client connection status" event as described in Table 9.2.3.7-1. The VAL server can also include the Non-3GPP access measurement information(like list of WLAN SSIDs, location-based measurement) in the request message.

2.	The SEALDD server subscribes to the NEF UE reachability, Application Detection and Loss of connectivity events using the procedure defined in clause 4.15.3.2.3b TS 23.502 [6]. It can also use the NRM Event monitoring procedure defined in clause 14.3.6.2.2 TS 23.434 [4].

3.	The SEALDD server also sends a SEALDD client connection status reporting configuration request to the SEALDD client. The request message consists of SEALDD-UU flow ID and method of reporting., which includes the reporting interval when the mode of reporting is periodic, and optionally the SEALDD client connection status reporting priority. The SEALDD-UU flow ID identifies the application traffic flow for which the reporting notification is configured. The SEALDD client monitors the application using SEALDD-UU flow ID. The method of reporting is defined as periodic or event triggered. The periodic reporting method uses the reporting interval to send the notification. The event triggered reporting method sends the notification if the current connection status is different from the previous connection status or if the application state changes (like crash, close, stop).

The SEALDD server can include the Non-3GPP access measurement in the SEALDD client connection status reporting configuration request. The SEALDD server can use the Non-3GPP access measurement information received in step 1 or Non-3GPP access measurement information configured in the SEALDD server as policy by VAL server or configured as internal policy.

4.	The SEALDD client configures the reporting configuration and provides the response to the SEALDD server. If the Non-3GPP access measurement includes list of WLAN SSIDs, then the SEALDD client performs the signal strength measurement for the requested WLAN SSID. If the request contains location-based measurement, then the SEALDD client measures the signal strength of nearby WLAN SSIDs based on its location. The response also includes the list of WLAN SSIDs and their signal strength measurements.

5.	The SEALDD server sends the connection status subscription response to the VAL server.

6a-6b. The SEALDD server receives the notification for the UE connection status from the subscribed NEF, NRM. It can also receive a notification from the SEALDD client regarding the connection status. If the UE is in power saving mode, then the SEALDD client sends the connection status reporting notification with the status as sleeping to the SEALDD server and suspends the connection status periodic reporting.

7.	Based on the NRM, NEF event subscription response and SEALDD client connection status reporting notification message with "SEALDD client connection status" event as described in Table 9.2.3.9-1, the SEALDD server processes the responses and sends the SEALDD client connection status of unreachable or sleeping status notification to the VAL server.

NOTE:	If the VAL server is not aware of the connection status like VAL UE reachability and may continue sending the application traffic, results in packet losses and degradation of the QoS of the VAL UEs.

* * * Next Change * * * *

#### 9.2.3.10	SEALDD client connection status reporting configuration request

Table 9.2.3.10-1 describes the information flow from the SEALDD server to SEALDD client to configure the SEALDD client for connection status reporting.

Table 9.2.3.10-1: SEALDD client connection status reporting configuration request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD-UU Flow ID | M | Indicates the SEALDD-UU flow ID of the application for which the reporting is required. |
| Mode of reporting | O | Indicates the mode of reporting: periodic and/or event triggered. If omitted, the default value is event triggered. |
| > Reporting interval | O (See NOTE) | Indicates the reporting interval to report the notification. |
| SEALDD client connection status reporting priority | O | Indicates the priority of the requested SEALDD-UU flow ID for the reporting of the SEALDD client connection status. |
| Non-3GPP access measurement information | O | Indicates the Non-3GPP access measurement information for the SEALDD client to perform measurement. It contains list of WLAN SSIDs or location-based reporting and signal strengths (e.g., RSRP, RSSI, RSRQ, SINR) values for measurement. |
| NOTE: This IE shall be present if the mode of reporting is periodic |  |  |

* * * END of Change * * * *