3GPP TSG-SA WG6 Meeting #65	S6-250187

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0137 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to non-3GPP access measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Non-3GPP access (such as WLAN) does not use the signal strength measurement methods RSRP, RSRQ, and SINR, which are currently described as examples for this. These methods are applicable as signal strength mesurement for cellular networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 9.2.3.10, 9.2.3.12, 9.7.3.6, 9.7.3.8, and 9.10.3.1 are updated: - RSRP, RSRQ, and SINR are removed as examples of signal strength measurements for non-3GPP access (e.g., WLAN). - “Non-3GPP access measurement information reporting” IE in table 9.2.3.12 and “Non-3GPP access measurement information” IE in table 9.7.3.8 are extended with two subelements: “Measured non-3GPP access” and “Signal strength value(s)” - “Non-3GPP access measurement policy” IE in table 9.10.3.1 is moved to be a subelement of the “SEALDD policy” IE. RSRP, RSRQ, and SINR are also removed as examples of signal strength measurements for non-3GPP access (e.g., WLAN) in 9.7.2.3 and 9.9.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect non-3GPP access signal measurement methods are described as examples. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.10, 9.2.3.12, 9.7.2.3, 9.7.3.6, 9.7.3.8, 9.9.2.4, 9.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 9.2.3.10	SEALDD client connection status reporting configuration request

Table 9.2.3.10-1 describes the information flow from the SEALDD server to SEALDD client to configure the SEALDD client for connection status reporting.

Table 9.2.3.10-1: SEALDD client connection status reporting configuration request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD-UU Flow ID | M | Indicates the SEALDD-UU flow ID of the application for which the reporting is required. |
| Mode of reporting | O | Indicates the mode of reporting: periodic and/or event triggered. |
| > Reporting interval | O (See NOTE) | Indicates the reporting interval to report the notification. |
| SEALDD client connection status reporting priority | O | Indicates the priority of the requested SEALDD-UU flow ID for the reporting of the SEALDD client connection status. |
| Non-3GPP access measurement information | O | Indicates the Non-3GPP access measurement information for the SEALDD client to perform measurement. It contains list of WLAN SSIDs or location-based reporting and signal strengths (e.g., RSSI) values for measurement. |
| NOTE: This IE shall be present if the mode of reporting is periodic |  |  |

# *** Next Change ***

#### 9.2.3.12	SEALDD client connection status reporting notification

Table 9.2.3.12-1 describes the information flow from the SEALDD client to the SEALD server for sending the SEALDD client connection status reporting configuration response.

Table 9.2.3.12-1: SEALDD client connection status reporting notification

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client connection status | M | Indicates the status of VAL UEs/users like reachable, unreachable, or sleeping |
| Non-3GPP access measurement information reporting | O | Indicates the Non-3GPP access measurement information report. |
| > Measured non-3GPP access | O | List of measured non-3GPP access (e.g., WLAN SSID(s)/BSSID(s) or location information). |
| >> Signal strength value(s) | M | List of signal strength values (e.g., RSSI) for the measured non-3GPP access. |
| SEALDD data transmission connection access usage | M | Indicates which access (3GPP or Non-3GPP) is used for the SEALDD-UU data transmission |

# *** Next Change ***

#### 9.7.2.3	Data transmission quality measurement reported by SEALDD client

Figure 9.7.2.3-1 illustrate the procedure for SEALDD enabled data transmission quality measurement for VAL traffic. The SEALDD client receives transmission quality measurement requirement, decides to start VAL data transmission monitoring and generates measurement reports.

Figure 9.7.2.3-1: VAL data transmission quality measurement reported by SEALDD client

1.	An on-going regular data transmission connection is established according to clause 9.2.2.2.

The transmission quality measurement can be triggered by VAL server or VAL client, which is described in step 2 to step 5 and step 6, correspondingly.

The request may include the crossflow measurement information (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2/DL]}) associated in the same multi-modal service for crossflow RTT measurement.

The detailed procedures of Tethering link measurement and provisioning is defined in clause 9.12.2.3.

2.	The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition.

NOTE 1:	The spatial and/or temporal condition can be used by SEALDD client to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday.

3.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server responds to the VAL server.

4-5.	The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server. The SEALDD client, based on the received service quality guarantee policy including thresholds and action, can take corrective action as described in clause 9.7.2.3.

If crossflow RTT measurement requirement is received from the VAL server, the SEALDD server identifies the SEALDD connections for the involved VAL UEs and sends to the SEALDD client(s) with crossflow RTT measurement requirement including corresponding UL/DL flow information.

The SEALDD server sends the Non-3GPP access measurement information in the SEALDD transmission quality measurement subscription request to the SEALDD client.

6.	The VAL client triggers the SEALDD transmission quality measurement procedure to the SEALDD client, in order to collect the measurement report information.

7.	After SEALDD client determines to start measurement process, upon UL packet arrival, the SEALDD client initiates the UL packet delay measurement. The SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload for VAL data transmission quality monitoring) with local time T1 when the SEALDD client sends out the UL monitoring packet. The SEALDD client considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD client stops/suspends the transmission quality measurement.

For crossflow RTT measurement, the SEALDD client received UL flow information starts recording local time T1 when sending out the 1st UL packet matching the UL flow information to the SEALDD server. The T1 is sent in the encapsulated UL monitoring packet to the SEALDD server.

8.	The SEALDD server receives the UL monitoring packet, and records the local time T2.

9.	Similarly, the SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring in case there is no DL VAL traffic for DL packet delay monitoring) with local time T2 recorded in step 8 and local time T3 when the SEALDD server sends out the DL monitoring packet.

NOTE 2:	When the SEALDD server sends the dummy UL packet as monitoring response to the SEALDD client depends on SEALDD server implementation.

For crossflow RTT measurement, when the 1st DL packet matching the received DL flow information is to be sent, the SEALDD server encapsulates the DL monitoring packet with previously received T1 and sends the DL monitoring packet to the SEALDD client.

10.	The SEALDD client records the local time T4 when the SEALDD client receives the DL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD client can also calculate the bitrate and jitter over a certain period over a specific SEALDD connection by recording the status of the SEALDD monitoring packets. The SEALDD client also evaluates the reporting criteria and the flow alignment reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report.

For crossflow RTT measurement, if T1 is received in the DL packet, the SEALDD client records local time T2 and calculates RTT based on T1 and T2.

Depending on which entity triggers the data transmission quality measurement, step 11 and step 12 corresponds to step 2 to step 5, step 13 corresponds to step 6.

For the Non-3GPP access measurement, if the Non-3GPP access measurement information includes list of WLAN SSIDs, then the SEALDD client performs the signal strength measurement for the requested WLAN SSID. If the request contains location-based measurement then the SEALDD client measures the signal strength of nearby WLAN SSIDs based on its location.

If the delay difference, i.e., the bound of the difference of the end-to-end time delay among all associated flows, is requested, the SEALDD client calculates the delay difference between VAL server and SEALDD client, based on the arrival time and the timestamp (e.g., timestamp as recorded in the RTP header) among all associated flows identified by the Multi-modal SEALDD flow ID. The arrival time of first detected flow and last detected flow, Ta and Tb, are recorded, and the delay difference is the difference between Ta and Tb. Then SEALDD client sends the calculated delay difference to the SEALDD server.

11-12.	The SEALDD client reports the data transmission quality measurement results (e.g. latency, jitter, bitrate) to the VAL server via the SEALDD server. The report also includes the list of WLAN SSIDs and their signal strength measurements (like RSSI). The Measurement range indicate the scope of the measurement, e.g., end to end measurement, tethering link measurement etc.

13.	The SEALDD client reports the data transmission quality measurement results to the VAL client.

NOTE 3:	The crossflow RTT measurement calculated by the SEALDD client in step 10 can be used as an estimation of the lower bound for the Motion-to-Photon latency in the XR scenarios.

When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 11 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server collects and aggregates the needed report for the VAL server.

Editor's note: Whether identifying packet in PDU set is necessary in crossflow RTT measurement is FFS.

# *** Next Change ***

#### 9.7.3.6	Transmission quality measurement subscription request

Table 9.7.3.6-1 describes the information flow from the SEALDD server to the SEALDD client for data transmission measurement subscription.

Table 9.7.3.6-1: Transmission quality measurement subscription request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD-UU flow ID | O (see NOTE) | Identifier of the SEALDD-UU flow. |
| Multi-modal SEALDD flow ID | O (see NOTE) | Identity of the multi-modal SEALDD flows associated with the SEALDD multi-modal connection for the measurement. |
| Measurement conditions | O | Indicates the temporal and/or spatial conditions. |
| Transmission quality measurement requirements list | M | The measurement requirement information |
| > Measurement ID | M | Measurement identifiers, e.g. latency, bitrate, jitter, signal strength (e.g. RSSI) |
| > Reporting frequency | O | The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting. |
| > Reporting periodicity | O | If the reporting frequency is periodic, the reporting periodicity shall be provided. |
| > Measurement period window | O | Indicates the measurement period window for transmission quality measurements |
| > Measurement expiration time | O | Indicates the measurement expiration time |
| > Reporting criteria | O | Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria if more than one criteria is specified. |
| > Delay difference indicator | O | Indicates the delay difference needs to be measured. |
| > Flow alignment reporting criteria | O | Indicates the criteria for reporting flow alignment measurement results, e.g. if the flow alignment buffering reaches below or above a certain value for a certain amount of time. It also includes a unique identifier for each criteria if more than one criteria is specified. |
| > SEALDD policy | O | Specifies quality guarantee policies associated with the SEALDD connection |
| >> Quality guarantee policy | M | Indicates the event (e.g. measurement threshold) to be measured for, the quality guarantee. |
| >> Non-3GPP access measurement information | O | Indicates the non-3GPP access(like WLAN) measurement policy (e.g. WLAN SSIDs, BSSID, location-based measurement) |
| NOTE: Only one of the information element is present. |  |  |

# *** Next Change ***

#### 9.7.3.8	Transmission quality measurement notification

Table 9.7.3.8-1 describes the information flow from the SEALDD client to the SEALDD server for notifying the transmission quality measurement reports.

Table 9.7.3.8-1: Transmission quality measurement notification

| Information element | Status | Description |
| --- | --- | --- |
| Transmission quality measurement reports list | M | The generated transmission quality results in SEALDD server |
| > Measurement ID | M | Measurement identifiers, e.g. latency, bitrate, jitter, delay difference, flow alignment measurement, signal strength (e.g., RSSI) |
| > Measurement range | O | Indicates the range of the measurement, e.g., end to end measurement, tethering link measurement etc. |
| > Average measurement value | O | The average measurement value of measurement results |
| > Minimum measurement value | O | The minimum measurement value of measurement results |
| > maximum measurement value | O | The maximum measurement value of measurement results |
| > Standard deviation measurement value | O | Standard deviation measurement value of measurement results |
| > kPercentile measurement value | O | Indicates the kpercentile measurement value of measurement results |
| > Measurement period | O | Indicates the measurement period |
| > Timestamp | O | Indicates the timestamp of measurement results |
| > Non-3GPP access measurement information | O | Indicates the non-3GPP access measurement information report. |
| > Measured non-3GPP access | O | List of measured non-3GPP access (e.g., WLAN SSID(s)/BSSID(s) or location information). |
| >> Signal strength value(s) | M | List of signal strength values (e.g., RSSI) for the measured non-3GPP access. |

# *** Next Change ***

#### 9.9.2.4	SEALDD enabled data transmission quality guarantee using Non-3GPP access

Figure 9.9.2.4-1 illustrates the procedure of using non-3gpp access for SEALDD enabled data transmission quality guarantee.

Pre-conditions:

1.	The SEALDD Client is authorized to use non-3GPP access on behalf of the VAL client.

Figure 9.9.2.4-1: SEALDD enabled data transmission quality guarantee using Non-3GPP access

1.	A regular data transmission connection is established according to clause 9.2.2.2 or via clause 9.3.2.3 and SEALDD data transmission measurement reporting as per clause 9.7.2.1 or clause 9.7.2.2 to get the data transmission quality.

2.	If Non-3GPP access measurement policy is available in the SEALDD server, the SEALDD server can request the SEALDD client to report the latest Non-3GPP access measurements(like RSSI for the WLAN SSIDs) as per steps 3-4 of the procedure defined in clause 9.2.2.6.

If Non-3GPP access measurement policy is not available, the SEALDD server fetches the UE location using the SEAL location service defined in 3GPP TS 23.434 [4] clause 9.3.12 and requests the SEALDD client to report the latest Non-3GPP access measurements of the nearby WLAN SSIDs. The SEALDD server uses the steps 3-4 of the procedure defined in clause 9.2.2.6 to get the report from the SEALDD client.

If the Non-3GPP access measurement values in the data transmission quality report or SEALDD connection status report defined in clause 9.2.2.6 are above the Signal strength thresholds mentioned in SEALDD Non-3GPP access measurement policy then the SEALDD server can offload the SEALDD-UU connection from 3GPP access to Non-3GPP access(WLAN) using steps 5-6.

3.	The SEALDD server subscribes to NWDAF analytics service for subscription to network performance analytics with Analytics ID as Network Performance using the procedure described in clause 6.6.4 3GPP TS 23.288 [19]. It also subscribes to PDU Session traffic analytics service using the procedure defined in clause 6.20 3GPP TS 23.288 [19].

The SEALDD server can subscribe to WLAN performance analytics using the procedure defined in clause 6.11 3GPP TS 23.288 [19]. The SEALDD server uses the SEALDD Non-3GPP access measurement policy information or information received in step 2 for subscription to WLAN performance analytics.

4.	Based on SEALDD client Non-3GPP access measurement reports, NWDAF analytics measurement reports(including predicted values), and data transmission quality measurement reports, the SEALDD server detects the 3GPP RAN congestion and determines to use non-3GPP access to meet the data transmission quality requirements.

5.	The SEALDD server can request 5GS to create or update URSP rules using clause 4.15.6.10 of 3GPP TS 23.502 [6] to establish a PDU session connection over Non-3GPP access. The SEALDD server can create a URSP rule with Access Type Route selection descriptors as non-3GPP to establish PDU session. The request may include the UE ID and application traffic descriptor containing the addresses or ports allocated by SEALDD server.

6.	The UE receives the new or updated URSP rules from the 5G core network. Based on the URSP rules, the UE establishes a PDU session connectivity over non-3GPP access with the 5GS.

7.	On the successful establishment of PDU session connection over Non-3GPP access, the SEALDD client updates the SEALDD data transmission connection with the updated SEALDD client IP address as per steps 7-8 mentioned in clause 9.3.2.2.

Based on continuous monitoring and measurement reports, the SEALDD server can decide to switch back to 3GPP access and use the SEALDD-UU connection over 3GPP access for data transmission.

# *** Next Change ***

#### 9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID). |
| Identities | O | Identifier of the VAL UEs or VAL users, or VAL UE group ID(as defined in clause 7.5 of 3GPP TS 23.434 [4]), or VAL UE identity list for which SEALDD policy applies |
| Slice identifier | O | The slice identifier (S-NSSAI) for which SEALDD policy applies. |
| SEALDD policy | O (See NOTE 4) | The SEALDD policy associated with application traffic identifiers, VAL UE identity. |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the event (e.g. measurement threshold) to be measured for the quality guarantee |
| > Quality optimization policy | O (See NOTE 1) | Indicates the data transmission adjustment policy (e.g., adjustment need) to be performed in SEALDD layer. |
| > Bandwidth control policy | O (See NOTE 2) | Indicate the bandwidth control preference, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL |
| > Geofence policy | O (See NOTE 3) | Indicates the geofence policy per VAL service. |
| >> Geofence location | O (See NOTE 3) | Indicates the geofence location information for the mentioned VAL service |
| >>Geofence policy action | O (See NOTE 3) | Indicates the policy action like allowed or blocked. |
| > temporal policy | O (See NOTE 3) | Indicates the time period for which the sealdd traffic is allowed |
| > policy expiration time | O | Indicates the validity of the policy. |
| > Non-3GPP access measurement policy | O | Indicates the non-3GPP access (like WLAN) measurement policy (e.g. WLAN SSIDs/BSSID, signal strength thresholds(high/low), time-based threshold for offload to WLAN). |
| Multi-modal SEALDD policy | O (See NOTE 4) | Multi-modal SEALDD policy associated with set of individual SEALDD flows. |
| > Synchronization policy | O | Indicates the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2]. |
| > Multi-modal flows alignment policy | O | Indicates the information to do the multi-modal flows alignment, e.g., Multi-modal Service ID, maximum acceptable duration for traffic flow alignment. |
| > Policy expiration time | O | Indicates the validity period of the policy. |
| > UE-to-UE policy | O | Specifies UE-to-UE direct communication policy including proximity threshold, QoS threshold (e.g. bitrate, latency and jitter, PLR, PER) or QoE threshold (e.g. MOS as specified in ITU-T P.1203.3 [18]) for entering direct communication mode, and proximity threshold, QoS threshold or QoE threshold for leaving direct communication mode. |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. NOTE 3: This IE is used for the SEALDD connection establishment and data delivery, as specified in clause 9.2 NOTE 4: At least one of these IEs shall be present. |  |  |

# *** End of Changes ***