3GPP TSG-SA WG6 Meeting #65	S6-250196

17 Feb 2025 - 21 Feb 2025	(revision of S6-xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0021 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLE Functional Description updata |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_APP |  |  |  |  |  | Date: |  |  | 2025-2-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The AIMLE Functional Description is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Supplement the AIMLE Functional Description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIMLE Functional Description is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Change 1 * * * *

6	AIMLE Functional Description

6.1	Support for ML model retrieval

This functionality covers the retrieval of ML models by an AIMLE client or a VAL server via the AIMLE server. The AIMLE server provides the functionality required to retrieve ML models stored in a ML repository based on a filtering criteria. The functionality is provided to the ML model consumer via an API following a request/response model or a subscribe/notify model.

6.2	Support for ML model training and inference

This functionality enables the AIMLE server to support ML model training based on requests from a VAL server. The AIMLE server provides the functionality to train a specific ML model or assist the ML model training at VAL server/client(s).

This functionality enables the AIMLE server to support ML model inference based on inference requests from a VAL server. The AIMLE server provides the functionality to receive inference requests from the VAL server and return the inference results to the VAL server.

6.3	Support for FL member registration

This functionality covers the registration and registration update of the candidate FL member to the ML repository which is keeping the FL member registrations. Such candidate member can be a VAL server functionality or an enabler layer functionality (e.g. AIMLE server) which is registering to the ML repository/registry to act as FL member for a given application event (analytics event or event triggered by a VAL layer application server).

6.4	Support for FL events subscription and notification

This functionality enables a consumer (who can be the AIMLE server or a VAL server e.g. acting as FL server) to subscribe for FL related events and getting notified on changes on the availability of the FL members which are to be used for the FL-related task (e.g., training). This capability at the ML repository acting as an AIML service registry supports the subscription for events related to FL members and the notification to the consumer in case of changes. This feature assumes that such FL members (AIMLE or VAL server or AIMLE clients) have previously registered to this registry their availability and capabilities.

6.5	Support AI/ML task transfer

This functionality covers the AIMLE support for ML task transfer, which is applicable to scenarios where an AI/ML member cannot finish the assigned AI/ML task during the performing process. In this scenario, the AIMLE server assists the source AI/ML member by support transferring the intermediate AI/ML information (e.g., the intermediate AI/ML operation status and results) to another AI/ML member (target AI/ML member) for further operations to complete the AI/ML task.

* * * Change 2 * * * *

8.x	ML model inference

8.x.1	General

This clause describes the procedure for supporting the ML model inference by the AIMLE server. The VAL server requests the AIMLE server for the ML model inference. VAL server requests the AIMLE server for ML model inference by specifying the details about the ML model or requirements to select a ML model for inference.

8.x.2	Procedure for ML model inference

Figure 8.x.2-1 illustrates the procedure for AIMLE server to support ML model training based on the request from the VAL server.

Figure 8.x.2-1: ML model inference

1.	The VAL server sends an ML model inference request to AIMLE server, requesting the inference result from the AIMLE server. This request consists of ML model information or ML model inference requirement information, etc.

2.	The AIMLE server checks whether the VAL server is authorized to perform the ML model inference request. If no model information is provided but only the model inference requirement information is provided in step 1, the AIMLE server identifies and selects the appropriate ML model for inference based on the ML model inference requirement information using the procedure as specified in clause 8.11.3. The AIMLE server handle the request and execute the inference task.

3.	If the VAL server is authorized, AIME server returns the inference response, otherwise a failure response indication the reason for failure.

8.x.3	Information flows

8.x.3.1	ML model inference request

Table 8.x.3.1-1 describes the information flow from the VAL server to the AIMLE server for requesting the ML model inference.

Table 8.x.3.1-1: ML model inference request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Input data | M | Original data for inference that may include text, images, audio, numerical data, etc. |
| Inference Parameters | O | Additional inference parameters that may be needed, such as maximum output length, temperature, batch size, etc.Depending on the specific requirements of the model |
| ML model information | O (NOTE 1) | Identifies the ML model that VAL server specified to perform inference. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model inference requirement information | O (NOTE 1) | Identity the type or goal of the inference task, such as 'text classification,' 'image recognition,' 'sentiment analysis,' etc |
| NOTE 1: At least one of these IE shall be present. |  |  |

8.x.3.2	ML model inference response

Table 8.x.3.2-1 describes the information flow of ML model inference response from the AIMLE server to the VAL server.

Table 8.x.3.2-1: ML model inference response

| Information element | Status | Description |
| --- | --- | --- |
| Inference response | O (NOTE) | Indicates that the ML model inference request was successful and the inference result was returned |
| > ML model identifier | O | Identifies the ML model selected by AIMLE server for inference. |
| Failure response | O (NOTE) | Indicates that the ML model inference request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: At least one of these information elements shall be present |  |  |

* * * End of changes * * * *