3GPP TSG-SA WG6 Meeting #65	S6-250198

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0087 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of response to non-3GPP Security Message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sepura Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCCI |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Non-3GPP security message handling was intended to mirror the handling of LMR KMMs where the LMR secuity layer decides whether a response is needed to a received message on the basis of the received payload. As the payload is opaque, the IWF is not able to judge whether it is correctly formatted (or the nature of the message) and a response (in some form) initiated via an LMR aware client would thus be provided to the MC system as another Non-3GPP Security Request message, since there is no ability to distinguish at the MC level between an opaque LMR level request and response. An end to end acknowledgement at MC level would require addition of some form of message ID to allow correlation of the Request with the Response, and associated incorporation of additional MCData SDS functional architecture into the MCPTT service plane for realisation in the MCPTT service. There are already alternative mechanisms available should a delivery notification at MC level be desired (i.e. MCData SDS). Key management entity may not always be internal to the LMR network and so any acknowldgement response (if needed) needs to be described in terms of sending and receiving URI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of formal Non-3GPP Security message response. Modification of text, indicated flows to indicate that a response can be sent as another Non-3GPP Security message Backwards compatibility: this change is not backwards compatible at Stage 2 level, but the Stage 3 has never been completely/correctly described and so there is no backwards impact on implementation in making this change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch of Stage 2 and Stage 3, effective reincorporation of MCData SDS architecture into the MCPTT architecture when implementing in the MCPTT plane. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.12.1.2, 10.12.2.1, 10.12.2.2, 10.12.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Begin Change * * * *

## 10.12	LMR security transport

### 10.12.1	Information flows for LMR security transport

#### 10.12.1.1	Non-3GPP security message request

Table 10.12.1.1-1 describes the information flow non-3GPP security message request from the MC service server to the IWF, from the IWF to the MC service server, from the MC service server to the MC service client and from the MC service client to the MC service server.

Table 10.12.1.1-1: Non-3GPP security message request

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user |
| URI of LMR security functional entity | M | URI of LMR key management functional entity user profile parameter defined in 3GPP TS 23.379 [7] |
| LMR type | O | The LMR technology, e.g. TETRA, P25. Required when sent toward the MC service client. |
| Payload | M | Opaque payload. Contents and format are out of 3GPP scope. |

#### 10.12.1.2	Void

* * * Next Change * * * *

### 10.12.2	LMR key management messages

#### 10.12.2.1	General

This subclause defines end to end messaging to convey the non-3GPP, LMR security information opaquely (message contents are out of 3GPP's scope) across the MC system, between the IWF and the LMR aware MC service client. The end to end messages are service independent, any MC service may support them.

#### 10.12.2.2	MC service client initiated

Figure 10.12.2.2-1 describes the case where an MC service client sends LMR security information to the IWF.

Pre-conditions:

1.	The MC service client is registered and the user is authenticated and authorized to use the MC service server.

Figure 10.12.2.2-1: Non-3GPP security messaging, MC service client to the IWF

1.	The MC service client sends a non-3GPP security message request to the MC service server. The contents of the message are opaque to the MC service and are out of scope of 3GPP.

2.	The MC service server forwards the contents of the non-3GPP security message request to the IWF.

3.	The IWF can respond with a non-3GPP security message request to the MC service server. How the IWF decides whether to respond is outside the scope of the present document.

4.	The MC service server forwards the contents of the non-3GPP security message request to the MC service client.

* * * Next Change * * * *

#### 10.12.2.3	IWF initiated

Figure 10.12.2.3-1 describes the case where the IWF sends LMR security information to an MC service client.

Pre-conditions:

1.	The MC service client is registered and the user is authenticated and authorized to use the MC service server.

Figure 10.12.2.3-1: Non-3GPP security messaging, from the IWF to MC service client

1.	The IWF sends a non-3GPP security message request to the MC service server. The contents of the message are opaque to the MC service and are out of scope of 3GPP.

2.	The MC service server forwards the contents of non-3GPP security message request to the MC service client.

3.	The MC service client can respond with a non-3GPP security message request to the MC service server. How the MC service client decides whether to respond is outside the scope of the present document.

4. The MC service server forwards the contents of the non-3GPP security message request to the  IWF.

* * * End Change * * * *