3GPP TSG-SA WG6 Meeting #65	S6-250205

Athens, Greece, 17th – 21st January 2025

Source:	Ericsson

Title:	Correction to SBA of 3GPP System Supporting SEAL Services

Spec:	3GPP TR 23.700-35 v0.2.0

Agenda item:	8.9

Document for:	Approval

1. Introduction

Application client can invoke SEAL server services via SEAL client(s), but not support to consume SEAL server services directly. This paper proposes to correct the error in clasue 8.1.1.

2. Reason for Change

In different specifications, different terms are used for represent the same thing i.e., VAL client. For example, VAL client in TS 23.434 is used to represent entity that provides the client side functionalities corresponding to the vertical applications, application client in TS 23.558 is used to represent the application resident in the UE performing the client function in application. This paper introduces corrections to clause 8.1.1 by removing direct link between application client and SEAL server.

3. Conclusions

This pCR proposes to remove the direct link between application client and SEAL server.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-35.

* * * First Change * * * *

## 8.1	Solution#1: Service based Architecture of 3GPP system supporting SEAL services

### 8.1.1	Solution description

This solution is to solve GAP# 5-1-2-1.

Figure 8.1.1-1 Service based Architecture of 3GPP system supporting SEAL services

In the figure 8.1.1-1, SEAL services layer represents the SEAL server(s) supporting SEAL services defined in the SEAL specification 3GPP TS 23.434.

SEAL services which are part of the 3GPP system can be deployed either by the MNO or third party.

SEAL server(s) can invoke service APIs exposed by 3GPP core network functions directly over the Core network SBA when deployed by MNO’s trusted domain. SEAL server(s) can invoke service APIs exposed by 3GPP core network functions via NEF as a non-trusted AF when deployed by the third party.

SEAL server(s) can invoke service APIs exposed by the OAM system over the OAM SBA when deployed by MNO. SEAL server(s) can invoke services APIs exposed by 3GPP core network functions via EGMF as non-trusted AF, when deployed by the third party.

SEAL services API can be exposed by MNOs or third party, to the Application server (VAL server), or to the Application client(s) (VAL client(s)) via SEAL client(s).

NOTE 1:	In some serverless application software implementations, application server (VAL server) may not be implemented and deployed. In such deployments, Application client(s) (VAL client(s)) can support all the application requirements by invoking SEAL service APIs via SEAL client(s).

NOTE 2:	3GPP TS 23.222 [9] supports where the API invokers can either be on the network side (Application server) or on the UE side (Application clients via SEAL client(s)), to use services API.

Editor’s note: It is FFS whether and how OAM interacts with SEAL in a service based manner.

Editor’s note: It is FFS whether and how to add the interface between UE and Radio network.

# *** End of Changes ***